# SA WG2 Meeting #S2-164		S2-2408910

# 19 - 23 August, Maastricht, Netherlands		(revision of S2-2408394)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0557 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on the LCS-UP procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the LS (S2-2407423, C1-243934) from CT1 WG, it is agreed that LCS-UP binding ID (called LCS UP connection ID currently in SA2) will be added in the user plane information sent by LMF. And the user plane connection establishment command is acknowledged over CP after the binding procedure is performed over UP. The LCS-UP binding ID is used to associate the UE with the user plane connection during the user plane connection establishment procedure. The LCS-UP binding ID will be released once used, in order to avoid security threats. It is proposed to update clause 6.18.1 and 6.18.2 to align with CT1’s specification with the following changes: - Change the term “LCS UP connection ID” to “LCS-UP binding ID” - Update the order of the steps to align that “the user plane connection establishment command is acknowledged over CP after the binding procedure is performed over UP” - Add clarifications about LCS-UP binding ID release - 6.18.2 is updated to refer to 6.18.1 for simplification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 6.18.1, 6.18.2 to align with CT1’s specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not aligned with the agreement in CT1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.1, 6.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.18.1	LMF initiated User Plane Connection

LMF may trigger the user plane connection establishment after receiving a location request from AMF if target UE does not have user plane connection with LMF. AMF may subscribe from LMF the status of LCS user plane connection for the target UE, using a Nlmf_Location_UP Subscribe message if the UE supports user plane positioning. Figure 6.18.1-1 shows a procedure triggered by LMF to support positioning over the user plane connection between UE and LMF.

Figure 6.18.1-1: Positioning via a User Plane Connection between UE and LMF initiated by LMF

NOTE 1:	User Plane protocol (LCS-UPP) to support generic transport between the UE and the LMF is defined in TS 24.572 [48].

1.	Based on UE user plane positioning capability, control plane congestion status (e.g. AMF load status) and other implementation factors, LMF decides whether to use the positioning procedure via a user plane connection between UE and LMF.

LMF may invoke Nnrf_NFDiscovery service operation to retrieve control plane congestion status (e.g. AMF load information). LMF may also invoke Nnrf_NFManagement_NFStatusSubscribe service to subscribe specific AMF load information. Based on AMF load information, LMF may determine to use user plane positioning, if there is available user plane connection between UE and LMF.

Steps 2-7 are skipped if there is already a user plane connection context of the target UE in LMF and LMF determines to utilize the user plane connection for positioning.

NOTE 2:	LMF can select user plane positioning for specific positioning methods (e.g. motion sensor-based method) and it is based on implementation and local configuration to determine which positioning method requires user plane transport.

NOTE 3:	The procedure can also be triggered when LMF receives a location request from AMF via control plane signalling as defined in clause 6.1 and clause 6.3.

2.	[Conditional] If LMF decides to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF invokes Namf_communication_N1N2MessageTransfer service operation to send the user plane information to AMF in a NAS container to indicate UE to utilize user plane over TLS for positioning. The user plane information includes the user plane positioning address of the LMF, security related information. The LMF allocates LCS-UP binding ID to be used to associate the user plane connection to be established with the target UE and includes this LCS-UP binding ID in the user plane information. The LMF associates the target UE identity (SUPI and/or GPSI) with this LCS-UP binding ID.

NOTE 4:	Security mechanism to support user plane positioning is defined in Annex Q.2 of TS 33.501 [50].

3.	[Conditional] When AMF receives the user plane information from LMF in step 2, AMF sends it to UE via a DL NAS TRANSPORT message.

4.	[Conditional] If there is no established applicable PDU session for the user plane positioning, the UE uses the URSP as defined in TS 23.503 [41] which includes user plane positioning related PDU session parameters, e.g. a dedicated DNN and S-NSSAI, to establish the PDU session for user plane positioning. UE may send an acknowledgement to LMF through AMF to indicate a success of utilization of a user plane connection for positioning service or a failure to utilize the user plane connection, e.g. no suitable PDU session established.

5.	[Conditional] AMF sends the acknowledgement received in step 4 to the LMF via Namf_N1messageNotify service.

4.	[Conditional] If there is no established applicable PDU session for the user plane positioning, the UE uses the URSP as defined in TS 23.503 [41] which includes user plane positioning related PDU session parameters, e.g. a dedicated DNN and S-NSSAI, to establish the PDU session for user plane positioning. UE establishes a secured user plane connection with LMF. If LMF send its FQDN to the UE, a DNS server/resolver is used to resolve the IP address of LMF (e.g. EASDF or local DNS for local LMF address resolution). After the secured user plane connection been established successfully, the UE sends the LCS-UP binding ID received in step 3 to LMF via the secured user plane connection to enable LMF to perform the correlation of the UE with the secured user plane connection. The LCS-UP binding ID will be released once the correlation is complete.

5.	[Conditional] UE may send an acknowledgement to LMF through AMF to indicate a success of utilization of a user plane connection for positioning service or a failure to utilize the user plane connection as defined in TS 24.572 [48].

6.	[Conditional] AMF sends the acknowledgement received in step 5 to the LMF via Namf_N1messageNotify service.

7.	[Conditional] LMF indicates AMF in the Nlmf_Location_UPNotify message that user plane connection between the UE and LMF has been established.

8.	[Conditional] The AMF stores the LCS-UP connection context as part of UE context.

9.	If LMF or UE determines to utilize the user plane connection for positioning and the secure user plane connection is established, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services messages including event report messages, periodic triggered invoke messages and MS cancel deferred location messages may also be transferred between LMF and UE via the established user plane connection.

# * * * * Next change * * * *

### 6.18.2	UE initiated User Plane Connection

UE may trigger the user plane connection establishment if the UE does not have user plane connection with LMF. Figure 6.18.2-1 shows a procedure triggered by UE to support positioning over the user plane connection between UE and LMF.

Figure 6.18.2-1: Positioning via a User Plane Connection between UE and LMF, initiated by UE

1.	UE sends a user plane connection establishment request to AMF via NAS Message, if UE decides to request a user plane connection for upcoming positioning requests.

2.	[Conditional] If the UE is authorized based on UE Subscription to use the user plane positioning, AMF selects an LMF which capable to establish a user plane session for positioning with the UE. AMF may either query the NRF or based on local configuration to discover and select a proper LMF.

3.	[Conditional] The AMF sends a Nlmf_Location_UPConfig Request towards LMF to request set up of an LCS-UP connection. The AMF shall include the target UE identity (see TS 29.572 [12]) (SUPI and/or GPSI) in the request.

4. [Conditional] If LMF accepts to utilize user plane for positioning, step 2-8 in Clause 6.18.1 LMF initiated User Plane Connection Establishment are performed.

5.	[Conditional] After the secure user plane connection is established, if LMF determines to utilize the user plane connection for positioning after receiving a positioning request from AMF, or UE determines to utilize the user plane connection for positioning, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services messages including event report messages, periodic triggered invoke messages and MS cancel deferred location messages may also be transferred between LMF and UE via the established user plane connection.

# * * * * End of changes * * * *