3GPP TSG-SA2 Meeting #164	S2-2408953

# Maastricht, Netherlands, 19 August – 23 August, 2024 (revision of S2-2408628+7875+8549)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0135 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support of network-assisted/ground-based mechanism for Detect And Avoid with 5GS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion for Key Issue#2 “Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information” in TR 23.700-59, it is proposed to add the functional description and information flows for the feature in clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the feature of Network-assisted/ground-based mechanism for DAA with 5GS information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature of Network-assisted/ground-based mechanism for DAA with 5GS information is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5.6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[16]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[17]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[18]	3GPP TS 26.502: "5G multicast-broadcast services; User Service architecture".

[19]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[x]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

* * * Second Change * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2X	Aircraft-to-Everything

AAM	Area Airspace Manager

BRID	Broadcast Remote Identification

BVLOS	Beyond Visual Line of Sight

C2	Command and Control

DAA	Detect And Avoid

DDAA	Direct Detect And Avoid

MBS	Multicast/Broadcast Service

NRID	Networked Remote Identification

NWDAA	Network-Based/Assisted DAA

PQI	PC5 5QI

RID	Remote Identification

TPAE	Third Party Authorized Entity

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

UAV-C/UAVC	Uncrewed Aerial Vehicle Controller

USS	UAS Service Supplier

UTM	Uncrewed Aerial System Traffic Management

UUAA	USS UAV Authorization/Authentication

UUID	Universal Unique Identifier

* * * Third Change (all new texts) * * * *

5.X	Support of Network-Based/Assisted DAA

5.x.1	Functional Description

This clause provides a network-based/assisted DAA mechanism. To enable NWDAA, the following information can be utilised:

-	Absolute location of a UAV provided by GMLC via UAS NF/NEF;

-	Relative Location of UAVs provided by GMLC via UAS NF/NEF;

-	Output of Relative Proximity Analytics provided by NWDAF.

Absolute location of a UAV is generated/derived in 5G network by using the existing LCS feature defined in TS 23.273 [8]. USS may request to GMLC to obtain one or multiple UAV location as defined in clause 6.2.1, 6.3.1 or 6.20.5 in TS 23.273 [8]. As an example, USS may use the absolute location of the UAV(s) provided by the network to e.g. calculate relative distance of the pair of UAVs, and based on that to deal with the UAV tactical deconfliction, collision avoidance etc.

Relative location of a UAVs is provided in 5G network by using the existing LCS Ranging feature defined in TS 23.273 [8]. USS may send a Ranging request to GMLC to obtain relative location between one and multiple UAVs as defined in clause 6.20.3 or 6.20.4 in TS 23.273 [8]. USS may use the relative location of the UAV(s) to avoid collision.

USS (acting as an AF) may request Relative Proximity Analytics provided by NWDAF for the purpose of DAA.

NOTE 8:	How USS uses the relative distance of UAVs or absolute location of a UAV or Output of Relative Proximity Analytics to determine the potential colllisions is out of 3GPP scope.

To deconflicte the potential collisions, the USS notifies the potential collision to the UAV UE or AAM or UAV-C which is responsible to inform the paired UAV about the collision. When the notification of the potential collision is receied by AAM, it performs the deconfliction by triggering the mechanism specified for ground-based DAA for an area in clause 5.7.2.  When the UAV UE receives the potential collision notification from USS or the paired UAV-C, it performs the deconfliction by triggering the mechanism specified for DAA based on PC5 in clause 5.6.1.

* * * Fourth Change (all new texts) * * *

5.X.2	Procedure for UAV Triggered Network-assisted DAA

Figure 5.X.2-1: Procedure for UAV/UAV-C Triggered Network-assisted DAA

1.	The UAV establishes a PDU Session for communication with the USS as described in clause 5.2.3.

2.	The DAA service may be triggered by UAV(s). The UAV(s) (via its paired UAV-C) requests DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides to subscribe/request to 5GC for GMLC service on Ranging/Sidelink Positioning location and/or Relative Proximity predictions on collision from NWDAF.

NOTE 1:	The other content of DAA service information derived at USS is out of scope.

3a.	The USS may request GMLC service via NEF for Ranging/Sidelink Positioning location results, i.e. the relative positioning between UAV UEs as described in clause 6.20.3 of TS 23.273 [8] for one notification, or clause 6.20.4 of TS 23.273 [8] for notifications (steps 1-20 for initiation monitoring, steps 21-31 for monitoring periodic).

3b.	USS as AF/LCS client may send the absolute positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI) as described in clause 6.2.1 and 6.3.1. GMLC replies to USS of the absolute location of UAV 1 and UAV 2. USS calculates the relative positioning result based on the UAV UE1 absolute location and UAV UE2 absolute location.

NOTE 2:	USS can request same or different GMLC for the UAV1 and UAV 2 location, e.g. depending on which PLMN the UAV registers.

4.	The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [Y] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [Y]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)). The other parameters included in the request are described in clause 6.5.3.3.

5.	After NEF receive the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [Y]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

Editor’s Note: If both step 3 and step 4 to step 6 happen, it is FFS how to reduce possible positioning signaling to same UEs.

6.	If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [Y] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [Y].

7.	If receiving the location of UAV1 and UAV2, USS calculates the relative positioning result for UAV UE1 location and UAV UE2 location from step 1 and step 2. The USS estimates the potential collision based on the information received in step 3 and/or the predictions from step 6. The USS informs the UAV-C(s) the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

USS may be replaced by TPAE.

8.	The UAV-C(s) informs its paired UAV(s) the potential collision, and information received from the USS, include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision, and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

9.	UAVs performs operations to avoid collision.

9a.	If both the two UAVs which may collision (e.g. UAV 1 and UAV 2) been informed by their paired UAV-C(s), the UAVs can be steered to avoid collision in accordance with the received information (e.g. trajectory correction information to avoid collision) and using mechanisms that are out of scope for 3GPP.

9b.	If only one of the two UAVs which may collision (e.g. UAV 1) been informed by its paired UAV-C, the UAV 1 triggers conflict resolution procedure with UAV 2 as described in steps 4-7 in clause 5.6.1.

* * * Fifth Change (all new texts) * * *

5.x.3	Procedure for AAM Triggered Network-assisted DAA

Figure 5.x.3-1: Procedure for AAM Triggered Network-assisted DAA

1.	As described in clause 5.7.2 steps 1-6, the UAV(s) listens for signals on the correspondingly destination Layer-2 ID configured for the used service type. The AAM scans the airspace over the area/arena for UAV(s), retrieves for each detected UAV the corresponding Remote-ID, and establishes a PC5 direct communication link with the discovered UAV. Using the PC5 unicast direct communication link the AAM and the UAV establishes a bidirectional communication channel for exchange of messages.

2.	The AAM establishes a PDU Session for communication with the USS as described in clause 5.2.3.

3.	The AAM may request network assist DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides, e.g. to request GMLC service for Ranging/Sidelink Positioning location, to subscribe/request to NWDAF for Relative Proximity predictions on collision.

NOTE:	The other content of DAA service information derived at USS is out of scope.

4.	The USS requests GMLC service for Ranging/Sidelink Positioning location, and/or subscribes/requests notification on Relative Proximity predictions provided by NWDAF via NEF, as described in clause 5.7.X steps 3-6.

5.	The USS estimates the potential collision based on the received information/analytics in step 4. The USS informs the AAM the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

6.	The AAM provides the determined steering policy to the specific UAVs (e.g. UAV 1 and UAV 2) according to the information received in step 5, and the UAVs are steered to avoid collisions in accordance with received policy and using mechanisms that are out of scope for 3GPP, as described in clause 5.7.2 steps 7-8.

* * * Sixth Change (all new texts) * * *

5.x.4	Procedure for USS Triggered Network-assisted DAA

Figure 5.x.4-1: Procedure for USS Triggered Network-assisted DAA

1.	The UAV and UAV-C establish PDU Session for communication with the USS as described in clause 5.2.3.

2.	The USS may decide to trigger NWDAA based on application layer request.

3.	The USS requests absolute locaton of UAV(s) based on the identifiers received in step 2 by invoking the 5GC-MT-LR procedure for the commercial location service in clause 6.1.2 or Initial and Reporting of Location Events in clause 6.3.1 or 5GC-MT-LR procedure using SL positioning in clause 6.20.5 in TS 23.273[8].

4.	[Optional] The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [x] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [x]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)).

5.	[Optional] If the step 4 is performed, after NEF receive the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [x]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

6.	[Optional] If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [x] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [x].

7. The USS performce step 7a and 7b  or step 7C.

7a.	If the USS detects the potential collision based on the information received in step 3 and/or step 6, the USS notifies the potential collision to UAV-C.

7b.	[Conditional] If the step 7a is performed, the UAV-C further notifies the potential collision to UAV.

7c. If the USS detects the potential collision based on the information received in step 3 and/or step 6 is performed, the USS notifies the potential collision to UAV UE.

8.	If the USS detects the potential collision based on the information received in step 3 and/or step 6, the USS notifies to potential collision to AAM.

NOTE Y:	The notification in steps 7a, 7b and 8 is out of 3GPP scope. As an example, the notification may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

9.	[Conditional] If the step 8 is performed, to avoid confliction, steps 6-8 in high-level procedure for Ground-based DAA for an Area in clause 5.7.2 are performed.

10.	[Conditional] If the step 7b or step 7c is performed, to avoid confliction, steps 4a-7 in DAA procedure based on PC5 in clause 5.6.1 are performed.

* * * End of Changes * * *