3GPP TSG-SA2 Meeting #164	S2-2408979

# Maastricht, Netherlands, 19 August – 23 August, 2024          			(revision of S2-2408624)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0564 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the usage of reliable indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the following SA3 answer in S2-2407408, SA3 answer: According to Rel-17 TS 33.256, reliability means using the existing network-assisted positioning method in Rel-17, which relies on location measurements from RAN nodes. There is no agreement in SA3 on additional/new positioning methods for this requirement in Rel-17 and Rel-18. The requirement is that the LMF selects network-assisted positioning method based on the indication of requiring reliable UE location information. This requirement is not specified in TS 23.273, so it is proposed to add the related function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the LMF selects positioning method as per TS.33.356. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsupported SA3 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 43.059: "Functional Stage 2 description of Location Services in GERAN".

[6]	Void.

[7]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[8]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[10]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	OMA MLP TS: "Mobile Location Protocol", [http://www.openmobilealliance.org].

[14]	Void.

[15]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[16]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[17]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[18]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[19]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[20]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[21]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[22]	Void.

[23]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 23.222: "Common Application Programming Interface (API) framework for 3GPP northbound APIs".

[25]	RFC 2396: "Uniform Resource Identifiers".

[26]	RFC 3261: "SIP: Session Initiation Protocol".

[27]	3GPP TS 23.228: "IP multimedia subsystem (IMS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[30]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[31]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[32]	Void.

[33]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[34]	Void.

[35]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[36]	3GPP TS 24.571: "5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3".

[37]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[38]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[39]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[41]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[42]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[43]	3GPP TS 29.563: "Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[44]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[45]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[46]	3GPP TS 33.533: "Security aspects of ranging based services and sidelink positioning".

[47]	3GPP TS 38.355: "Sidelink Positioning Protocol (SLPP)".

[48]	3GPP TS 24.572: " User Plane Location Services (LCS) Protocols And Procedures; Stage 3".

[49]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[50]	3GPP TS 33.501: " Security architecture and procedures for 5G system".

[X]	3GPP TS 33.256: " Security aspects of Uncrewed Aerial Systems (UAS)".

* * * Second Change * * *

4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information. If the indication of requiring reliable UE location information is received, the LMF selects positioning method as per the TS.33.256[x].

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

* * * End of Change * * *