3GPP TSG-WG SA2 Meeting #164	S2-2408985

# 19 - 23 August, 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | xxxx | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy Consumption calculation and exposure of per UE granularity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, SK Telecom, Rakuten Mobile, China Unicom, NEC, ETRI, Tencent, Interdigital, Toyota, KPN, Orange, Deutsche Telekom, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySaving |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This paper provides some detailed method to calculate the per UE level energy consumption information in 5GC and NG-RAN. This proposal intent to show that how the new NF EECF do the data collection and calculate the E2E energy consumption information per UE through cooperation with OAM and NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.X	Support for Energy Saving

5.X.0	General

5GS is enhanced to support energy consumption information collection, calculation and exposure. The 5GS may utilize the energy consuption information to optimize the policy control, session management and NF selection in this release.

5.X.Y	Energy Consumption information calculation and exposure

Energy consumption information calculation and exposure is supported in 5GS for per UE granularity in this release.

Editor’s NOTE: The finer granularity e.g., per UE per AF, per PDU session or per QoS flow will be defined in normative phase.

The energy consumption information of per UE is mainly because of user plane traffic transmission, i.e. the energy consumption in RAN, PSA-UPF, and I-UPF (if existing).

The Energy Efficiency and Consumption Function (EECF) is responsible for the energy consumption information collection and calculation.

NOTE: The EECF can be deployed standalone or co-located with other NFs .

AF or 5GC NF (e.g., NWDAF, PCF) may subscribe the EECF for specific UE energy consumption information exposure, with providing UE GPSI, reporting frequency or reporting threshold.

EECF requests OAM to get the UPF and/or NG-RAN node level energy consumption information based on a measurement time period T (e.g., with including start/end time) as specified in SA5 TS 28.541.

Editor’s NOTE: The coordination between SA2 and SA5 to help OAM expose Node level Energy Consumption information is needed in normative phase.

NOTE: It is assumed that the measurement time period T is aligned over the UPF/NG-RAN nodes for which information is collected, i.e. the start/stop of the time periods is time-synchronized.

The per Node-level EC information received from OAM, could be used by EECF for all the UEs serving by the NF Node.

For per UE granularity, the EECF gets the energy consumption information in 5GC as described in 5.X.Y.Z1, and gets the energy consumption information of NG-RAN as described in 5.X.Y.Z2.

5.X.Y.Z1	Energy Consumption information calculation in 5GC

EECF subscribe the UDM through Nudm_UECM_Get service operation to retrieve the SMF ID(s) which serving the UE. EECF gets the UE’s traffic data volume through invoking Nsmf_EventExposure service. SMF request the corresponding UPFs which serving the UE, with including I-UPF(s) and PSA-UPF(s), to report the data volume of the UE in the UPF, and the data volume of the UPF node, with reporting frequency provided by SMF. Then SMF provide the data volume of the UE in the UPF, the data volume of the UPF node, and the UPF node ID to EECF.

EECF request the UPF node level energy consumption information from OAM based on the UPF node ID reported from SMF.

When (I-)UPF is relocatied, the SMF may get the new (I-)UPF ID and report it to EECF.

For 5G core network side, the user plane energy consumption information per UE is calculated based on the data volume of the UE in the UPF, the data volume of the UPF node, and the UPF node level energy consumption information.

NOTE: The potential formula for Energy Consumption of UE in UPF(s) over a time period T is 
EUE,UPFs= UPFi∈UPFUETEUE, UPFi

,

where UPFUET are all (I-)UPFs used by the UE in a time period T, and EUE, UPFi is the energy consumption the UE in i-th UPF. One potential approach to estimate it is

EUE,UPFi=EUPFiDVUE,UPFiDVUPFi

Where EUPFi is the total energy consumption of the UPF (provided by OAM), DVUE,UPFi is the data volume correspond to the UE in the UPF and DVUPFi is the total data volume of the UPF.

5.X.Y.Z2	Energy Consumption information calculation in NG-RAN

If EECF based on local configuration decides not to involve NG-RAN in the whole procedure, the EECF based on the AF/5GC NF request, gets the AMF ID through invoking Nudm_UECM_Get service operation. And EECF request the AMF and gets the gNB ID which serving the UE. During the handover, the target gNB ID will be sent to AMF and AMF reports this new gNB ID to EECF. The EECF requests the OAM to report the data volume and the energy consumption information of this gNB, within reporting period. And EECF uses the UPF data volume within reporting period, as the NG-RAN side data volume for the UE. For 5G RAN side, the user plane energy consumption information per UE is calculated based on the data volume of the UE in the UPF, the data volume of the gNB node, and the gNB node level energy consumption information.

NOTE: The potential formula for Energy Consumption of UE in gNB over a time period T

is 
EUE,gNBs=gNBi∈gNBUETEUE, gNBi

,

where gNBUET are all gNBs used by the UE a time period T, and EUE, gNBi is the energy consumption the UE in i-th gNB. One potential approach to estimate it is

EUE,gNBi=EgNBiDVUE,gNBiDVgNBi

Where EgNBi is the total energy consumption of the gNB (provided by OAM), DVUE,gNBi is the data volume correspond to the UE in the gNB and DVgNBi is the total data volume of the gNB.

Editor’s NOTE: It is based on RAN WGs decision whether the NG-RAN support to report UE level energy consumption information.

# * * * * End of changes * * * *