SA WG2 Meeting #164	S2-2409000

# Maastricht, Netherlands, 19 – 23 August, 2024                  (revision of S2-2407597)

Source: 	CATT

Title: 	Discussion and conclusion proposals for KI#3

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper discussed KI#3 procedures and made some conclusion proposals.

1	Discussion

Based on the solutions documented in TR 23.700-13 and RAN2 agreements, it can be seen that the following three cases may be supported for AIoT Services:

- Inventory only

- Command only

- Inventory and Command

For Inventory only and Command only, the high level procedure based on existing solutions is illustrated in Figure 1.

Figure 1: High level procedure for Inventory only and Command only

1. AF sends AIoT Service Request message to NEF, and the Request message includes Inventory only or Command only service operation, Device ID info and Area info, etc.

2. NEF authorizes the AF request and determines the AIoTF for AIoT Services.

3. NEF sends AIoT Service Request message to AIoTF, and the Request message includes Inventory only or Command only service operation, Device ID info and Area info, etc.

4. AIoTF determines the A-RAN for AIoT Services.

5. AIoTF sends AIoT Service Request message to A-RAN, and the Request message includes Inventory only or Command only service operation, Device ID info, etc.

6. A-RAN sends Initial Trigger Message (Device ID info) to AIoT Device.

7. AIoT Device performs Random Access procedure.

8. AIoT Device sends AIoT Service Response message to AIoTF via A-RAN. For Inventory only operation, the Response message includes Device ID. For Command only operation, the Response message includes Device ID and Command Result info.

9. AIoTF performs validation and/or authentication of AIoT Device.

10. AIoTF registers AIoT Device in to UDM.

11-12. AIoTF sends AIoT Service Response to AF via NEF.

For Inventory and Command, the high level procedure based on existing solutions is illustrated in Figure 2.

Figure 2: High level procedure for Inventory and Command

1. AF sends AIoT Service Request message to NEF, and the Request message includes Inventory and Command service operation, Device ID info and Area info, etc.

2. NEF authorizes the AF request and determines the AIoTF for AIoT Services.

3. NEF sends AIoT Service Request message to AIoTF, and the Request message includes Inventory and Command service operation, Device ID info and Area info, etc.

4. AIoTF determines the A-RAN for AIoT Services.

5. AIoTF sends AIoT Service Request message to A-RAN, and the Request message includes Inventory and Command service operation, Device ID info, etc.

6. A-RAN sends Initial Trigger Message (Device ID info) to AIoT Device.

7. AIoT Device performs Random Access procedure.

8. AIoT Device sends AIoT Service Response message to AIoTF via A-RAN, and the Response message includes Device ID.

9. AIoTF performs validation and/or authentication of AIoT Device.

10. AIoTF registers AIoT Device in to UDM.

11. AIoTF sends Command Request message to AIoT Device via A-RAN.

12. AIoT Device sends Command Response message to AIoTF via A-RAN, and the Response message includes Command Result info.

13-14. AIoTF sends AIoT Service Response message to AF via NEF.

The S2-2409000 tries to merge the S2-2408250 and S2- 2408296, and made some update based on the drafting session discussion.

The S2-2408250 proposed the following conclusions for KI#3:

8.3	Conclusions for key issue 3

It is proposed that the service principles below be considered for the normative work:

Editor's note:	Additional conclusions are FFS.

- Common for topology 1 and 2:

- Topology 1:

- Topology 2:

- The UE reader(s) is selected by the new NF responsible for AIoT service handling (e.g., AIoTF) based on the location information provided by the AF, UE authorization information, UE capability, UE related analytics, UE expected behaviours.

Editor's note: 	Whether the BS selects the UE reader(s) indicated by the CN as they are, or selects some or none of them based on the radio conditions, will be further discussed with RAN WG3.

- The new NF responsible for AIoT service handling (e.g., AIoTF) creates and maintains a binding table that indicates the association information between the AIoT devices and their UE reader(s) in order to avoid performing the UE selection procedure each time the 5GC receives the AIoT request for the same AIoT devices.

- The UE reader(s) forwards the data read from the AIoT devices to the AIoTF, over either CP or UP path. The UL data forwarding method is determined by the new NF responsible for AIoT service handling (e.g., AIoTF), considering the UE’s capability and the application requirements (e.g., aggregated data volume, latency, communication frequency)

- The data read from the AIoT devices can be aggregated by the UE reader(s) and/or the new NF responsible for AIoT service handling (e.g., AIoTF) before responding to the consumer AF.

- The selected UE(s) is configured with the necessary information for communicating with the AIoT devices and the CN, including the UE role, UP data forwarding method, aggregation policy, target reporting destination.

The S2- 2408296 proposed the following conclusions for KI#3:

8.Z	Conclusions for KI#3 Support of Ambient IoT Services

Regarding Support of Ambient IoT Services, it is proposed to move to normative phase the following principles:

-	Shared PDU Session establishment is used to support Data Transfer between AF and Devices through MNO.

-	Device-to-Device Data Transfer may be used from the Device to Base.

-	Monitoring Coverage of Interesting Area API is supported as an Ambient IoT Service.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1	Conclusions for Key Issue #3: Support of Ambient IoT Services

For Key Issue #3, the following principles are concluded for normative work:

The following principles are applied to both Topology 1 and Topology 2:

-	Based on the AF request, the AIoTF requests one of three AIoT Services operations (Inventory only, Command only, Inventory and Command) to BS Reader or UE Reader.

-	The AIoTF provides Filter information to the BS Reader or UE Reader for the AIoT Services.

The following principles are applied to Topology 1:

-	AF request one of the AIoT Services operations to AIoTF via NEF.

-	AIoTF requests one of the AIoT Services operations to BS Reader.

-	AIoT Device responds to AIoTF with service operation result.

-	AIoTF responds to AF via NEF with service operation result.

The following principles are applied to Topology 2:

- The UE reader(s) is selected by the new NF responsible for AIoT service handling (e.g., AIoTF) based on, e.g. the location information provided by the AF, UE authorization information, UE capability, UE related analytics, UE expected behaviours, Device capability.

Editor's note: 	Whether the BS selects the UE reader(s) indicated by the CN as they are, or selects some or none of them based on the radio conditions, will be further discussed with RAN WG3.

- The new NF responsible for AIoT service handling (e.g., AIoTF) creates and maintains a binding table that indicates the association information between the AIoT devices and their UE reader(s) in order to avoid performing the UE selection procedure each time the 5GC receives the AIoT request for the same AIoT devices.

- The UE reader(s) forwards the data read from the AIoT devices to the AIoTF, over either CP or UP path. The UL data forwarding method is determined by the new NF responsible for AIoT service handling (e.g., AIoTF), considering the UE’s capability and the application requirements (e.g., aggregated data volume, latency, communication frequency)

- The data read from the AIoT devices can be aggregated by the UE reader(s) and/or the new NF responsible for AIoT service handling (e.g., AIoTF) before responding to the consumer AF.

- The selected UE(s) is configured with the necessary information for communicating with the AIoT devices and the CN, including the UE role, UP data forwarding method, aggregation policy, target reporting destination.

# * * * * End of Changes * * * *