3GPP TSG-SA WG2#164	S2-2409009

Maastricht, NL, August 19 – 23, 2024	(Revision of S2-2408172)

Source:	Sony

Title:	KI#2, Sol#16: Update to address Editor’s notes

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution addresses editor’s note in solution 16.

# 1.	Discussion

This contribution addresses editor’s notes in solution 16.

----

Editor's note:	Whether a registered AIoT device would responds to a Registration poll message to perform e.g. periodic registration is FFS.

During the study phase other solutions have discussed re-registration and how a AIoT device can detect and track time. When concluding KI#2 and if it is decided to specify a registration/onboarding/activation procedure involving the AIoT device, it can also be decided whether or not an AIoT device should preform re-registration if the AIoT device have not had any communication with the network for a long time.

There is also a case when the AIoT moves out of the network coverage area and into a new network coverage area. In this case the AIoT device must register in this new network to communicate with the network.

The above EN is replace with text addressing the above.

-----

Editor's note:	Whether or not an AIoT device checks the network ID to a list of preferred network IDs stored in the device is FFS.

This EN is place in both step 0 and step 2. As there is nothing to study, this open issue can be left to the conclusion of KI#2.

The EN in step 0 is rephrased to a NOTE that mentions whether or not a AIoT device a pre-provisioned with a list of preferred network IDs as step 0 is about pre-provisioning. Furthermore, this can be decided when concluding KI#2.

The EN in step 2 is rephrased to a NOTE whether or not an AIoT device checks the network ID to a list of preferred network IDs can be decided when concluding KI#2.

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

## 6.16	Solution #16: AIoT Registration/Onboarding

### 6.16.1	Description

This solution addresses the part of KI#2 related to the AIoT device registration/onboarding. The first bullet and NOTE 1 in solution#1 clause 6.1.2 mention the onboarding and parameter provisioning. This solution provides a procedure to achieve this.

The relation between this solution and solution 1, is that this solution will provide means to securely authenticate the AIoT device and provision the AIoT with necessary parameters to control the temporary ID as described in solution 1 i.e. an efficient way to generate new temporary ID and to enable AIoT device privacy protection. The proposed onboarding procedure requires minimum three messages to be sent over the radio interface, but the following Inventory and command procedures can be achieved with only two messages (one DL trigger + one UL) and fulfil the SA1 requirement in clause 5.2.6 of TS 22.369 [2]:

The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g. location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE.

NOTE 1:	The actual number of physical messages sent over the radio interface between AIoT device and Reader depends on the RAN design. The numbers used above should be seen as the number of logical messages between the AIoT device and Reader.

As the CN NF would get the information of all AIoT devices that the CN NF performed this registration procedure with, it can also be seen as an initial Inventory collection by the network. This Inventory information can be exposed to an AF subscribing to such exposure service.

The solution is based on the following assumptions:

-	The AIoT device is not capable to initiate the registration procedure. The Network will trigger AIoT devices to register.

-	The AIoT reader (UE or Base station) periodically or on-demand broadcasts a message, Registration poll message, that triggers nearby unregistered AIoT device(s) to respond. The Registration poll message is assumed to be non-AIoT device specific to allow all unregistered AIoT device(s) to respond in an area.

-	An AIoT device does not respond to the Registration poll message if it is already registered in the network or the PLMN is not allowed.

NOTE 2: There may not be a requirement to respond to a Registration poll message when already registered in the network. When concluding KI#2 it can be decided if an AIoT device should respond Registation poll message if it has been a long time since last communication with the network i.e., Inventory or Command.

In case the AIoT device is registered to a network but have not received any Registration poll message from this network for some time, then the AIoT device should consider itself unregistered and perform registration/onboarding with a new network after receiving a Registration poll message from this network.

NOTE 3: It cannot be assumed that an AIoT device has a running clock to track time. However, an AIoT device could determine the lapsed time based on broadcasted time by the Reader.

-	The AIoT device responds with a Registration Request that includes e.g. device owner ID and concealed device ID. This will allow the CN NF to retrieve further information to proceed with the registration/onboarding of the AIoT device. If the CN NF was not able to retrieve the required information or the AIoT device was not possible to authenticate and authorize, the AIoT device will be rejected to register in the network and shall refrain from trying to register again in this network.

-	During the registration procedure the CN NF will provision the AIoT device with e.g. security parameters, initial TempID or parameters to derive the initial Temp ID i.e. parameters for the Temp ID generation algorithm as discussed in clause 6.1.

-	An AIoT device has two states: Unregistered or Registered. The AIoT device will remain in Unregistered state until the AIoT device receives the Registration Accept message.

### 6.16.2	Procedures

CN NF is used to name the network function that performs the necessary task to register/onboard an AIoT device to the network. The CN NF will be defined together with the system architecture design to support the Ambient IoT in 5GC.

Figure 6.16.2-1: Registration/onboarding procedure for an AIoT device.

0.	It is assumed that the AIoT device is pre-provisioned by the device owner with information to be used when register/onboarding to a network. The information includes a device unique identifier (unique at owner level, not necessary globally unique), device owner ID and security IE.

NOTE 1: Whether or not an AIoT device is pre-provisioned with a list of preferred network IDs can be decided when concluding KI#2.

It is assumed that the device owner and the MNO has an SLA. A device owner AF may have provisioned the network with information needed to onboard an AIoT device(s) or a URL or FQDN to establishing IP connection to an AF that holds the device unique onboarding information.

1.	The AIoT reader (UE or base station) broadcasts a Registration poll message that includes e.g. the PLMN ID or NID. The broadcast can either be an immediate broadcast triggered by CN NF or a scheduled/periodic broadcast configured by the CN NF.

Editor's note:	The format of the Registration poll message needs to be defined in cooperation with RAN WG.

2.	An AIoT device that is not registered in the network responds with a Registration Request (device owner ID, concealed device ID). The Reader forwards the Registration request message to the CN NF.

NOTE 2: Whether or not an AIoT device checks the network ID to a list of preferred network IDs stored in the device can be decided when concluding KI#2.

Editor's note:	How the concealed device unique ID is created will be defined by SA3.

3.	The CN NF may hold AIoT device owner information or can retrieve AIoT device owner information from the UDM/UDR using the received device owner ID. In case the AF has provisioned the network with the device unique information needed to onboard the AIoT device, then step 4 can be skipped.

4.	The CN NF uses the retrieved AIoT device owner information to establish a secure IP connection with the AF to retrieve the device unique onboarding information by providing the concealed device ID to the AF. The AF must be able to un-conceal the device ID and verify that the device ID is part of the owner's inventory list. The AF provides the CN NF with authentication/authorization result and device unique information needed to onboard the AIoT device e.g. device security information.

5.	In case the AF have provisioned the network with device unique onboarding information in step 0, then the CN NF uses the retrieved information from UDM/UDR to authenticate and authorize the AIoT device.

6.	The CN NF sends a Registration response (Accept/Reject, security parameters). The AIoT device uses the security parameters together with the pre-provisioned security IE to e.g. derive an initial Temp ID as proposed in solution 1. Additional AIoT configuration can be provision at this step.

### 6.16.3	Impacts on services, entities and interfaces

Impacts on existing entities:

CN NF:

-	Configure/trigger the Reader to broadcast Registration poll message.

-	Retrieve device owner information and device unique onboarding information.

-	Accept or reject the AIoT device registration/onboarding request.

AIoT device:

-	Receive a Registration poll message.

-	Transmit a Registration Request message to the Reader.

-	Receive the Registration accept message with configuration parameters.

-	Manage state transition between Unregistered and Registered.

AIoT Reader (UE or Base station):

-	Broadcast Registration poll (on-demand or periodically configured).

-	Transparently forward any messages between the AIoT device and the CN NF.

UDM/UDR:

-	Data management of device owner information.

-	Be provisioned with device owner information.

# >>>>END OF CHANGES<<<<