3GPP TSG-WG SA2 Meeting #164	S2-2409023

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-2408436, 8766)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0474 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Unicast link profile for UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PC5 unicast link for 5G ProSe direct communication is associated with a Unicast Link Profile. PC5 unicast link used for UE-to-Network Relay is associated with a Unicast Link Profile. Thus, Unicast Link Profile for UE-to-UE Relay also needs to be described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the Unicast Link Profile for UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarification on the Unicast Link Profile for UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.7.1	Common part for Layer-2 link management over PC5 reference point for 5G ProSe UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.1 and clause 6.7.2:

-	The Direct Communication Request message over the first hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of source 5G ProSe End UE: the identity of the source 5G ProSe End UE requesting relay operation.

-	User Info ID of 5G ProSe UE-to-UE Relay: the identity of the UE-to-UE Relay provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure.

-	(optional) Destination Layer-2 ID of target 5G ProSe End UE: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

-	ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the first hop PC5 link establishment (see TS 33.503 [29]).

The Direct Communication Request message over the second hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of source 5G ProSe End UE.

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay.

-	ProSe Service Info: the information about the ProSe identifier(s).

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the second hop PC5 link establishment (see TS 33.503 [29]).

The Direct Communication Accept message over the second hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE.

The Direct Communication Accept message over the first hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay.

The Link Modification Request message over the first hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure.

-	(optional) Destination Layer-2 ID of target 5G ProSe End UE: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

The Link Modification Request message over the second hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of source 5G ProSe End UE.

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE.

The Link Modification Accept message over the second hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE.

The Link Modification Accept message over the first hop PC5 reference point includes:

-	user info (i.e. Application Layer ID) of target 5G ProSe End UE.

Each end-to-end unicast link for 5G Layer-2 ProSe UE-to-UE Relay is associated with a Unicast Link Profile, which includes:

-	user info (i.e. Application Layer ID) and Layer-2 ID of source 5G ProSe End UE; and

-	user info (i.e. Application Layer ID) and Layer-2 ID of target 5G ProSe End UE; and

-	Relay Service Code; and

-	information about PC5 QoS Flow(s).

Each PC5 unicast link for 5G Layer-3 ProSe UE-to-UE Relay over the first hop is associated with a Unicast Link Profile, which includes:

-	user info (i.e. Application Layer ID) and Layer-2 ID of the 5G ProSe End UE; and

-	User Info ID and Layer-2 ID of 5G ProSe UE-to-UE Relay; and

-	Relay Service Code; and

-	the network layer protocol and the information about PC5 QoS Flow(s).

Each PC5 unicast link for 5G Layer-3 ProSe UE-to-UE Relay over the second hop is associated with a Unicast Link Profile, which includes:

-	User Info ID and Layer-2 ID of 5G ProSe UE-to-UE Relay; and

-	user info (i.e. Application Layer ID) and Layer-2 ID of the 5G ProSe End UE; and

-	Relay Service Code; and

-	the network layer protocol and the information about PC5 QoS Flow(s).

The Unicast Link Profile shall be updated accordingly after a Layer-2 link modification or Layer-2 link identifier update.

# * * * * End of changes * * * *