3GPP TSG-SA2 Meeting #164	S2-2409068

# Maastricht, Netherlands, August 19 - 23, 2024	(revision of S2-2407991, S2-2408785)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.204 | CR | 0120 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MPS priority for SMS over IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, T-Mobile USA, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support MPS priority for SMS over IP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: Adds a reference to TS 23.401. Second change: Adds Resource-Priority handling to the list of IP-SM-GW functions. Third change: Adds storing the MPS for Messaging indication to the list of HLR/HSS functions. Fourth change: In the "Service-level Interworking: IM or CPM capable UE sends an Instant Message to an SMS user", if the P-CSCF received an indication from the S-CSCF that MPS for Messaging is set (enabled) for the UE in the HSS, the P-CSCF sets the Resource Priority information. If the IP-SM-GW receives a SIP MESSAGE with an MPS RPH value, the IP-SM-GW sets the DSCP, Diameter priority or Message Priority header to a value appropriate for MPS. Fifth change: In the "Service-level interworking: IM or CPM capable UE sends an Instant Message to an SMS user with Interworking in the terminating side", the P-CSCF checks whether MPS for Messaging is enabled for the originating UE in the HSS and if so, sets the RPH. If the IP-SM-GW receives a SIP MESSAGE with an MPS RPH value, the IP-SM-GW sets the DSCP, Diameter priority or Message Priority header to a value appropriate for MPS. Sixth change: In the "Service-level interworking: IM or CPM user receives Short Message from an SMS user", add checks for MPS for Messaging enabled for the UE in the HSS, priority handling and transport level priority marking by the SMS-GMSC and IP-SM-GW. There's no need to specify Paging Policy Differentiation as was specified in TR 23.700-75. It's not needed for the AMF to decide to page with priority. The AMF keys on the ARP value in the N1N2 message from the SMF to determine whether to page with priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An MPS subscribed UE will not have priority for MO and MT SMS over IP. An SMS over IP terminating to a UE with MPS for Messaging enabled will not page with priority. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3.1.1, 5.3.2, 6.7, 6.13, 6.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***** First change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS) Point to Point (PP)".

[3]	Void.

[4]	Void.

[5]	Void.

[6]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[7]	Void.

[8]	Void.

[9]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[10]	Void.

[11]	Void.

[12]	OMA-TS-SIMPLE_IM-V1_0: "Instant Messaging using SIMPLE", http://www.openmobilealliance.org/.

[13]	IETF RFC 5438: "Instant Message Disposition Notification", February, 2009.

[14]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[15]	OMA-RD-CPM-V1_0: "CPM Requirements", http://www.openmobilealliance.org/.

[16]	OMA-AD-CPM-V1_0: "CPM Architecture", http://www.openmobilealliance.org/.

[17]	OMA-TS-CPM_Conv_Fnct-V1_0: "CPM Conversation Functions", http://www.openmobilealliance.org/.

[18]	3GPP TS 22.340: "IP Multimedia System (IMS) messaging; Stage 1".

[19]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)", September 2007.

[20]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[21]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[22]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[23]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[24]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[XX]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

# ***** Second change *****

### 5.3.1	IP-Short-Message-Gateway (IP-SM-GW)

#### 5.3.1.1	General

The IP-SM-GW shall provide the protocol interworking for delivery of the short message between the IP-based UE and the SMS-SC. The message is routed to the SMS-SC for delivery to the SMS-based user or the message is received from the SMS-SC of an SMS-based UE for delivery to an IP-based UE.

The general functions of the IP-SM-GW are:

-	to determine the domain (CS/PS or IMS) for delivery of a Short Message;

-	to connect to the SMS-GMSC using established MAP or Diameter based protocols, appearing to the SMS-GMSC as an MSC, SGSN, MME or SMSF using the E, Gd/Gdd or SGd interfaces;

-	to respond to Send Routeing Information for Short Message requests made by the SMS-GMSC, and forwarded from the HSS, with its own address;

-	to connect to the SMS-IWMSC using established MAP or Diameter based protocols, appearing to the SMS-IWMSC as an MSC, SGSN, MME or SMSF using the E, Gd/Gdd or SGd interfaces;

-	to connect to the HSS using established MAP or Diameter based protocols, to obtain the address of MSC/SGSN/MME/SMSF address(es) for SM termination in CS/PS;

NOTE:	The IP-SM-GW need not support all of the functionality defined in MAP in TS 29.002 [6].

-	to acquire and maintain knowledge of the association between the MSISDN, IMSI and the address of the S-CSCF serving of the user;

-	to check that it has a valid address in SMS for the sender as well as the recipient when receiving an IMS message for an SMS user. The IP-SM-GW shall obtain a valid address for both from the SIP headers of the IMS message (e.g. the sender would be identified in the asserted id in form of TEL URI);

-	for terminating procedures:

-	if TEL URI is available, it maps the recipient's address from an MSISDN/IMSI to TEL URI format when receiving an SMS for an IP-based UE, and then it is the responsibility of the IMS core to perform any further mapping towards a SIP URI as required;

-	if TEL URI is not available, it maps the recipient's address from an IMSI to SIP URI format when receiving an SMS for an IP-based UE.

-	to act as an Application Server towards the IMS core;

-	to perform domain selection to choose the appropriate domain to deliver a message to a recipient and to obtain the MSC, SGSN, MME and/or SMSF addresses from the HSS; and

-	to manage flags indicating user availability for SMS termination in the HSS and in IP-SM-GW.

-	for MSISDN-less SMS operation:

-	if the recipient is not addressed with MSISDN but with SIP-URI, the originating IP-SM-GW sends the SM toward to the target IMS network,

-	if delivery to target UE failed, the terminating IP-SM-GW returns MSISDN-less correlation ID to allow the originating IMS network to retry later,

-	if originating IP-SM-GW receives temporary delivery failure with MSISDN-less correlation ID from target IMS network, it forwards the SM along with the MSISDN-less correlation ID to SMSC for store and forwarding.

-	to handle Resource-Priority information, including setting the transport priority of an outgoing Short Message.

# ***** Third change *****

### 5.3.2	HSS/UDM

In order to support SMS over generic IP access, the HSS/UDM shall support the following functions:

-	storing the pre-configured address of the IP-SM-GW on a subscriber basis;

NOTE:	If all subscribers are assigned to a single IP-SM-GW address, the IP-SM-GW address does not need to be pre-configured in the HSS.

-	handling an indication that the terminal is registered with an IP-SM-GW for delivery of SMS;

-	storing the SMSF address when the SMSF registers with the UDM as specified in TS 23.502 [24];

-	responding to the "send routing information for short message" query from IP-SM-GW with the address of the MSC/SGSN/MME/SMSF;

-	forwarding the Send Routeing Information for Short Message, from an SMS-GMSC, towards the IP-SM-GW and forwarding any responses to the originator of the Send Routeing Information for Short Message;

-	returning the IMSI and the MSC, SGSN, MME and/or SMSF addresses as a response to Send Routeing Information for Short Message required from IP-SM-GW;

-	alerting the SCs stored in the message waiting data when the terminal is registered with an IP-SM-GW for delivery of short message;

-	reporting notification to the IP-SM-GW of the reachability of a UE at the transport layer after a delivery failure;

-	accepting delivery status reports from IP-SM-GWs instead of SMS-GMSC.

-	storing the MPS for Messaging indication, as specified in TS 23.401 [XX] and TS 23.501 [23].

-	setting the transport priority for all messages related to MPS for Messaging.

# ***** Fourth change *****

## 6.7	Service-level Interworking: IM or CPM capable UE sends an Instant Message to an SMS user

Figure 6.7: Successful IM origination to SMS procedure

1)	The UE registers to S-CSCF according the IMS registration procedure.

2)	UE submits the Instant Message to the S-CSCF using an appropriate SIP method. The UE may request to hide its Public User Identity from the recipient within the Instant Message, as described in OMA-TS-SIMPLE_IM-V1_0 [12] or in OMA-TS-CPM_Conv_Fnct-V1_0 [17]. If the P-CSCF (not shown in the figure) has received an indication from the S-CSCF that MPS for Messaging is set (enabled) for the UE in the HSS (see TS 23.228 [9]), the P-CSCF sets the Resource-Priority information on the Instant Message to a value appropriate for MPS, sets the transport priority of the Instant Message to a value appropriate for MPS and handles the Instant Message with priority.

3)	S-CSCF forwards the Instant Message to IP-SM-GW based on stored iFC.

NOTE 1:	Subscribers with no subscription for service level interworking will not be provided with the relevant iFCs.

4)	The IP-SM-GW shall decide whether to perform service-level interworking depending on SIP request header (e.g. Request-URI), operator policy, when the Instant Message is not routable in the IMS. If an ISF is deployed in the network, the ISF may take the decision to interwork using SMS and sends the message to the IP-SM-GW to perform the interworking. If IP-SM-GW decided to perform service-level interworking, the IP-SM-GW performs service authorization based on the stored subscriber data retrieved from the HLR/HSS at the time of the registration procedure as specified in clause 6.1. The IP-SM-GW shall check whether the originating subscriber is authorised to use the interworking service. If the result of service authorization is negative, the IP-SM-GW shall not forward the message, and shall return the appropriate error information to the UE in a failure report. Otherwise, the IP-SM-GW shall use the SC Address in the subscriber data retrieved from the HSS at registration or provisioned by configuration and translates the Instant Message to a Short Message (SMS- SUBMIT) carrying SC Address, then forwards it towards SMS-SC (SC Address) via the SMS-IWMSC (as described in TS 23.040 [2]). If the size of the content of the Instant Message is larger than the size of the content that one Short Message could transfer, the IP-SM-GW shall split the content of the Instant Message into several parts, translate them to concatenated Short Messages, and forward the concatenated Short Messages to the SMS-SC as described in TS 23.040 [2]. If the sender of the Instant Message requests to hide its Public User Identity from the recipient and operator policy allows for this, the IP-SM-GW shall anonymize the identity of the user to the recipient. Otherwise, if operator policy prohibits this, the IP SM GW shall return an appropriate error to the user.

If the Resource-Priority information on the Instant Message has a value appropriate for MPS, the IP-SM-GW sets the transport priority of the outgoing Short Message to a value appropriate for MPS and handles the Short Message with priority.

5)	If service authorization is successful, the IP-SM-GW acknowledges the Instant Message.

6)	Instant Message acknowledgement is forwarded by S-CSCF to UE.

NOTE 2:	Steps 5 and 6 can occur anytime after the subscriber authorization check has been performed by the IP-SM-GW.

7)	The SMS-IWMSC forwards the Short Message (SMS- SUBMIT) to the SMS-SC (see TS 23.040 [2]).

8)	The SMS-SC sends a Submit report (SMS-SUBMIT REPORT) to the SMS-IWMSC (see TS 23.040 [2]).

9)	SMS-IWMSC sends a Submit report to IP-SM-GW (see TS 23.040 [2]).

NOTE 3:	The procedure can end in step 9. Steps 10 to 13 occur only if the IM user requested a processing notification in the Instant Message sent in step 2, as described in IETF IMDN RFC 5438 [13].

10)	IP-SM-GW translates the received Submit report to an appropriate Instant Message, and forwards it to the S-CSCF. If the IP-SM-GW sent concatenated Short Messages to SMS-SC in step 4, the IP-SM-GW should wait for the last Submit report, and translate the last Submit report to an appropriate Instant Message, and forward it to the S-CSCF.

11)	S-CSCF sends the translated Instant Message to the UE.

12)	UE acknowledges the translated Instant Message.

13)	Acknowledgement of the translated Instant Message is forwarded by S-CSCF to IP-SM-GW.

# ***** Fifth change *****

## 6.13	Service-level interworking: IM or CPM capable UE sends an Instant Message to an SMS user with Interworking in the terminating side

This procedure describes the delivery of an Instant Message to a registered or an un-registered IMS subscriber. For the unregistered case, the S-CSCF forwards the Instant Message to the IP-SM-GW based on the unregistered iFC of the subscriber.

Figure 6.13: Successful IM terminating to SMS procedure with Interworking in the Terminating Side

1)	UE submits an Instant Message, destined to another IM user or CPM user in another IMS domain, using an appropriate SIP method. The UE may request to hide its Public User Identity from the recipient within the Instant Message, as described in OMA-TS-SIMPLE_IM-V1_0 [12] and in OMA-TS-CPM_Conv_Fnct-V1_0 [17]. If the P-CSCF (not shown in the figure) has received an indication from the S-CSCF that MPS for Messaging is enabled for the originating UE in the HSS (see TS 23.228 [9]), the P-CSCF sets the Resource-Priority information on the Instant Message to a value appropriate for MPS, sets the transport priority of the outgoing Instant Message and handles the Instant Message with priority.-

2)	The S-CSCF resolves the destination domain and routes the message towards the S-CSCF in the terminating network ("Terminating S-CSCF").

3)	The terminating S-CSCF forwards the Instant Message to the IM AS ("Terminating IM AS") or to the CPM AS ("Terminating CPM AS") based on stored iFC.

NOTE:	Depending on iFC configuration, it is possible that the IM AS or CPM AS is not triggered for the unregistered subscribers.

4)	The terminating IM AS or CPM AS invokes terminating IM or CPM services as applicable for the destination IM or CPM user.

5a)	The IM AS or CPM AS can forward the Instant Message back to the terminating S-CSCF, e.g. the terminating IM user is offline or the CPM user has no terminating interworking user preferences.

5b)	In the case of the CPM AS, if terminating interworking user preferences are set, the Instant Message is routed directly to the IP-SM-GW in which case the procedure continues with step 7.

6)	The terminating S-CSCF forwards the Instant Message to the IP-SM-GW, e.g. based on stored iFC.

7)	If the user is authorized, the IP-SM-GW performs service-level interworking by converting the Instant Message to Short Message. The IP-SM-GW shall obtain the routeing information for the UE from the HLR/HSS and deliver the message to the UE. If the sender of the Instant Message requests to hide its Public User Identity from the recipient and operator policy allows for this, the IP-SM-GW shall anonymise the identity of the user to the recipient. Otherwise, if operator policy prohibits this, the IP-SM-GW shall return an appropriate error to the user.

8)	The IP-SM-GW obtains the routeing information for the destination UE from the HLR/HSS. The HLR/HSS also returns the indication whether MPS for Messaging is enabled for the UE in the HSS (see TS 23.228 [9]), sets the transport priority of the message and handles the message with priority.

9)	The IP-SM-GW sends the Forward Short Message message to the target MSC/SGSN/MME/SMSF. If the Resource-Priority information on the Instant Message is a value appropriate for MPS, the IP-SM-GW sets the transport priority of the outgoing Short Message to a value appropriate for MPS and handles the Short Message with priority.

10)	The MSC/SGSN/MME/SMSF sends the Short Message to the UE.

11)	The UE acknowledges the receipt of the Short Message.

12)	The MSC/SGSN/MME/SMSF sends a Delivery report (SMS-DELIVER-REPORT) to the IP-SM-GW.

13)	The IP-SM-GW sends OK response to the terminating S-CSCF.

14)	The S-CSCF forwards the OK to the terminating IM AS or CPM AS.

15)	The terminating IM AS or CPM AS forwards the OK response back to the terminating S-CSCF.

16)	The terminating S-CSCF forwards the OK back towards the originating S-CSCF.

17)	The originating S-CSCF forwards the OK to the originating UE.

# ***** Sixth change *****

## 6.14	Service-level interworking: IM or CPM user receives Short Message from an SMS user

An IMS registered user with SIMPLE IM or CPM service receives a Short Message formatted via service-level interworking to an Instant Message.

Figure 6.14: Successful IM termination after service-level interworking

1)	The UE registers to the S-CSCF according to the IMS registration procedure.

2)	The SMS-SC forwards a Short Message to the SMS-GMSC.

3a)	The SMS-GMSC interrogates the HLR/HSS to retrieve routeing information. Based on the pre-configured IP-SM-GW address for the user, the HLR/HSS forwards the request to the corresponding IP-SM-GW. The SMS-GMSC also interrogates the HLR/HSS to check whether MPS for Messaging is enabled for the UE (see TS 23.228 [9]). If the MPS for Messaging is set (enabled) for the UE (see TS 23.228 [9]), the HLR/HSS sets the transport priority of the message and handles the message with priority.

3b)	The HLR/HSS returns the IMSI and the address(es) of the current MSC, SGSN, MME and/or SMSF to the IP-SM-GW for delivery of the SM in CS/PS domain. The HLR/HSS also returns the indication whether MPS for Messaging is set (enabled) for the UE (see TS 23.228 [9]), sets the transport priority of the message and handles the message with priority.

3c)	The IP-SM-GW creates an MT Correlation ID as per TS 23.040 [2], which associates the Routing Info retrieval with the subsequent Forward Short Message messages(s), and stores this along with the IMSI of the receiving subscriber. The IP-SM-GW returns to the SMS-GMSC the address of itself, along with the MT Correlation ID in the IMSI field, as routeing information. The IP-SM-GW returns the indication whether MPS for Messaging is set (enabled) for the UE in the HLR/HSS (see TS 23.228 [9]). If MPS for Messaging is enabled for the UE, the IP-SM-GW sets the transport priority on the message and handles the message with priority.

4)	The SMS-GMSC delivers the Short Message to the IP-SM-GW in the same manner that it delivers the Short Message to an MSC, SGSN, MME or SMSF, including the MT Correlation ID received from the IP-SM-GW, in place of the IMSI. If MPS for Messaging is enabled for the UE in the HLR/HSS (see TS 23.228 [9]), the SMS-GMSC sets the transport priority for MPS and handles the Short Message with priority.

5)	The IP-SM-GW checks whether the recipient is authorized for the interworking service.

NOTE 1:	The IP-SM-GW will determine whether the transport-level interworking or the service-level interworking is to be performed according to clause 6.8.3.

6)	If the user is authorized for service-level interworking, the IP-SM-GW converts the Short Message to an Instant Message. It sends the Instant Message using the appropriate SIP method towards the S-CSCF. If the MPS for Messaging indication for the UE is activated, the IP-SM-GW applies an MPS appropriate Resource-Priority information to the SIP message, sets the transport priority of the message and handles the message with priority.

7)	The S-CSCF forwards the Instant Message to the UE.

The mechanim defined in TS 23.401 [70], clause 5.3.4.3 and TS 23.502 [24], clause 4.2.3.3 ensures that if the UE needs to be paged, the MME/AMF pages the UE with priority.

8)	The UE acknowledges the SIP request to the S-CSCF.

9)	The S-CSCF forwards the acknowledgement of the SIP request to the IP-SM-GW.

10)	The IP-SM-GW acknowledges the Forward Short Message to the SMS-GMSC.

11)	The SMS-GMSC sends a Delivery report (SMS-DELIVER-REPORT) to the SMS-SC.

12)	The IP-SM-GW may send a Report SM Delivery Status to the HLR/HSS.

13)	The SMS-GMSC may send a Report SM Delivery Status to the HSS. The HSS shall ignore the information provided in this report.

# ***** End of changes *****