3GPP SA WG2 Meeting #164	S2-2409081

# Maastricht, NL, 19-23 August 2024	               										    (was S2-2408005)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0459 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe multi-hop UE-to-Network Relay connection management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, AT&T, LG Electronics, FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution added the support of 5G ProSe multi-hop UE-to-Network Relay communication and link management based on Model A discovery and the conclusions in TR 23.700-03. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of the operations for the support of multi-hop UE-to-Network Relay link management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper support of 5G ProSe multi-hop UE-to-Network Relay feature in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 6.4.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.304 CR 0458 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented together with CR#0458 to TS 23.304, due to cross referencing of the clauses. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.4.3.x	Layer-2 link management over PC5 reference point for 5G ProSe multi-hop UE-to-Network Relay

##### 6.4.3.x.1	Layer-2 Link establishment and management (based on Model A discovery)

Each of the 5G ProSe Intermediate UE-to-Network Relay establishes a Layer-2 Link with its immediate parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. RSC based configuration for the 5G ProSe Intermediate UE-to-Network Relay indicates whether it should establish the Layer-2 Link with a parent relay before it can participate in the discover operation.

Layer-2 link management procedures as defined in clause 6.4.3.6 for 5G ProSe UE-to-Network Relay operation are used. When establishing the Layer-2 link with its parent relay (either a 5G ProSe UE-to-Network Relay or another 5G ProSe Intermediate UE-to-Network Relay), the 5G ProSe Intermediate UE-to-Network Relay takes the role of the 5G ProSe Remote UE and its parent relay takes the role of 5G ProSe UE-to-Network Relay. The Layer-2 link is established per RSC.

The 5G ProSe Remote UE uses the procedure defined in clause 6.4.5.6 to establish a Layer-2 Link with the selected 5G ProSe Intermediate UE-to-Network Relay. The Intermediate UE-to-Network Relay takes the role of of 5G ProSe UE-to-Network Relay in the procedure.

NOTE:	In the case of Layer-3 UE-to-Network Relay operation, only the root 5G ProSe UE-to-Network Relay needs to establish the PDU session with the network.

When an Intermediate UE-to-Network Relay has a child Intermediate UE-to-Network Relay or a 5G ProSe Remote UE connected, it updates the Layer-2 link with its parent relay, using the procedure defined in clause 6.4.3.4, with the following enhancements:

-	The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s). The Link Modification Request message contains a list of 5G ProSe Remote UE (Remote User ID, Remote UE Info) and the corresponding PC5 QoS Flow associated with each 5G ProSe Remote UE. The Remote User ID and Remote UE Info are as defined in clasue 6.5.1.1.

An Intermediate UE-to-Network Relay triggers a link modification procedure towards its parent relay if a new Remote User ID is received from its child relay during the link modification procedure.

The 5G ProSe UE-to-Network Relay's Remote UE reporting procedure includes all the Remote UE information reported by its child relays.

##### 6.4.3.x.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay needs to act as an IP Router.

The actual IP address allocation is performed by the 5G ProSe UE-to-Network Relay, as defined in clause 6.5.1.

To support DHCP based IP address configuration, the 5G ProSe Intermediate UE-to-Network Relay acts as a DHCPv4 or DHCPv6 proxy. To support IPv6 SLAAC, the 5G ProSe Intermediate UE-to-Network Relay also relays the Router Solicitation and Router Announcement messages between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

The 5G ProSe Intermediate UE-to-Network Relay builds its local IP routing table based on the Remote UE info from the link modification procedures as described in clause 6.4.3.x.1. The local IP routing table is per RSC and used based on RSC of the link the packet is received.

##### 6.4.3.x.3	Relay reselection and mobility support

There are two types of situations for parent relay reselection, i.e. for a single 5G ProSe Remote UE, and for a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs.

For a single 5G ProSe Remote UE, when the connection with an old parent relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold, the 5G ProSe Remote UE triggers the reselection of the relays that offers the same service, identified by the same RSC.

Figure 6.4.3.x.3-1: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays

Figure 6.4.3.x.3-1 depicts the Relay reselection operation for this situation. When the 5G ProSe Remote UE has lost the connection or has an imminent connection loss/deteriorated connection with the 5G ProSe Intermediate UE-to-Network Relay-1, the 5G ProSe Remote UE performs relay reselection by using the relay discovery procedure in clasue 6.3.2.x. The 5G ProSe Remote UE discovered the 5G ProSe Intermediate UE-to-Network Relay -2 that offers the same RSC.  In case there are multiple 5G ProSe Intermediate UE-to-Network Relays discovered, the 5G ProSe Intermediate UE-to-Network Relay that offers connection to the same 5G ProSe UE-to-Network Relay is prioritized.

5G ProSe Remote UE establishes the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2 as specified in clause 6.4.3.x.1. After the connection establishment, the Intermediate UE-to-Network Relay-2 uses the Layer-2 Link Modification procedure to update the 5G ProSe UE-to-Network Relay regarding the 5G ProSe Remote UE. If the same 5G ProSe UE-to-Network Relay is used, it switches the connection/routing context for the 5G ProSe Remote UE from the Layer-2 link with the 5G Intermediate UE-to-Network Relay-1 to that of the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2.

The 5G ProSe Remote UE tears down the link with the 5G ProSe Intermediate UE-to-Network Relay-1 (or the link failure is detected), which will trigger the 5G ProSe Intermediate UE-to-Network Relay-1 to update the 5G ProSe UE-to-Network Relay to remove its association with 5G ProSe Remote UE.

If the same 5G ProSe UE-to-Network Relay is selected after the relay reselection, the 5G ProSe Remote UE can keep its IP address/prefix (in case of Layer-3 UE-to-Network relay).

If a new 5G ProSe UE-to-Network Relay is used after the relay reselection, the procedures as that described in clause 6.4.3.x.1 will be executed. In this case, IP address/prefix changes happes (in case of Layer-3 UE-to-Network relay).

The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure.

End of CHANGEs