3GPP TSG-SA WG2 Meeting #164	S2-2409085

# Maastricht, Netherlands, 19 – 23 August, 2024						  (revision of S2-2407598)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0444 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization and Provisioning for 5G ProSe multi-hop Relays |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Telecom, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-03 clause 8, it is concluded to support multi-hop UE-to-Network Relay and Layer-3 multi-hop UE-to-UE Relay in Rel-19, and authorization and provisioning for multi-hop Relays needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add authorization and provisioning for multi-hop UE-to-Network Relay and Layer-3 multi-hop UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorization and provisioning for multi-hop Relays not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.1a, 5.1.4.1a (new), 5.1.4.2.1a (new), 5.1.4.2.2a (new), 5.1.5.1a (new), 5.1.5.2.1a (new), 5.1.5.2.2a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

5.1.1	General

In 5GS, the parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay, 5G ProSe UE-to-UE Relay, 5G ProSe Layer-3 multi-hop UE-to-Network Relay and 5G ProSe Layer-3 multi-hop UE-to-UE Relay services may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided or updated by the ProSe Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

If the same parameters described in clauses 5.1.2.1, 5.1.3.1, 5.1.4.1 and 5.1.5.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided or updated by the PCF (including parameters determined by the PCF itself and parameters provided by the ProSe Application Server to the PCF as specified in clause 6.2.5);

-	provided or updated by the ProSe Application Server via PC1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

The parameters provided or updated by the ProSe Application Server via PC1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The ProSe Application Server can provision the same ProSe parameters via 5GC as specified in clause 6.2.5 or directly to the UE via PC1 reference point and can revoke (e.g. delete) the ProSe parameters via 5GC as specified in clause 6.2.5 in order for the provisioning via PC1 reference point to take effect.

The basic principles of service authorization and provisioning for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay, 5G ProSe UE-to-UE Relay, 5G ProSe Layer-3 multi-hop UE-to-Network Relay and 5G ProSe Layer-3 multi-hop UE-to-UE Relay services are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use 5G ProSe Direct Discovery.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to use 5G ProSe Direct Communication.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-Network Relay. Authorisation for 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5GC via 5G ProSe UE-to-Network Relay (i.e. to act as 5G ProSe Remote UE). Authorisation to access via 5G ProSe Layer-2 UE-to-Network Relay and via 5G ProSe Layer-3 UE-to-Network Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe UE-to-UE Relay. Authorisation for 5G ProSe Layer-2 UE-to-UE Relay and 5G ProSe Layer-3 UE-to-UE Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5G ProSe UE-to-UE Relay (i.e. to act as 5G ProSe End UE). Authorisation to access via 5G ProSe Layer-2 UE-to-UE Relay and via 5G ProSe Layer-3 UE-to-UE Relay are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 multi-hop Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 multi-hop Intermediate UE-to-Network Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5].

-	The ProSe Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following ProSe Policy/parameters:

-	ProSe Policy/parameters for 5G ProSe Direct Discovery as specified in clause 5.1.2.1;

-	ProSe Policy/parameters for 5G ProSe Direct Communications as specified in clause 5.1.3.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-Network Relay as specified in clause 5.1.4.1;

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 Remote UE as specified in clause 5.1.4.1.

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 UE-to-UE Relay as specified in clause 5.1.5.1.

-	ProSe Policy/parameters for 5G ProSe Layer-2 and/or Layer-3 End UE as specified in clause 5.1.5.1.

-	ProSe Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay as specified in clause 5.1.4.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 multi-hop Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay as specified in clause 5.1.4.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 multi-hop Intermediate UE-to-Network Relay as specified in clause 5.1.4.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay as specified in clause 5.1.5.1a.

-	ProSe Policy/parameters for 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay as specified in clause 5.1.5.1a.

-	The PCF includes the 5G ProSe Policy/parameters in a Policy Section identified by a Policy Section Identifier (PSI) as specified in clause 6.1.2.2.2 of TS 23.503 [9].

In addition to the above, ProSe usage reporting configuration and rules for charging can be (pre)configured in the UE or provided by the PCF.

In addition to the above, the path selection policy can be (pre)configured in the UE or provided by the PCF as defined in clause 5.11. A path preference for ProSe Services can be provided by ProSe Application Server to UDR and may be used by PCF for path selection policy generation and update.

When a 5G ProSe Layer-3 Remote UE is using a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported.

* * * * 2nd Change * * * *

5.1.1a	General principles for applying policy/parameters

For services (i.e. 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay discovery, 5G ProSe UE-to-Network Relay communication, 5G ProSe UE-to-UE Relay discovery, 5G ProSe UE-to-UE Relay communication, 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery, 5G ProSe Layer-3 multi-hop UE-to-Network Relay communication, 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery and 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication) over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for the service, without the need for the UEs to connect to the 5GC to get this initial configuration. The following apply:

-	The provisioning parameters for the service could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

-	The UE shall use radio resources for the service as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for 5G ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell. The UE is allowed to perform the service with another UE not served by the same PLMN;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe that are not operated by the UE's serving cell, as specified in the related Policy/Parameter provisioning, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN and authorization for the service to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN and that cell belongs to a PLMN authorized for the service and provides radio resources for ProSe then the UE shall perform PLMN selection triggered by the service as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for the service the UE shall not use the service;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize the service, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) provisioned in the ME or the UICC for the service if the UICC indicates it is authorized;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.2.1, then the UE shall perform the service using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize the service, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for the service when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for 5G ProSe based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR Sidelink communication defined in TS 38.331 [16] indicates that the service (the service) is available. This is to cover the scenario when e.g. the radio resources used for the service are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform the service over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for the service when the UE attempts to use a carrier frequency configured for the service, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The service is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

* * * * 3rd Change * * * *

5.1.4.1a	Policy/Parameter provisioning for 5G ProSe multi-hop UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 Remote UEs supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

NOTE 2:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

3)	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay to relay Ethernet or Unstructured traffic from 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay by using IP type PDU Session:

-	Mapping of ProSe Service(s) to ProSe Application Server address information (consisting of IP address/FQDN and transport layer port number).

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery with Model A and Model B.

NOTE 3:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	Includes the parameters for 5G ProSe Relay Discovery and for enabling the UE to connect to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC(s) dedicated for emergency service may be provisioned to enable the support of emergency services for multi-hop UE-to-Network Relaying. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement and Relay Discovery Additional Information messages;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic without using N3IWF access, or an indication of N3IWF access, for each ProSe Relay Service Code;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, optionally the ProSe application Traffic Descriptor(s) (as defined in TS 23.503 [9]) to be used for the relayed traffic for each ProSe Relay Service Code;

3)	Policy/parameters for N3IWF selection for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

NOTE 4:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, indicates whether the UE is authorised to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 5:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

NOTE 6:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Intermediate UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay, and in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

2)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 7:	The validity time of these radio parameters is the same as the validity time of the Policy/Parameter listed above for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Policy/parameters related to privacy:

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

* * * * 4th Change * * * *

5.1.4.2.1a	Principles for applying parameters for 5G ProSe multi-hop UE-to-Network Relay discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe multi-hop UE-to-Network Relay discovery service.

* * * * 5th Change * * * *

5.1.4.2.2a	Principles for applying parameters for 5G ProSe multi-hop UE-to-Network Relay communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe multi-hop UE-to-Network Relay communication service.

* * * * 6th Change * * * *

5.1.5.1a	Policy/Parameter provisioning for 5G ProSe Layer-3 multi-hop UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

-	when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay over PC5 reference point.

-	when the UE is not "served by NG-RAN":

-	Indicates whether the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and sending/receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 UE-to-UE Relay discovery with Model A and Model B.

NOTE 2:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and for enabling the UE to connect to the 5G ProSe Layer-3 UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery parameters (Relay Service Code(s), UE-to-UE Relay Layer indicator, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay:

1)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 3:	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

* * * * 7th Change * * * *

5.1.5.2.1a	Principles for applying parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery service.

* * * * 8th Change * * * *

5.1.5.2.2a	Principles for applying parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication

The general principles for applying policy/parameters defined in clause 5.1.1a apply to the 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication service.

* * * * End of Changes * * * *