3GPP TSG-SA2 Meeting #164	S2-2409094

# Maastricht, NL,  19th - 23rd Auguest, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1439 | rev | 01 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to support the conclusion of KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.2 in 3GPP TS 23.700-77 concludes KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of IMS data channel capabilies exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.1, AA.2.4.X (new) and AC.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_SessionManagement | Create | Request/Response | DC AS, NEF |
|  | Update | Request/Response | DC AS, NEF |
|  | Delete | Request/Response | DC AS, NEF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

# * * * * Second change * * * *

### AA.2.4.X	Nimsas_SessionManagement service

#### AA.2.4.X.1	General

Service description: This service enables the consumer NF to create, update and terminate an IMS session with audio, video and/or data channel media.

#### AA.2.4.X.2	Nimsas_SessionMangement_Create service operation

Service operation name: Nimsas_SessionManagement_Create

Description: This service operation enables the consumer NF to create an IMS session with audio, video and/or data channel media.

Inputs, Required: Serving Number, Destination Number, Media Information set.

Serving Number: it is the number served by the DC AS.

Destination Number: it is the called party number.

Media Inforamaiton set includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Type: it indicates the media type used in the created session, and can be Audio, Video or Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it need indicate the codec type the consumer NF requires to use.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P) and the party with wich the P2A ADC is used.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: DC Info.

DC Info indicates the attributes of data channel to be created when initiating the call.

Outputs, Required: Result indication and when the operation is successful: Session ID.

Outputs, Optional: None.

#### AA.2.4.X.3	Nimsas_SessionMangement_Update service operation

Service operation name: Nimsas_SessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as creating or releasing bootstrap data channel(s) or application data channel(s).

Inputs, Required: Session ID, Media operation set.

Media operation set includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	operation type: it indicates adding, updating or removing the corresponding media.

-	media type: it indicates the operated media is audio, video or data channel.

-	media info: it indicates the related media parameters. For different media, it contains different parameters.

-	When the media type is audio or video, it need indicate the codec type the consumer NF requires to use.

-	When the media type is data channel, it need indicate the data channel type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P) and the party with wich the P2A ADC is used.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.X.4	Nimsas_SessionMangement_Delete service operation

Service operation name: Nimsas_SessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

# * * * * Third change * * * *

### XX.X		IMS data channel capability exposure procedures

#### XX.X.1	General Description

There are two scenarios in IMS data channel capability exposure scenarios:

-	Adding data channel to an existing session, which Nimsas_SessionManagement_Update is used

-	Creating a standalone data channel session, which Nimsas_SessionManagement_Create is used.

Editor’s Note: The details of the procedures are FFS.

### XX.X.2	Adding Data Channel to an Existing Session

Figure XX.X.2-1 shows the procedure of adding bootstrap data channel(s) or application data channel(s) to an existing P2A session.

Figure XX.X.2-1: Procedure of adding data channel(s) to an existing P2A session

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to create a data channel with UE A in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to create a data channel with UE A in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding a data channel with UE A. Then the DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

5.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the data channel required by the DC AS with the other existing media descriptions.

6.	The IMS AS-2 sends the re-INVITE request to UE A to negotiate the data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the data channel resource according to the SDP answer in the received 200 OK response.

8.	The MF-2 establishes the data channel with UE A and the DC AS respectively.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

10.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

### XX.X.3	Establishing Standalone Data Channel Session

Figure XX.X.3-1 shows the establishment procedure of a standalone data channel session.

Figure AC.X.4.2-1: Establishment procedure of a standalone data channel session

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF requiring to create a standalnone DC session with UE A.

2.	The NEF queries the HSS for the address of the IMS AS serving the service number of the enterprise if needed. If the service number is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Create request to the IMS AS-2 requiring to create a standalone DC session.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of creating a data channel with UE A. Then the DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The detailed procedure is depicted in step 3~10 in Figure AC.7.1-1.

5.	The IMS AS-2 generates an INVITE request in which the SDP offer contains the media information of the data channel required by the DC AS.

6.	The IMS AS-2 sends the INVITE request to UE A to negotiate the data channel media. Then the UE A returns a 200 OK response with the SDP answer.

7.	The IMS AS-2 interacts with the MF-2 to update the data channel resource according to the SDP answer in the received 200 OK response.

8.	The MF-2 establishes the data channel with UE A and the DCSF-2 respectively.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Create response to the NEF.

10.	The NEF returns a successful Nnef_ImsSessionManagement_Create response to the DC AS.

### XX.X.4	Removing Data Channel from an Existing session

Figure XX.X.4-1 shows the termination procedure of an application data channel in an existing session.

Figure AC.X.4.3-1: Termination procedure of bootstrap data channel

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to terminate the data channel with UE A in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to terminate the data channel with UE A in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of removing the data channel with UE A. Then the DCSF-2 instructs the IMS AS-2 to terminate the data channel by sending a Nimsas_MediaControl request.

5.	The IMS AS-2 generates a re-INVITE request in which the port of "m=application" line for the data channel to be removed is set to zero.

6.	The IMS AS-2 sends the re-INVITE request to UE A to terminate the data channel. Then the UE A returns a 200 OK response with the SDP answer.

If in a P2P session, the IMS AS-2 also sends a re-INVITE request to UE B to terminate the data channel.

7.	The IMS AS-2 interacts with the MF-2 to release the data channel resource according to the SDP answer in the received 200 OK response.

8.	The MF-2 terminates the data channel.

9.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

10.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

# * * * * End of changes * * * *