3GPP TSG-SA2 Meeting #164	S2-2409098

# Maastricht, NL,  19th - 23rd Auguest, 2024								(Revision of S2-2407988)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1418 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of IMS data channel interworking between DCMTSI UE and MTSI UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA2 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Mobile, Huawei, HiSilicon, ZTE, vivo |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution adds the functionality requirements and procedures to support standalone DC establishment based on the KI#3 conclusion in TR 23.700-77. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Provide the procedure for the case of IMS data channel interworking with MTSI UE, when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE. - Update the DCSF, IMS AS, and MF functions to support IMS data channel interworking with MTSI UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC interworking is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.2.2, AC.7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.2.2	Functional entities

AC.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function that provides data channel control logic. The DCSF is not involved in SIP signalling. The DCSF supports the following functionalities:

-	The DCSF receives event reports from the IMS AS and decides whether data channel service is allowed to be provided during the IMS session;

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the MF or MRF via the IMS AS;

-	The DCSF manages transforming between application data channel media and audio/video media when applied and instructs to the MF via the IMS AS;

NOTE 1:	Transforming between application data channel and voice/video refers to the change of transporting protocols which is specified in SA4.

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via MF and/or MRF to the UE based on UE subscription.

-	The DCSF downloads data channel applications from the Data Channel Application Repository;

NOTE 2:	The detailed procedures to download data channel applications to the UE are out of scope of this specification.

-	The DCSF interacts with NEF for data channel capability exposure via N33;

NOTE 3:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control via DC4/DC3, and for traffic forwarding via MDC3/MDC2.

NOTE 4:	Interaction with the DC Application Server for resource control is not specified in this release of the specification.

-	The DCSF interacts with HSS for retrieving and storing DCSF service specific data via N72/Sc.

AC.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores the verified data channel applications which are retrieved by the DCSF when required.

AC.2.2.3	Data Channel Media Function (MF) / MRF

The MF and MRF provide the media resource management and forwarding of data channel media traffic. The MF and MRF provide the following functionalities:

-	The MF and MRF manage the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS;

-	The MF and MRF terminate the bootstrap data channel from the UE and forward HTTP traffic between the UE and DCSF via MDC1;

-	The MF and MRF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs;

-	The MF and MRF relay traffic on the A2P/P2A application data channels between the UE and the DC Application Server via MDC2.

-	The MF handles transforming of data channel media following the media instructions from the DCSF.

AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

AC.2.2.5	S-CSCF

In addition to the functions defined in clause 4.6.3, the S-CSCF is enhanced to support the following functionalities:

-	The S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request from UE.

AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

# * * * * Second change * * * *

AC.7.X	IMS data channel setup Signalling Procedure for interworking with MTSI UE

AC.7.X.1	Bootstrap Data Channel Setup Signalling Procedure

Figure AC.7.X.1-1 depicts a signalling flow diagram for establishing a bootstrap data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE.

Figure AC.7.X.1-1: Bootstrap Data Channel set up Signalling Procedure for IMS data channel interworking with MTSI UE

- UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains SDP offers for the bootstrap data channel with bootstrap DC stream ID.

2.	Data channel management procedure in originating network is executed referring to step2~10 in Figure AC.7.1-1.

3-4.	IMS AS sends the INVITE request to remote network with SDP containing data channel media for terminating network and UE #2.

5-7.	UE#2 and terminating network returns bootstrap DC negotiation result to originating network. The prot number of bootstrap data channel in the SDP answer is set to zero.

8.	IMS AS requests MF to update the local DC media resources accordingly.

9-11.	When IMS AS sends the bootstrap DC negotiation result to DCSF, since the port of bootstrap data channel for UE #2 is set to zero, the DCSF knows that the terminating network or UE #2 does not support or does not use IMS data channel in this session.

12-18.	These procedures are same as step14~20 in Figure AC.7.1-1.

19. The bootstrap data channels have been established only between originating MF and UE#1.

AC.7.X.2	 Signalling Procedure of Application Data Channel Interworking via MF

Figure AC.7.X.2-1 depicts a signalling flow diagram for establishing the application data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, providing interworking of application data channel to MTSI UE via MF. This signaling flow takes the data channel application Real-time Screen Sharing for instance.

Figure AC.7.X.2-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via MF

1-3.	UE#1 initiates a re-INVITE request to establish an application data channel in which the application data channel and data channel application information (for instance, application ID) are contained. The IMS AS validates user subscription data to determine whether this request should be notified to the DCSF as stated in Clause AC.7.2.

4.	After receiving the request, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information as application ID) in the notification and/or DCSF service specific policy.

5-7.	Since the DCSF has known the terminating network or UE #2 does not support or does not use IMS data channel in this session, the DCSF determines that interworking of the application data channel media to MTSI UE is needed, based on the application information, i.e. appliation id. The DCSF invokes Nimsas_MediaControl_MediaInstruction to the IMS AS and instructs the IMS AS to create resources and association for the application data channels on originating side and video media on terminating side. And it also includes the instruction of how to transform data channel media to video media. The IMS AS invokdes Nmf_MediaResourceManagement to the MF to allocate resource and transform data channel media to video media according to DCSF’s instruction.

NOTE:	The media instruction is based on media transforming protocol specified by SA4.

8-9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1. The DCSF stores the media resource information and responds to the request received in step 3.

10-11.	After the media resource reservation, the IMS AS sends a re-INVITE request with the SDP offer for anchoring the video media of UE#2 to the MF.

12-14.	After terminating network side negotiation, UE#2 and terminating network returns 200 OK for the re-NVITE.

15-16.	DCSF is notified about the terminating network media negotiation result.

17-21.	The IMS AS sends 200 OK for re-INVITE initiated by UE#1 and UE#1 returns ACK.

22.	UE#1 initiates SCTP association and DTLS connection establishment procedures and establishes the application data channels with the MF.

23-24.	The IMS AS reports media negotiation result to the DCSF, and reports that anchoring video stream is successful. The DCSF sends a Call Control Request to the MMTel AS indicating it to continue the call process.

25.	The IMS AS sends a Media Resource Management Request to the MF to associate application data channel stream between UE#1 and the MF with the allocated video streams between UE#2 and the MF.

The application data channels between the MF and UE#1 are associated with the RTP video streams between the MF and UE#2. The RTP stream of UE#1's screenshot sequence is received by UE#2 from video streams.

AC.7.X.3		Signalling Procedure of Application Data Channel Interworking via DC AS

Figure AC.7.X.3-1 depicts a signalling flow diagram for establishing an application data channel from DCMTSI UE to DC application server, providing interworking of application data channel to MTSI UE via DC application server.

In the call flow the UEs have already established an IMS audio session, and the DCMTSI UE (UE#1) wants to provide DC application data to MTSI UE (UE#2) during the call, such as transfer a file (e.g. a photo) to UE#2.

Figure AC.7.X.3-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via DC application server

The steps in the call flow are as follows:

1. DC AS subscribes to IMS AS the event that application DC for an DC application that requires interworking is established.

2. IMS session and bootstrap data channel have been established. Selected DC application(s) have been downloaded to UE#1.

The IMS AS and DCSF can know the UE#2 is a MTSI UE based on the port of bootstrap data channel between the DCSF and UE #2 is set to zero in the SDP answer.

NOTE 1:	The UE#2 can be seen as an MTSI UE if UE#2 does not support DC or the network of UE#2 does not support DC.

3-5. Steps 2-3 of clause AC.7.2.1 applies.

The UE#1 may initiate a P2P application DC if the UE#1 does not know the UE#2 is a MTSI UE.

6. After receiving the session event notification, the DCSF determines that IMS DC interworking for the requested DC application is required and perform the P2A application DC establishment procedure.

If the UE#1 initiates a P2P application DC request in step 3, the DCSF may change to P2A application DC.

NOTE 2:	How the DCSF can determine that IMS DC interworking for the requested DC application is required is up to implementation. DCSF can for example determine the interworking is required based on the UE#2 is a MTSI UE, and DC application corresponding to the binding information supports interworking.

7-16. Steps 5-14 of clause AC.7.2.2 applies.

The DCSF may inform the IMS AS the P2A application DC procedure is performed.

17. IMS AS sends a notification to indicate the DC AS that the interworking should be perform for UE#2.

The IMS AS may sends the notification based on the UE#2 is a MTSI UE, the DC application corresponding to the binding information supports interworking, and a P2A application DC is established for the DC application.

18-19. IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, which does not include bootstrap data channel information and application data channel information in the SDP for the application data channel.

20-29. Steps 17-26 of clause AC.7.2.2 applies.

The difference is that in step 25, the IMS AS may inform the UE#1 that the P2A application data channel is established as defined in TS 26.114 [76].

30.	UE#1 can use the established application data channel to provide DC application specific information to the DC AS, such as, sends the file (e.g. a photo) which will be sent to UE#2 to DC AS.

31.	DC AS performs interworking actions based on the notification received in step 17 and the data received in step 30.

NOTE 3:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 30 and sends the hyperlink to UE#2 through a SMS, the user of UE#2 can download the file using the hyperlink if he/she wants.

# * * * * End of changes * * * *