3GPP TSG SA WG2 Meeting #164	S2-2409099

# 19 - 23 August, 2024, Maastricht, Netherlands				    		(Revision of S2-2407949)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1411 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS DC architecture enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support Rel-19 features concluded in TR 23.700-77, the architecture of the IMS DC needs update to add new reference points: - Add reference point between IMS AS and NEF for event subscription and notification; - Add reference point between IMS AS and DC Application Server for event subscription and notification; - Add reference point between HSS and DC Application Server for event subscription; - Add reference point between HSS and NEF for event subscription. There is no new NF introduced in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add reference point between IMS AS and NEF for event subscription and notification; - Add reference point between IMS AS and DC Application Server for event subscription and notification; - Add reference point between HSS and DC Application Server for event subscription; - Add reference point between HSS and NEF for event subscription; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.2.1, AC.2.2.4, AC.2.2.6, AC.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex AC (normative):
Support of Data Channel services in IMS

# AC.1	General

This annex describes IMS architecture enhancements to support data channel services.

Data channels are always established in the context of an IMS MMTel session.

Service based interfaces applicable to data channel services are specified in clauses AA.2.4 and AA.2.5.

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided in two ways:

-	by enhancing the existing IMS MRF (called MRF) to perform media functions related to DC, which interacts with the IMS AS via enhanced Mr'/Cr interfaces; and

-	by introducing a new network function called Media Function (MF), which interacts with the IMS AS via the service-based interface DC2. MF provides media capabilities in support of IMS DC and Augmented Reality.

NOTE:	In this release of the specification the IMS AS supporting data channel services is collocated with the TAS.

In this Release of the specification IMS Data Channel is supported in the case of non-roaming and home routed roaming scenarios.

# AC.2	Architecture and functions

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

Figure AC.2.1-2 shows the data channel architecture when using the MRF.

Figure AC.2.1-2: Architecture option of IMS supporting DC usage with MRF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

NOTE 5: A trusted DC Application Server can use Nxx.

## AC.2.2	Functional entities

### AC.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function that provides data channel control logic. The DCSF is not involved in SIP signalling. The DCSF supports the following functionalities:

-	The DCSF receives event reports from the IMS AS and decides whether data channel service is allowed to be provided during the IMS session;

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the MF or MRF via the IMS AS;

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via MF and/or MRF to the UE based on UE subscription.

-	The DCSF downloads data channel applications from the Data Channel Application Repository;

NOTE 1:	The detailed procedures to download data channel applications to the UE are out of scope of this specification.

-	The DCSF interacts with NEF for data channel capability exposure via N33;

NOTE 2:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control via DC4/DC3, and for traffic forwarding via MDC3/MDC2.

NOTE 3:	Interaction with the DC Application Server for resource control is not specified in this release of the specification.

-	The DCSF interacts with HSS for retrieving and storing DCSF service specific data via N72/Sc.

### AC.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores the verified data channel applications which are retrieved by the DCSF when required.

### AC.2.2.3	Data Channel Media Function (MF) / MRF

The MF and MRF provide the media resource management and forwarding of data channel media traffic. The MF and MRF provide the following functionalities:

-	The MF and MRF manage the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS;

-	The MF and MRF terminate the bootstrap data channel from the UE and forward HTTP traffic between the UE and DCSF via MDC1;

-	The MF and MRF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs;

-	The MF and MRF relay traffic on the A2P/P2A application data channels between the UE and the DC Application Server via MDC2.

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS interacts with the NEF and DC application server for event subscription and notification;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS interacts with HSS to register its NF instance for event subscription;

### AC.2.2.5	S-CSCF

In addition to the functions defined in clause 4.6.3, the S-CSCF is enhanced to support the following functionalities:

-	The S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request from UE.

### AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

-	The HSS interacts with IMS AS to subscribe to event notifications on behalf of other NF, e.g. NEF or DC application server.

## AC.2.3	Reference points

The following reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and MF.

-	DC3:	Reference point between the DCSF and NEF.

-	DC4:	Reference point between the DCSF and DC Application Server.

-	DC5:	Reference point between the DCSF and DCAR.

-	DC6:	Reference point between the IMS AS and NEF.

-	DC7:	Reference point between the IMS AS and DC Application Server.

-	N72/Sc:	Reference point between the DCSF and HSS.

The following reference points are updated to support data channel signalling control in IMS:

-	N70/Cx/Dx:	Reference point between the CSCF and HSS.

-	N71/Sh:	Reference point between the IMS AS and HSS.

-	Nxx:	Reference point between the NEF and HSS.

-	N74:	Reference point between the DC Application Server and HSS.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between data channel media function (either MF or MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function (either MF or MRF) and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

The following reference point is updated to support data channel media handling:

-	Mr'/Cr:	SIP based reference point between IMS AS and MRF.

**** End of Changes ****