3GPP SA WG2 Meeting #164	S2-2409123

Maastricht, Netherlands, August 19 – 23, 2024           	(revision of S2-2408811)

Source: 	Nokia

Title:	Updated Conclusions for KI#5 to include TTNB

Document for: 	Approval

Agenda Item: 	19.3.1

Work Item / Release:    FS_XRM_Ph2 / Rel-19

Abstract of the contribution: Updated conclusions for KI#5 to include Time to Next Burst (TTNB) in DL traffic characteristics information changed dynamically.

# Discussion

3GPP SA2#163 sent LS S2-2407351 to SA4, RAN2, RAN3. Question #2 is related to Time to Next Burst (TTNB). Assuming positive feedback from SA4, RAN2 the current document updates the conclusions of KI#5 to include TTNB in DL traffic characteristics information changed dynamically. The principles are the following:

1.	TTNB is the time interval between the current burst and the next burst. TTNB is signalled in-band by the RTP sender. TTNB is set to 0, if the current PDU is not the last PDU of the burst or if the sender cannot determine the TTNB. TTNB applies only for the immediate next burst to come. Further details are provided in S4-241018.

2.	Periodicity is transmitted via the control plane and applies up to the point that it is updated, i.e. it applies for more than one bursts.

3.	TTNB is provided on top of the periodicity. When TTNB is available this is the valid time interval to the immediate next burst and not the one described by the periodicity.

4.	Periodicity does not capture the flexible transmission nature of video streams, nor rate adaptation mechanisms. It is bound to isochronous media capture and transmission.

5.	Periodicity is an estimated theoretical value valid for many bursts, whereas TTNB is a calculated precise value for the immediate next burst.

6.	TTNB can be used to initiate the most adequate sleep state.

# Proposal

It is proposed to capture the following changes in XRM Ph2 TR 23.700-70 v1.0.0.

# * * * * First change * * * *

## 8.5	Conclusions for Key Issue #5

The following aspects are concluded as principles for the normative work:

-	Support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling.

1.	The AF may indicate to the NEF/PCF to mark the Burst size.

2.	Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size.

3.	Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header.

NOTE 1:	This conclusion assumes that SA4 will specify Burst size indication in the header extension (i.e. RTP HE).

4.	The NG-RAN may use the received Burst size to assist radio resource management.

5.	The AF may include information in the DL Protocol Description provided to the NEF/PCF to determine the Time to Next Burst.

6.	Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Time to Next Burst and to further notify NG-RAN on the Time to Next Burst.

7.	Based on SMF instruction, the UPF identifies and marks the Time to Next Burst, based on the input carried in N6 DL packet header. The UPF sends the Time to Next Burst to NG-RAN via DL GTP-U header.

NOTE 2:	This conclusion assumes that SA4 will specify Time to Next Burst in the header extension (i.e. RTP HE).

NOTE 3:	Feedback from RAN2, RAN3, SA4 on LS S2-2407351 on Time to Next Burst will be considered as part of normative work.

# * * * * End of changes * * * *