3GPP TSG-WG2 Meeting #164	S2-2409132

19th – 23rd August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0532 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 - LMF enhancements for UE positioning using a ML model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes to specify the conclusions in the TR 23.700-84 for KI#1 for the option when LMF performs both training of the ML Model and inference, as such principle #2 and #5 are the main ones listed below for reference, although other principles are also applicable to this option. Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning, the trigger for data collection and for model training in LMF is up to implementation. Principle #5: Data used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs. The related procedures for data collection will be coordinated with RAN WGs in the normative phase. NOTE 3: The user's authorization/consent of collecting UE related training data is needed as specified in TS 23.288 [5] and TS 23.273 [7]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The LMF collects input data for ML Model training from the UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion in the TR 23.700-84 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

### 6.X.1	Data collection at LMF to train the AI/ML based positioning model to perform positioning based on UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Editor’s note: 	The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Figure 6.x.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

- The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = ”any UE”, Event ID = “UEs in/out area of interest”.

- The AMF send Namf_EventExposure_Subscriber_Response (list of SUPIs in the area of interest”).

Editor’s Note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

- The LMF checks the user consent for data collection for model training.

Editor´s Note:	Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS.

Editor´s Note:	Further details on user consent for data collection for model training will be aligned with SA3.

- For each SUPI that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor’s note: 	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor’s note: 	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data to train the AI/ML based positioning model.

********** End of changes **********