3GPP TSG-SA2 Meeting #164	S2-2409139

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024            (revised from S2-2408889)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1104 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID to support Signalling storm Mitigation and Prevention |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SK Telecom, AT&T, China Mobile, ETRI, KDDI, KPN N.V., LG Uplus, OPPO, Rakuten Mobile, Samsung, vivo, ZTE, Huawei, HiSilicon, Nokia, NTT DOCOMO, China Telecom, Lenovo, Ericsson, Verizon? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-84 v1.0.0 has documented the conclusion for key issue#4 “NWDAF enhancements to support network abnormal behaviours (i.e. signalling storm) mitigation and prevention”. To solve the issue, the following conclusions are reached: “- By means of statistics and predictions, NWDAF supports assistance to signalling storm mitigation and prevention by statistically learning the expected normal level of signalling and identifying a significant deviation indicating a signalling storm caused by massive signalling of UEs and/or caused by NFs signalling. - A new Analytics ID to support Signalling storm Mitigation and Prevention caused by NFs signalling will be defined. - It will be determined in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs. - Analytics Filter includes information to assist the NWDAF for the signalling storm as defined above. NOTE 1: The consumers of the analytics ID and the parameters of the analytics request/subscribe (determined by the consumer) are to be specified in normative work.” This contribution provides the analytics detail of the prevent network abnormal behaviour use case based on the new NF’s information. |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Analytics ID for the prevention of abnormal NF behaviour causing signalling storm and mitigation of its impact in the network use case in the TS 23.288. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for the “Analytics-assisted prevention of abnormal NF behaviour”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new )* * * *

6.x	Signalling Storm Analytics

6.x.1	General

This clause specifies how NWDAF support network abnormal behaviours (i.e. signalling storm) mitigation and prevention.

The signalling storm analytics provides analytics information (statistics or predictions) regarding the expected normal level of signalling and significant deviations indicating a signalling storm. The analytics of signalling storm requires the identification of whether the signalling storm is due to signalling from a NF or massive signalling from UE based on the requested analytics ID.

The NWDAF provides statistics and/or prediction analytics to consumer NFs, e.g. AMF, SMF, UDM, PCF, NRF, MDAF/MDAS and AF.

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "Signalling storm analytics";

-	Target of Analytics Reporting: a list of NFs, or one or several UE group IDs (only if reporting of signalling storm caused by UEs is desired);

- 	Target Incident ID(s) (e.g. Signalling Storm Caused by UEs, Sigalling storm caused by NF, or all)

-	Analytics Filter Information:

-	NF ID List: indicates instance IDs or NF Set IDs of target NF that signalling storm analytics should be executed for.

-	Area of Interest (AOI) and/or S-NSSAI: restricts the scope of signalling storm analytics to the specific area and/or slice;

-	Expected Report Time : indicates the time limit by which the analytics results need to be received., e.g. 10 seconds, 5 minutes, etc;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

- 	Time slot: indicates the time slot during which the analytics is derived, e.g. 10 seconds, 5 minutes, etc;

-	Optionally, preferred level of accuracy of the analytics;

-	Analytics Reporting Information:

-	Reporting threshold and filter:

-	Signalling frequency threshold: indicates the frequency of the received signalling within the time period;

-	UE number threshold: indicates a threshold of the amount of UEs of which the number of one type of request messages meets the Frequency of UE requests threshold. For example, if the number of UEs that initiate registration requests exceeds the pre-set threshold (10 times per hour) and the number of such UEs exceeds 100 (the threshold is set to 100), the NWDAF will provide the analytics.

Editor’s Note :	Additional Analytic Filter Information parameters are FFS.

6.x.2	Input data

The NWDAF collects the signalling feature data, NF context, AF data and MDAF data from the sources listed in Table 6.x.2-1, 6.x.2-2, 6.x.2-3, 6.x.2-4.

Editor’s Note :	The already supported counters/KPIs by OAM defined per NF in TS 28.552 [11] are collected from OAM; other remaining counters/KPIs are collected from the corresponding NFs.

The NWDAF also collects UE behavioural information per group as specified in clause 6.7.5.2.

The NWDAF may use output of abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and 6.10.3 as input data of NF signalling storm analytics. This information may be collected from a different NWDAF (e.g. distributed architecture).

The NWDAF may collect the subgroup of input data based on implementation.

Editor’s Note:	It needs to be confirmed wether all input data and sources are sufficient or required. Input data should preferably reuse data available via existing exposure mechanisms. New Input data require related extensions to event exposure mechanisms, and currently unavailable parameters via event exposure needs to be defined.

In order to reduce signalling overhead, offline bulk data report as specified in clause 6.2.6.1 may be used.

Table 6.x.2-1: UE related Context Data collection by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| NF ID | AMF, SMF, NRF | NF instance ID of the service producer or consumer. |
| UE type ID | AMF, SMF, NRF | Identifies a group of UEs, e.g. internal group ID, slice ID, or a list of UE IDs. |
| Signalling feature data | AMF, SMF, NWDAF | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures. NOTE 1 |
| > Request type and number from UE/RAN | AMF | Type and number of requests for N1 or N2 interface and number of request, such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc. |
| > Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| > Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests, such as Registration Response, etc. |
| > Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests, such as Registration Reject, Service Reject, etc. |
| > Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| > A posterior Type of requests of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN. |
| > > A posterior Type of responses of UE/RAN (0..max) | AMF | A posterior Response types triggered from UE/RAN, for NF Service response, or response to UE/RAN. |
| > Type of requests from NF | AMF | Request type received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. |
| > Number of requests from NF | AMF | Number of requests received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. |
| > Number of total failure responses of NF | AMF | Number of total failure responses associated to their initial requests. |
| >> Time duration from receiving request from NF to response to NF | AMF | Time duration between the request from NF and response to NF. |
| >> Number of successful responses of NF | AMF | Number of successful responses associated to their initial requests. |
| > Number of failed responses of NF | AMF | Number of failed responses associated to their initial requests. |
| > Reason of failed responses of NF | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| > Number of redundant signalling of NF | AMF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted in multiple times. |
| > Number of unhandled signalling of NF | AMF | Number of received unhandled signalling. The unhandled signalling means the signalling, which is not processed, e.g. because the value of Ies in the signaling is abnormal. |
| > A posterior Type of requests of NF (0..max) | AMF | A posterior Request types triggered from NF, for NF Service request. |
| > > A posterior Type of responses of NF (0..max) | AMF | A posterior Response types triggered from NF, for NF Service response. |
| > Public Warning information | AMF | Public Warning information as defined in the TS 23.041[X], such as WRITE-REPLACE WARNING REQUEST, etc. |
| Frequent Mobility Registration Update | AMF | The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator’s configurable parameters. |
| > Request type and number of requests from NF | SMF | Type and number of request received from NF, such as PDU Session Establishment, Modification, Release Request, etc. |
| > Number of requests from NF | SMF | Number of requests received such as PDU Session Establishment, Modification, Release Request, etc. |
| > Time duration from receiving request from NF to response to NF | SMF | Time duration between the request from NF and response to NF. |
| > Number of successful responses of NF | SMF | Number of successful responses associated to their initial requests, such as PDU Session Establishment, Modification, Release Response, etc. |
| > Number of total failure responses of NF | SMF | Number of total failure responses associated to their initial requests. |
| >> Number of failed responses of NF | SMF | Number of failed responses associated to their initial requests, such as PDU Session Establishment, Modification, Release Reject, etc. |
| >> Reason of failed responses of NF | SMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| > Number of redundant signalling of NF | SMF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted in multiple times. |
| > Number of unhandled signalling of NF | SMF | Number of received unhandled signalling. The unhandled signalling means the signalling which is not processed, e.g. because the value of Ies in the signaling is abnormal. |
| > A posterior Type of requests of NF (0..max) | SMF | A posterior Request types triggered from NF, for NF Service request. |
| > A posterior Type of responses of NF (0..max) | SMF | A posterior Response types triggered from NF, for NF Service response. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| NF Context information |  | NF context information related to a particular UE/Session/NF context. |
| UE Context | AMF, SMF | The UE MM or SM context in the NF. |
| >State transition information | AMF, SMF | UE related state transition information such as transition type, frequency of CM state changes etc. State transition identifier: - “Access Type change to 3GPP access”; - “Access Type change to non-3GPP access”; - “RM state change to RM-DEREGISTERED”; - “RM state change to RM-REGISTERED “; - “CM state change to CM-IDLE”; - “CM state change to CM-CONNECTED”; - “Handover”; or - “Mobility Registration Update”. - “Frequent Mobility Registration Update”(table 6.7.2.2-1) Or, PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - “PDU Session Establishment”; - “PDU Session Release”; - “Communication failure”; or - “PLMN change”. It can be reported for a group of UEs(e.g., the number of total transitions or percentage of the group UEs who have transitions). |
| > timer information | AMF | Timer information which has been set for the UE(s), such as timer type, timestamp for starts, duration, etc. |
| NOTE 1: NWDAF can optionally provide transaction dispersion analytic information for MM and SM transactions. |  |  |

Table 6.x.2-2: NF Context Data collection by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| NF ID | NF | NF instance ID of the service producer or consumer. |
| > Usage information of UE IP address resources | SMF, OAM | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. |
| > Load information of connected UPFs | SMF, OAM | Load information of connected UPFs such as using PFCP Load Control Information. |
| > Type of requests from NRF | NRF, OAM | Request type received such as NF registration and discovery, etc. |
| > Number of requests from NF | NRF, NF, OAM | Number of NF requests received such as NF registration and discovery, etc. |
| > Time duration from receiving request from NRF to response to NF | NRF, NF, OAM | Time duration between the request from NRF and response to NF. |
| > NF profile | NRF, OAM | NF Profile information such as allowed NF information per NF and NF Service. |
| > NF load and status information of registered NFs and NF Services | NRF, SCP, OAM | Load information indicates the current load of NFs and NF Services, e.g. CPU, memory, and/or percentage of load information. |
| > Capacity and priority information of NFs and Services | NRF, SCP, OAM | Capacity and priority information of registered NFs and NF Services. |
| > NF heart-beat related information | NRF, OAM | NF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals, etc. |
| > Success and failure trend | NRF, OAM | Number Requests/response related trend such as within a certain time interval, ratio, cause/error code, etc. |
| SCP Signalling statistics | SCP | The number of different types of signalling received and sent by SCP during a target time period, etc. |

Table 6.x.2-3: Application activation time information collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Number of UEs | AF | The total number of UEs |
| > Active Time | AF | The time stamp of the users per application switch to active, or the start and end time of the users activity per application. |
| > Inactive Time | AF | The time stamp of the users per application switch to inactive, or the start and end time of the users inactivity per application, if applicable. |
| > UE type ID | AF | Identifies a group of UEs, e.g. external group ID, or a list of UE IDs. |

Table 6.x.2-4: Data collection from MDAS/MDAF of control plane congestion analysis

| Information | Source | Description |
| --- | --- | --- |
| affectedObject | MDAF | Indication of 5GC NFs where congestion issues occurred or potentially may occur. |
| cPCongestionIssueID | MDAF | This field holds the ID of the control plane congestion issue which is reported. |

6.x.3	Output analytics

The output analytics of signalling storm provided by NWDAF is defined in Table 6.x.3-1, and Table 6.x.3-2. The Table 6.x.3-3 gives examples of NF actions for solving each risk. The final mitigation or prevention operations are based on operator's policy/configuration and NF implementation.

Table 6.x.3-1: Signalling storm statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm statistics. |
| > Target NF ID | A list of target NFs signalling storm detected by NWDAF. |
| > Cause of the signalling storm | The potential cause of NF Abnormality (e.g. massive signalling from UE, NF abnormal signalling, etc.). |
| > Source UE/NF | group of the UE(s) identified as slice ID or internal group ID or NF(s) identified as NF list which cause the signalling storm. |
| >Signalling information | The statistics of signalling information. |
| >> Received Signalling Analytics | Information of signalling received by the NF. |
| >>> Total number of received signalling | Indicates the statistics on the number of signalling messages received by the NF in the time slot. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the NF in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. |
| >>>> number of signalling requests from the same UE (1…max) | Number of received signalling requests initiated by the same UE within the time slot. |
| >>>>>Number of UEs | Number of UEs whose signalling requests sent by the same UE within the time slot. |
| > Timer List | The list of timer information |
| >> Type of timer | The type of timer which has been set. |
| >> Timer range | The timer range that can be selected by the NF. |

Table 6.x.3-2: Signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of predicted signalling storm analytics. |
| > Target NF ID | A list of target NFs signalling storm predicted by NWDAF. |
| > Cause of the signalling storm | The potential cause of NF Abnormality (e.g. massive signalling from UE, NF abnormal signalling, etc.). |
| > Source UE/NF | group of the UE(s) identified as slice ID or internal group ID or NF(s) identified as NF list which cause the signalling storm. |
| >Signalling information | The predicted of signalling information. |
| >> Reference Point | The information of the reference point impacted. |
| >> Service Operation(s) | The information of the service operation(s) impacted. |
| >> Received Signalling Analytics | Information of signalling received by the NF. |
| >>> Received number of signalling | Received number of signalling of the specific signalling type. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the NF in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. |
| >>>> number of signalling requests from the same UE (1…max) | Number of received signalling requests initiated by the same UE within the time slot. |
| >>>>>Number of UEs | Number of UEs whose signalling requests sent by the same UE within the time slot. |
| > Timer List | The list of timer information |
| >> Type of timer | The type of timer which has been set. |
| >> Timer range | The timer range that can be selected by the NF. |
| > Priority | Priority (relative to other NFs of the same type) of candidate NFs to be used for NF selection as defined in the TS 29.510 [18]. |
| > Capacity | Candidate NF capacity information, expressed as a weight relative to other NF instances of the same type as defined in the TS 29.510 [18]. |
| > Confidence | Confidence of this prediction. |

NOTE 1:	The statistics and predictions are provided with a Validity Period, as defined in clause 6.1.3.

Table 6.x.3-3: Example mechanisms to mitigate and prevent the signalling storm

| Cause of the signalling storm | Actions of NFs |
| --- | --- |
| massive signalling from UE | AMF sets MM NAS related timer (e.g. back-off, T3512) with suggested time range for a selected set of UEs. |
| massive signalling from UE | SMF sets SM NAS related timer (e.g. back-off) with suggested time range for a selected set of Sessions. |
| massive signalling from UE | AMF/SMF sets suggested N1/N2 interface related ingress/egress threshold, or AMF triggers RAN to initiate overload control for a selected set of UEs in specific slice or priority as defined in clause 8.7.7 of TS 38.413 [8] to start overload control. The AMF may assign a abnormal slice to the UEs and initiate overload control for the specific slice. |
| NF abnormal signalling | NRF configures the local policy to prevent the source NF with abnormal signalling from being discovered or discovering others. |
| NF abnormal signalling | Source NF configures to (re)select other NFs instead of NF with abnormal signalling and may unsubscribes the NF with abnormal signalling. |
| NF abnormal signalling | Source NF configures to deprioritize the NFs/Services with abnormal signalling from being selected. |
| NF abnormal signalling | Source NF triggers UE Reregistration and/or Session Reestablishments to avoid NF with abnormal signalling. |

6.x.4	Procedures

The NWDAF can provide information on network signalling storm as follows. (Figure 6.x.4-1).

Figure 6.x.4-1: Procedure for NWDAF-assisted Network Signalling Storm Mitigation and Prevention

1.	The consumer NF or (e.g. MDAF/MDAS subscribes to or sends a request to NWDAF assistance information for the signalling storm analytics using either Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation. The request additionally includes thresholds such as confidence level and accuracy of detection. The request may also include as analytic filters an indication of the use case for signalling storm (e.g. signalling storm due to UE mobility).

The Analytics ID is set to "signalling storm analytics". The target for analytics reporting is set to be any NF or any UE. Analytic filters may be provided as shown in clause 6.x.1.

The Consumer NF can request statistics or predictions or mitigation or prevention or all for a given Analytics target period.

The consumer NF may also subscribe to “NF load analytics” as depicted in clause 6.5, and trigger the signalling storm analytics based on output of the NF load analytics, e.g. CPU usage is over 80%.

The consumer NF may also subscribe to “Abnormal bheaviour” analytics as depicted in clause 6.7.5 and trigger the signaling storm analytics based on the suspicion DDoS attack.

The consumer NF may also subscribe to “Dispersion Analytics” analytics as depicted in clause 6.10 and trigger the signaling storm analytics when the transaction dispersion is above the expected transaction dispersion configurable threshold.

2.	The NWDAF retrieves Input data from NFs, using Nnf_EventExposure_Subscribe, such as UE and/or NF releated context data, NF context, AF data, MDAF data and UE behavioural information to derive the required analytics as depicted in clause 6.x.2. The NWDAF may optionally request additional data aligned with the analysis target identified in step 1.

3.	The NWDAF derives the required analytics based on the consumer NF request. The NWDAF can collect additional Input data from different NFs, e.g. including from MDAF/MDAS/NWDAF. The NWDAF collects input data from NFs taking into account the use case of signalling storm if provided in step 1.

4.	The NWDAF invokes Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response or response to the consumer NF for the Output data analytics as depicted in clause 6.x.3.

5.	The consumer upon receiving the detection and/or prediction and/or mitigation may execute based on the example mechanisms as depicted in clause 6.x.3.

# * * * * Second change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
|  | Request | Request / Response | H-NWDAF, V-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Editor´s note:	It is FFS whether separate Roaming Analytics and Roaming data services for inbound and outbound roaming scenarios are required.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| E2E data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behavior | Analytics ID: Movement Behavior | Statistics or predictions on movement behavior for an applicable area |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions on controlling signalling storm for network abnormal behaviour mitigation or prevention. |

# * * * * End of changes * * * *