3GPP SA WG2 Meeting #164	 S2-2409176

# Maastricht, NL, 19-23 August 2024															(was S2-2409092)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1425 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting of network initiated IMS Data Channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#2 on " Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session " are agreed: 1. IMS network exposes its capabilities through the NEF, which can be provided by a specific NEF instance serving the IMS network exposure only or by the NEF instance serving the 5GC network exposure simultaneously. 2. The following capabilities need to be exposed by the IMS network: - establishing, updating, terminating of bootstrap data channels; - establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels. 3. The Nnef and Nimsas services and/or service operations need to be enhanced to support the capabilities mentioned above. NOTE: The detailed impacts to the services and/or service operations are defined in normative phase. 4. In Rel-19, the only service provider is IMS AS. The service consumer needs to discover the IMS AS for a given user via HSS before sending a request to IMS AS. If the user is not registered yet, the request is rejected. This CR implements the approved conclusions to support network initiated IMS data channel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New services and operations in IMS AS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network initiated IMS DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.1, AA.2.4.X(new), AC.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_ImsSessionManagement | Create | Request/Response | NEF, Trusted AF |
|  | Update | Request/Response | NEF, Trusted AF |
|  | Delete | Request/Response | NEF, Trusted AF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

# >>>> NEXT CHANGES (ALL NEW)<<<<

### AA.2.4.X	Nimsas_ ImsSessionManagement Service

#### AA.2.4.X.1	General

Service description: This service enables the consumer to request IMS AS to create and release an IMS session and modify the media in a specific IMS session.

NOTE: In this release only session management of IMS data channel is specified.

Editor’s Note: The parameter design of this service is FFS.

#### AA.2.4.X.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with standalone data channel media.

Inputs, Required: Serving Number, Destination Number, Media Information set.

Editor’s Note: How the parameter design aligns with OMA API is FFS.

Serving Number: it is the subscriber served by the DC AS.

Destination Number: it is the number that this request targeted to.

Media Inforamaiton set includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Type: it indicates the media type used in the created session, e.g., Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: None.

Outputs, Required: Result indication and Session ID when the operation is successful.

Outputs, Optional: None.

#### AA.2.4.X.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as creating or releasing bootstrap data channel(s) or application data channel(s).

Inputs, Required: Serving Number, Destination Number, Media Information set.

Editor’s Note: How the parameter design aligns with OMA API is FFS.

Serving Number: it is the subscriber served by the DC AS.

Destination Number: it is the number that this request targeted to.

Media operation set includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Operation Type: it indicates adding, updating or removing the corresponding media.

-	Media Type: it indicates the operated media is Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Session ID.

Outputs, Required: Result indication and Session ID when the operation is successful.

Outputs, Optional: None.

#### AA.2.4.X.4	Nimsas_ImsSessionManagement_Delete service operation

Service operation name: Nimsas_ImsSessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

# >>>> NEXT CHANGES (ALL NEW)<<<<

# >>>> NEXT CHANGES<<<<

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS receives data channel control request from NEF/DC AS for establishment, update or release of bootstrap data channel and/or application data channel.

# >>>>END OF CHANGES<<<<