3GPP TSG-SA2 Meeting #164	S2-2409180

# Maastricht, NL,  19th - 23rd Auguest, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1439 | rev | 03 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to support the conclusion of KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.2 in 3GPP TS 23.700-77 concludes KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of IMS data channel capabilies exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## Annex XX (normative): IMS Capability Exposure Framework in IMS

### XX.1		IMS data channel capability exposure procedures

#### XX.1.1	General Description

##### There are two scenarios in IMS data channel capability exposure scenarios:

-	Adding data channel to an existing IMS session, for which Nimsas_SessionManagement_Update is used

-	Creating a standalone data channel session, for which Nimsas_SessionManagement_Create is used.

Editor’s Note: The details of the procedures are FFS.

### XX.1.2	Adding Data Channel to an Existing IMS Session

Figure XX.X.2-1 shows the procedure of adding bootstrap data channel(s) or application data channel(s) to an existing IMS session.

Figure XX.1.2-1: Procedure of adding data channel(s) to an existing IMS session

0.	DC AS determines to add data channel to an existing IMS session based on local policy or event notification of the related IMS Session.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add bootstrap data channel(s) or application data channel(s) in the exisiting IMS session.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to add bootstrap data channel(s) or application data channel(s) in the specific session.

4.	The IMS AS-2 validates the user subscription data and determine whether the data channel request should be notified to DCSF-2. If the serving UE does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause.

The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding data channel(s). Then the DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The existing procedures depicted in Annex AC.7 is reused.

5.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the data channel required by the DC AS with the other existing media descriptions.

The IMS AS-2 sends the re-INVITE request according to the existing IMS DC session update procedure.

6.	The DC media negotiation is completed and corresponding data channel are established.

7.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

8.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

### XX.1.3	Establishing Standalone Data Channel Session

Figure XX.X.3-1 shows the establishment procedure of a standalone data channel session.

Figure XX.1.3-1: Establishment procedure of a standalone data channel session

0.	The DC AS decides to establish a standalone data channel session.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF requiring to create a standalnone DC session with UE A.

2.	The NEF queries the HSS for the address of the IMS AS serving the service number of the enterprise if needed. If the service number is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response.

3.	The NEF sends a Nimsas_SessionManagement_Create request to the IMS AS-2 requiring to create a standalone DC session.

4.	The IMS AS-2 validates the user subscription data and determine whether the data channel request should be notified to DCSF-2. If the serving UE does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause.

The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of creating a data channel. Then the DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The existing procedures in Annex AC.7 is reused.

5.	The IMS AS-2 generates an INVITE request in which the SDP offer contains the media information of the data channel required by the DC AS.

The IMS AS-2 sends the INVITE request according to the existing IMS DC session establishment procedure.

6.	The DC media negotiation is completed and corresponding data channel are established.

7.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Create response to the NEF.

8.	The NEF returns a successful Nnef_ImsSessionManagement_Create response to the DC AS.

### XX.1.4	Removing Data Channel from an Existing IMS session

Figure XX.X.4-1 shows the termination procedure of bootstrap data channel(s) or application data channel(s) in an existing IMS session.

Figure XX.1.4-1: Termination procedure of bootstrap data channel

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to terminate the data channel with UE A in the specific session identified by Session ID.

2.	The NEF queries the HSS for the address of the IMS AS serving the specific UE if needed.

3.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to terminate the data channel in the specific session identified by Session ID.

4.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of removing the data channel. Then the DCSF-2 instructs the IMS AS-2 to terminate the data channel by sending a Nimsas_MediaControl request.

5.	The IMS AS-2 generates a re-INVITE request in which the port of "m=application" line for the data channel to be removed is set to zero.

The IMS AS-2 sends the re-INVITE request according to existing IMS procedure to terminate the data channel.

6.	The media negotiation is completed and corresponding data channel are removed.

7.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

8.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

# * * * * End of changes * * * *