3GPP TSG-SA WG2#164	S2-2409194

19-23 August 2024, Maastricht, Netherlands	(was S2-2408131, S2-2406824)

Source:	Apple, ETRI

Title:	Conclusion for KI#1.2

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.2.

# 1.	Introduction

This contribution proposes conclusion for KI#1.2 based on NWM discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.2

The following principles are concluded for the normative work:

-	The UE(s) of the DualSteer Device include(s) information in the Registration request whether it is (they are) DualSteer capable.

-	The UEs of the DualSteer Device include information in the Registration request whether they support simultaneous transmission.

-	In roaming scenario, DualSteer can be supported if the serving PLMN of each registered access supports the DualSteer feature.

-	A DualSteer Device performs a separate registration procedure for each of the SUPIs of the DualSteer Device. The DualSteer Device can choose either of the SUPIs to be the first one to register with the network.

- 	If the DualSteer Device sends its DualSteer capability as part of Registration Request, the serving network sends that information to the HPLMN. The HPLMN performs DualSteer authorization based on the subscription data associated with the SUPI, the capability of DualSteer Device and the serving network, and sends the authorisation result to the AMF. The AMF provides the DualSteer capability to UE PCF. The AMF indicates to the DualSteer Device whether the network authorizes the DualSteer feature during registration. If DualSteer is not enabled on one of the registered 3GPP access, the DualSteer feature cannot be enabled. Whether the DualSteer Device can stay registered on more than one 3GPP accesses when DualSteer is not enabled is controlled by operator policy and enforced by the network.

NOTE: Whether there are any impacts on the second SUPI’s PLMN selection are pending on SA1 and CT1.

# >>>>END OF CHANGES<<<<