3GPP TSG-SA WG2#164	S2-2409195

19-23 August 2024, Maastricht, Netherlands	(was S2-2408133, S2-2407087)

Source:	Apple, ETRI

Title:	Conclusion for KI#1.3

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.3.

# 1.	Introduction

This contribution proposes conclusion for KI#1.3 based on NMW discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.3

The following principles are concluded for the normative work:

-	The DualSteer PDU session subject for traffic switching is anchored in a common UPF (PGW-U) and managed by a common SMF (PGW-C).

-	The SMF manages the DualSteer PDU Session using a single PCF association that provides unified policies and single PFCP association/N4 rules towards UPF.

-	The DualSteer PDU Session is applicable to specific DNNs/S-NSSAIs. The applicable DNNs/S-NSSAIs are controlled via URSP and SM subscription data.

-	The SMF manages the DualSteer PDU Session using a single PCF association that provides unified policies and single PFCP association/N4 rules towards UPF.

-	Steering of services for LBO sessions is not in the scope for DualSteer.

# >>>>END OF CHANGES<<<<