3GPP TSG-SA WG2#164	S2-24009196

19-23 August 2024, Maastricht, Netherlands	(was S2-2408134, S2-2406827)

Source:	Apple, ETRI

Title:	Conclusion for KI#1.4

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.4.

# 1.	Introduction

This contribution proposes conclusion for KI#1.4 based on NMW discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.4

The following principles are concluded for the normative work:

-

-	The DualSteer steering rules are provided by the PCF to the UE(s) at registration as part of UE policies, as necessary.

-	The DualSteer switching rules are provided during the DualSteer PDU Session establishment and modification as part of SM NAS signaling and defined to control the DualSteer traffic switching in the DualSteer Device.

-	For a DualSteer Device the DualSteer steering and DualSteer switching rules identify the UE/SUPI for steering and switching decision by using an access descriptor in the rules.

The PCF can take into account the UE capability and the operator policy for simultaneous transmission for creating the DualSteer steering and DualSteer switching rules.

-	The DualSteer steering and DualSteer switching rules are defined with service-level granularity.

-	The PCF can take into account UE's PLMN, RAT, or Access Type to generate the DualSteer steering rules and PCC rules that is used to generate the DualSteer switching rules.

-	The SM PCF, based on PCRT, can update the PCC rules and that will trigger the SMF to update the DualSteer switching rules.

-	Optionally, when the registration for the first SUPI is accepted, the PCF may provide a registration-related policy to the DualSteer Device as part of UE policies when to register the second SUPI, in which case this policy is enforced by the network.

NOTE: Whether there are any impacts on the second SUPI’s PLMN selection are pending on SA1 and CT1.

# >>>>END OF CHANGES<<<<