3GPP TSG-WG SA2 Meeting #164	S2-2409213

# Maastricht, Netherlands, 19 August – 23 August, 2024				          was S2-2407630

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4856 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align H-PCF selection with H-SMF selection in 23.502 R19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is a 23.502 corresponding paper to a 23.501 paper (S2-2407629). 1. Currently during PDU session establishment procedure for HR roaming scenario, the AMF discovers both vSMF and hSMF. It provides both hSMF ID and hSMF URI (e.g FQDN/IP-address) to vSMF. This obviates the need for vSMF to re-discover hSMF. 2. Similarly to (1), Currently during UE registration procedure for a roaming UE, the AMF discovers both vPCF and hPCF. However, based on 23.501 6.3.7, it provides only hPCF ID to vPCF (it does not provide hPCF URI). This requires vPCF to discover the FQDN/IP-addr/port of the hPCF by querying the NRF. In addition, it creates a misalignment, between the above two discoveries (i.e. H-SMF discovery vs H-PCF discovery), which was not the original intention. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to update Npcf_UEPolicyControl service such that H-PCF address can be provided to V-PCF along H-PCF ID (same as it is done for the H-SMF). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between H-SMF discovery and H-PCF discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 5.2.5.6.2	Npcf_UEPolicyControl_Create service operation

Service operation name: Npcf_UEPolicyControl_Create

Description: NF Service Consumer can request the creation of a UE Policy Association by providing relevant parameters about the UE context to the PCF.

Inputs, Required: Notification endpoint, SUPI.

Inputs, Optional: H-PCF ID/address (if the NF service producer is V-PCF and AMF is NF service consumer), information provided by the AMF as define in clause 6.2.1.2 of TS 23.503 [20], such as Access Type, Permanent Equipment Identifier, GPSI, User Location Information, UE Time Zone, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), RAT type, LBO Information (see clause 6.1.2.2.4 of TS 23.503 [20]), UE policy container including the list of PSIs, OS id, the indication of UE support for ANDSP, UE capability of reporting URSP rule enforcement to network (see clause 6.6.2.4 of TS 23.503 [20]), UE indication of support of URSP delivery in EPS and Internal Group (see TS 23.501 [2]), Satellite backhaul category (see clause 5.43 of TS 23.501 [2]), "5GS to EPS Mobility" indication, request to update the UE policies, request to be notified when updated UE policies have been provided to the UE, the indication of UE support for N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access, the indication of UE support for TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access.

Outputs, Required: Success or Failure, UE Policy Association ID.

Outputs, Optional: Policy Control Request Trigger of UE Policy Association. In the case of H-PCF is producer, UE policy information (see clause 5.2.5.6.1).

# *** End of Change ***