3GPP SA WG2 Meeting #164	S2-2409240

# Maastricht, NL, 19-23 August 2024	               					 (was S2-2408893 was S2-2408513)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5596 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support of Mobile Wireless Access Backhaul in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, LG Electronics, IIT Bombay, OPPO, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This contribution added the general description of the MWAB feature and corresponding architecture enhancements in TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add general description of the system architecture for MWAB support in new clause 5.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of VMR_Ph2 feature in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; (new) 5.x, (new) Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	Void.

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	Void.

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	3GPP TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[72]	Void.

[73]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[74]	IETF RFC 3162: "RADIUS and IPv6".

[75]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[76]	3GPP TS 26.238: "Uplink streaming".

[77]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156"; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[80]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[81]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[82]	IETF RFC 8803: "0-RTT TCP Convert Protocol".

[83]	IEEE Std 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[84]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[86]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[87]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[88]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[89]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[90]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[91]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[92]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[93]	BBF TR-456 issue 2: "AGF Functional Requirements".

[94]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The reference to BBF WT-457 will be revised when finalized by BBF.

[95]	Void.

[96]	Void.

[97]	IEEE Std 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[98]	IEEE Std 802.1Q-2022: "IEEE Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[99]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[100]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[101]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[102]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; stage 2".

[103]	3GPP TS 29.563: "5G System (5GS); HSS services for interworking with UDM; Stage 3".

[104]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[105]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[106]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[107]	IEEE Std 1588-2008: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[108]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[109]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[110]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[111]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[112]	3GPP TR 38.824: "Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)".

[113]	IEEE: "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

[115]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[116]	3GPP TS 38.415: "PDU Session User Plane Protocol".

[117]	3GPP TS 24.535: "Device-side Time-Sensitive Networking (TSN) Translator (DS-TT) to network-side TSN Translator (NW-TT) protocol aspects; Stage 3".

[118]	3GPP TS 32.274: "Charging Management; Short Message Service (SMS) charging".

[119]	3GPP TS 23.008: "Organization of subscriber data".

[120]	3GPP TS 38.314: "NR; Layer 2 measurements".

[121]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[122]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[123]	3GPP TS 32.254: "Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[124]	3GPP TS 33.535: "Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (5GS)".

[125]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[126]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[127]	ST 2059-2:2015: "SMPTE Standard - SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications".

[128]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[129]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[130]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[131]	IEEE Std 802.3: "Ethernet".

[132]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[133]	3GPP TS 29.513: "Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[134]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[135]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[136]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[137]	GSMA NG.116: "Generic Network Slice Template".

[138]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[139]	3GPP TS 24.539: "5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3".

[140]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[141]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[142]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

[143]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[144]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[145]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

[146]	IEEE Std P802.1Qdj-d1.3: "IEEE Draft Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment XX: Configuration Enhancements for Time-Sensitive Networking".

[147]	Void.

[148]	3GPP TS 28.557: "Management and orchestration; Management of Non-Public Networks (NPN)".

[149]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM)".

[150]	IETF RFC 8655: "Deterministic Networking Architecture".

[151]	IETF RFC 8343: "A YANG Data Model for Interface Management".

[152]	IETF RFC 8344: "A YANG Data Model for IP Management".

[153]	IETF RFC 7224: " IANA Interface Type YANG Module".

[154]	IETF draft-ietf-detnet-yang: "Deterministic Networking (DetNet) YANG Model".

Editor's note:	The reference to draft-ietf-detnet-yang will be revised to RFC when finalized by IETF.

[155]	IETF RFC 6241: "Network Configuration Protocol (NETCONF)".

[156]	IETF RFC 8040: "RESTCONF Protocol".

[157]	IETF RFC 8939: "Deterministic Networking (DetNet) Data Plane: IP".

[158]	IETF RFC 5279: "A Uniform Resource Name (URN) Namespace for the 3rd Generation Partnership Project (3GPP)".

[159]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[160]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)".

[161]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[162]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[163]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[164]	ITU-T Recommendation G.810: "Definitions and terminology for synchronization networks".

[165]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[166]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[167]	IETF RFC 9001: "Using TLS to Secure QUIC".

[168]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[169]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[170]	IETF RFC 9298: "Proxying UDP in HTTP".

[171]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

[172]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[173]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[174]	draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[175]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[176]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[177]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[178]	IEEE Std 802.1CBdb-2021: "Amendment 2: Extend Stream Identification Functions".

[179]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[180]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[181]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[182]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[183]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[184]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2".

[185]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[186]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[187]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[188]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[189]	IETF RFC 8285: "A General Mechanism for RTP Header Extensions".

[190]	3GPP TS 28.405: "Quality of Experience (QoE) measurement collection; Control and configuration".

[191]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[192]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[193]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[194]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[x]	3GPP TS 38.414: "NG-RAN; NG data transport".

NEXT CHANGE

## 5.x	Support for Mobile gNB with Wireless Access Backhauling (MWAB)

### 5.x.1	General

#### 5.x.1.1	Principles and functional entities

Mobile gNB with Wireless Access Backhauling (MWAB) provides an NR access link to UEs in proximity and connects to the 5GC serving the UE through an IP connectivity provided by a Backhaul PDU session(s), as illustrated in figure 5.x.1.1-1. The MWAB consists of a gNB component (MWAB-gNB) and a UE component (MWAB-UE). The MWAB-gNB is based on the gNB functionality specified in TS 38.300 [27] and TS 38.401 [42]. The MWAB may be mounted on a moving vehicle and may serve UEs inside or outside the vehicle.

The MWAB-UE establishes the IP connectivity for the backhaul links for the MWAB-gNB, via NR Uu, using the existing registration procedure and PDU session establishment procedure. The backhaul links are between the MWAB-gNB and entities of the network (e.g. AMF, UPF, other gNBs and OAM server) that a MWAB-gNB cell serves. The IP connectivity provided by the MWAB-UE may be either via the same PLMN/SNPN that the MWAB-gNB serves or a different PLMN/SNPN, depends on the MWAB-UE PLMN selection mechanism as specified in TS 23.122 [17]. Different possible deployment scenarios are presented in Annex X.

MWAB operation supports both PLMN and SNPN cases. When the MWAB-gNB is serving a PLMN, the UEs served by MWAB may be non-roaming or roaming in the MWAB Broadcasted PLMN. In case the MWAB-gNB is serving a SNPN, the subscribed SNPN of the UEs may be different from the MWAB Announced SNPN. The UEs served by the MWAB are not aware of the network serving the MWAB-UE.

Figure 5.x.1.1-1: MWAB architecture for 5GS

NOTE 1:	In this release, the CU/DU split of the MWAB-gNB is not supported.

NOTE 2: 	The interface between MWAB-UE and MWAB-gNB is implementation based and not in scope of this specification.

MWAB-UE has a single NR Uu hop to the NG-RAN, using either TN or NTN access technology.  NR access is used for the radio link between a MWAB-gNB and the served UEs. The NR Uu access link between the MWAB-gNB and the served UE(s) does not use NTN access technology. 5G MOCN can be supported by the MWAB-gNB.

NOTE 3:	The use of multiple PDU Sessions for N2, N3, Xn, OAM access, follows the existing URSP logic as defined in TS 23.503 [45].

The details of the MWAB configuration/provisioning process are described in clause 5.x.2.

In order to operate as MWAB, the MWAB-gNB component and MWAB-UE component are authorized and controlled separately. The detailed procedures for the authorization of MWAB are described in clause 5.x.3.

Service continuity for the UE(s) served by the MWAB-gNB is supported, when the MWAB moves and the UE(s) move or do not move together with the MWAB. The detailed procedures for the support of mobilty are described in clause 5.x.5.

The MWAB shall be able to serve UE(s) without any MWAB-specific enhancements. For some operations, the MWAB may be configured to provide access to only certain UEs. Existing access control mechanisms, e.g. CAG control, can be used to manage the UE(s)'s access to the MWAB-gNB. The details of the access control of the UEs served by MWAB are described in clause 5.x.6.

The LCS framework as defined in TS 23.273 [87] is used for providing the location service to the UE(s) served by MWAB. Details on supporting the LCS over MWAB are described in clause 5.x.7.

Regulatory services (e.g. emergency services, priroity services) can be supported by the MWAB, and the details are provided in clause 5.x.8.

#### 5.x.1.2	Support of QoS for UEs served by MWAB

Based on configuration and traffic mapping specified in TS 38.414 [x], the MWAB-gNB and the UPFs in the MWAB broadcasted PLMN set proper marking information (e.g. DSCP) for the N3 GTP-U traffic.  The MWAB-UE estlishes and modifies the BH PDU sessions according to the QoS requirements identified by the MWAB-gNB, e.g. based on the information related to the PDU sessions in the UE context(s) at the MWAB-gNB and/or OAM.  The MWAB-gNB takes the traffic mapping specified in TS 38.414 [x] into account (e.g. using a mapping of DSCP values used for N3 traffic in the MWAB Broadcasted PLMN/SNPN to 5QIs to be used in te BH PLMN) when generating the QoS Rules (including the required 5QI) for the BH PDU sessions to be used by the MWAB-UE.

Editor's Note: It is FFS if or how much of the contents in this claues should be documented in Annex.

#### 5.x.1.3	Support of network slicing over MWAB

Network Slicing is supported over MWAB-gNB. To support UEs' access to different slices in the MWAB Broadcasted PLMN, the MWAB-gNB may trigger the MWAB-UE to establish separate Backhaul PDU Sessions for the corresponding N2 and N3 interfaces for the UEs, e.g. accessing different AMFs or UPFs. The MWAB-gNB may be configured with the supported S-NSSAI(s) and the need of separate IP connectivity/PDU Sessions for N2 and N3 for the corresponding slice.  The MWAB-gNB UE obtains the S-NSSAI(s) of the UE(s) served by MWAB from the UE(s)' AMF, e.g. the Allowed or Partially allowed NSSAI, or in the S-NSSAI of the PDU sessions for a UE.

The association of the N2 and N3 connections to the Backhaul PDU Session(s) and the establishment of PDU session(s) are based on the MWAB-UE Local Configuation or URSP rules configured for the MWAB-UE, as specified in TS 23.503 [45].  The MWAB-gNB interacts with MWAB-UE using an MWAB internal interface to influence the association of the N2 and N3 traffic of different slices to the corresponding Backhaul PDU Session(s) of the MWAB-UE. The MWAB-gNB may also inform the MWAB-UE that a particular Backhaul PDU Session for the slice support is no longer required. The interactions between MWAB-gNB and MWAB-UE is out of scope of this specification.

Editor's Note: It is FFS if or how much of the contents in this claues should be documented in Annex.

Next CHANGEs

Annex X (informative): Architecture examples and protocol stacks for MWAB support

# X.1	General

This Annex provides system architecture examples for MWAB operations in PLMN cases and NPN cases. The example protocol stacks for the MWAB support is also provided.

# X.2	System architecture for MWAB operation in PLMNs

Figure X.2-1 presents an example architecture for the MWAB operation when no roaming was involved for the MWAB-UE. In this case, there may be two PLMNs involved, i.e. the BH PLMN that serves the MWAB-UE, and the MWAB Broadcasted PLMN that serves the UE connected to the MWAB. In some cases, the BH PLMN and the MWAB Broadcasted PLMN can be the same PLMN.

The MWAB-gNB logically belongs to MWAB Broadcasted PLMN and announces the PLMN IDs of the MWAB Broadcasted PLMN. The MWAB-gNB establishes N2 and N3 connection with the UE AMF and UE UPF in the MWAB Broadcasted PLMN via the PDU session of the MWAB-UE established with BH PLMN.

The BH UPF in the BH PLMN serves the MWAB-UE and provides the connection via a N6 interface towards the MWAB Broadcasted PLMN, to carry the N2 and N3 traffic from MWAB-gNB. The BH UPF also supports the access to the OAM servers in the MWAB Broadcasted PLMN by the MWAB-gNB.

NOTE:	Depending on deployment requirement, a security gateway may be required between the BH UPF and the MWAB Broadcasted PLMN network. In that case, the MWAB-gNB need to connect to the security gateway based on pre-configured security credentials. The traffic between MWAB-gNB and the MWAB Broadcasted PLMN goes inside the security tunnel established via the security gateway.

UE connected to the MWAB-gNB can access the 5GS services offered by MWAB Broadcasted PLMN. No MWAB specific enhancement to the UE is required. The UE connected to the MWAB-gNB is not aware of the BH PLMN, and thus does not need any roaming agreement between its HPLMN and the PLMN 1.

Figure X.2-1: Architecture for MWAB operation support - non-roaming

Figure X.2-2 presents an example architecture for the MWAB operation when MWAB-UE is roaming with a Local Breakout PDU Session for its operation. In this case, there may be three PLMNs involved, i.e. the BH PLMN that serves the MWAB-UE, and the MWAB Broadcasted PLMN that serves the UE connected to the MWAB, and the HPLMN of the MWAB-UE. The use of the Local Breakout PDU Session by the MWAB-UE can be configured by its HPLMN, e.g. with some VPLMN specific URSP rules. In some cases, the BH PLMN and MWAB Broadcasted PLMN can be the same PLMN.

The BH PLMN can access the MWAB-UE's HPLMN UDM for the subscription information. The rest of the operations are similar to that shown in figure X.2-1.

Figure X.2-2: Architecture for MWAB operation support - MWAB-UE roaming with Local Breakout

Figure X.2-3 presents an example architecture for the MWAB operation when MWAB-UE is roaming with a Home Routed PDU session for its operation. In this case, PDU session of the MWAB-UE is routed by BH PLMN to the HPLMN of the MWAB-UE. The rest of the operations are similar to that shown in figure X.2-2. In some cases, the BH PLMN and MWAB Broadcasted PLMN can be the same PLMN.

Figure X.2-3: Architecture for MWAB operation support - MWAB-UE roaming with Home Routed

In figure X.2-1, figure X.2-2 and figure X.2-3, if the UE served by the MWAB is roaming, there can be another PLMN (not shown in the figures), i.e. the HPLMN of the UE served by the MWAB, involved. The interaction of the HPLMN of the UE and the MWAB Broadcasted PLMN follows that described in clause 4.2.4 for the roaming case. The UE served by the MWAB-gNB is not aware of BH PLMN, and thus does not need any roaming agreement between its HPLMN and the BH PLMN.

# X.3	Architectures to support MWAB in SNPN case

The MWAB operation also supports SNPN, and the corresponding architectures are presented in figure X.3-1 and X.3-2. The MWAB operation also supports mixed PLMN and SNPN operation, i.e. in the architecture presented in X.3-1 and X.3-2, the BH SNPN can be replaced by a PLMN and MWAB Broadcasted SNPN remains a SNPN. The SNPN operations as described in clause 5.30 apply to the MWAB-UE and the UE served by the MWAB-gNB.

Figure X.3-1 presents an example architecture for the MWAB operation when the serving SNPN of the MWAB-UE is the same as subscribed SNPN of MWAB-UE. In this case, there may be two SNPNs involved, i.e. the BH SNPN that serves the MWAB-UE, and the MWAB Broadcasted SNPN that serves the UE connected to the MWAB-gNB. The MWAB-gNB logically belongs to MWAB Broadcasted SNPN and establishes N2 and N3 connection with the UE AMF and UE UPF via the PDU session of the MWAB-UE established with BH SNPN. MWAB-gNB announces SNPN ID of MWAB Broadcasted SNPN.

Figure X.3-1: Architecture for MWAB operation support for SNPN – with MWAB-UE served by the subscribed SNPN (BH SNPN)

Figure X.3-2 presents an example architecture for the MWAB operation when the MWAB-UE accesses its serving SNPN using credentials owned by a Credential Holder. In this case, besides the two SNPNs, i.e. the BH SNPN that serves the MWAB-UE, and the MWAB Broadcasted SNPN that serves the UE connected to the MWAB-gNB, the Credential Holder of MWAB-UE is also involved. The MWAB-gNB logically belongs to MWAB Broadcasted SNPN and establishes N2 and N3 connection with the UE AMF and UE UPF via the PDU session of the MWAB-UE established with BH SNPN. MWAB-gNB announces SNPN ID of the MWAB Broadcasted SNPN.

Figure X.3-2: Architecture for MWAB operation support for SNPN – with MWAB-UE accesses BH-SNPN with credentials from a CH

End of CHANGEs