3GPP TSG-WG SA2 Meeting #164	S2-2409295

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-2408551, 8751)

Source:	Huawei, HiSilicon

Title:	KI#2, Sol#2 Update: Enabling Operator and 3rd Party Assignment of IDs

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Solution 2 updates to add information on how to assign either Operator or 3rd Party ID and use different elements of the AIoT Device ID to access subscription-like information in different scenarios.

# 1. Introduction/Discussion

The Ambient IoT Device ID format is updated to allow 2 different exclusive assigners, either an Operator or a 3rd Party. The operator assigned element of the format is now left to be entirely for the operator to control, allowing flexibility on the contents. The 3rd Party assigned ID is flexible as well and can support e.g., EPCs.

It is then further clarified how the fields within the Device ID can be used to access subscription-like information for both the operator assigned and 3rd Party assigned formats.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

## 6.2	Solution #2: Operator or 3rd Party Assigned AIoT Device ID

### 6.2.1	Description

This solution addresses the KI#2 and the aspect about how to identify and format the identifier of Ambient IoT Device in order to identify specific Ambient IoT Device or a group of Ambient IoT Devices.

In this solution, it is assumed that the Ambient IoT Device is either configured with Operator allocated identifier or configured with 3rd Party allocated identifier (e.g. an EPC).

According to TS 22.369 [2], the 5G system shall provide suitable mechanisms to support communication between an authorized 3rd party and an Ambient IoT device or group of Ambient devices. In addition, subject to user consent, operator's policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the 3rd party via the 5G network. Based on these service requirements, the relationship among the Ambient IoT Device, Mobile Network Operator and the 3rd party is illustrated below.

Figure 6.2.1-1: Relationship Among Ambient IoT Device, Mobile Network Operator and 3rd Party

It is assumed an Ambient IoT Device is owned by a third party who has a service agreement with a Mobile Network Operator to enable Ambient IoT service in 3GPP system. A PLMN may manage the Ambient IoT Device (e.g. holds the credentials of Ambient IoT Device, etc) and enables the communication between the 3rd party and the Ambient IoT Device via the 5G network. Therefore, based on this model, in the 3GPP system, different PLMNs may manage Ambient IoT Devices owned by different third parties. If uniqueness of Device ID cannot be guaranteed, the management of Ambient IoT Devices and services provided by the 3GPP system may be impacted.

The assignment an Ambient IoT Device ID is either by:

-	an Operator, or

-	a 3rd Party.

The AIoT Device is configured with either an Operator assigned Ambient IoT Device ID or a 3rd Party assigned Ambient IoT Device ID.

Operator assigned Device IDs have the following components:

-	Network Identifier: an identifier to indicate the network operator (e.g. MMC+MNC) which allocated the Device ID and holds the subscription-like information for the AIoT Device.

-	Operator Allocated Identifier: an identifier used to identify the specific Ambient IoT Device.

NOTE 1:	The Operator Allocated Identifier format is determined by the operator and can follow any format. For example, it can be divided into subfields e.g. Enterprise ID or Group ID and an Instance ID, or use any other format, e.g. EPC, which, depending upon other solutions, for example could be used as part of a MASK during inventory operations.

Figure 6.2.1-2: Structure of Ambient IoT Device ID assigned by Operator

3rd Party assigned Device IDs have the following components:

-	Network Identifier: an identifier to indicate the operator (e.g. MCC+MNC) who holds the subscription-like information for the Ambient IoT Device. The Network Identifier may be set to indicate no operator holds subscription-like information for the AIoT Device, e.g. by setting the value to 999999 or a flag to indicate this.

NOTE 2:	If the Network Identifier is set, then it is expected that the Device ID (Network Identifier + Application Allocated Allocated) will be able uniquely identify the AIoT Device, to allow e.g. subscription-like data retrieval. How the Operator and 3rd Party ensure uniqueness is operator implementation dependent.

NOTE 3:	If it is required that AIoT Devices must have subscription-like information then the Network Identifier needs to indicate the operator holding the subscription-like information and setting Network Identifier to indicate no operator holds subscription-like information will not be supported.

-	Application Allocated Identifier: an identifier used to identify the specific Ambient IoT Device allocated by the 3rd Party. The format of the Application Allocated Identifier is determined by the 3rd Party, for example an EPC.

Figure 6.2.1-3: Structure of Ambient IoT Device ID assigned by 3rd Party

The length of the Ambient IoT Device ID is variable according to the service requirements.

The Ambient IoT Device ID is a permanent identifier and of the Network Identifier is set is used by the MNO to derive subscription-like data related to Ambient IoT Devices.

When a 3rd party (e.g. AF) sends a service request (e.g. inventory or command) to 5GC, the 5GC can trigger the reader(s) to inventory a group of Ambient IoT Device by broadcasting a partial/full Ambient IoT Device ID. The Ambient IoT Devices matching the broadcasted message will perform random access responding to the broadcast message. For example, when the partial value is the Network Identifier, an Ambient IoT Device matching that partial value (i.e. belonging to a specific PLMN), will respond to the broadcast message to perform random access and report to the network.

With such format, the network can enable different group of Ambient IoT Device to respond the broadcast message for inventory.

Editor's note:	It is FFS whether it can be assumed that the device and the CN can be pre-provisioned with Ambient IoT Device ID and the optional 3rd Party-defined identifier.

Depending on the agreements between a PLMN and an AF, the PLMN may allow the 3rd Party (i.e., the owner of the device) to provision the Ambient IoT Device information to a PLMN via an OAM mechanism or NEF provisioning service, and further to re-home the management to different PLMNs as the Ambient IoT device travels through the supply chain. A PLMN may first check whether subscription-like information has been provisioned locally first before using the Network Identifier to locate subscription-like information. The Ambient IoT Device information provisioned locally may be full subscription-like information or a delegation to a credential holder outside the PLMN (depending on operator policy). Hence, a PLMN can locally the control of the AIoT Device with local subscription-like information (e.g., in a UDM) or by via an external credential holder, without interaction with other networks.

### 6.2.1a	Locating subscription-like information

When a PLMN is interacting with a specific AIoT Device (e.g., sending commands/data) it can use the Network Identifier component of the Ambient IoT Device ID to locate where subscription-like information is for the device and then the Operator Allocated Identifier or 3rd Party Allocated Identifier component of the Ambient IoT Device ID to locate the subscription-like information for the specific Ambient IoT Device. How an AF requests an operation that results in interaction with the device will depend on other solutions (e.g., it may be directly request to the PLMN interacting with the device, via a partner PLMN, etc is not assumed by this Device ID solution).

The following is possible, based on the value of the Network Identifier, as shown in Figure 6.2.1a-1:

-	If the Network Identifier matches the PLMN making the request, then this PLMN itself holds the subscription-like information.

-	If the Network Identifier is another PLMN then the other PLMN holds the subscription-like information and the PLMN interacting with the device can contact the other PLMN for access to the subscription-like information.

-	If the Network Identifier is set to indicate no network holds subscription-like information, then based on the local configuration or a delegation to a Credentials Holder, the PLMN interacting with the device can contact a Credentials Holder. If a Credential Holder cannot be determined or accessed then no subscription-like information is available for the AIoT Device and any services, actions or security operations, etc that require subscription-like information cannot be completed.

NOTE	: How the PLMN determines a Credential Holder for a specific device is up to operator policy.

Figure 6.2.1a-1: Locations for Subscription-like information

### 6.2.2	Procedures

How to utilize such format of Ambient IoT Device ID when network performs service operations will be specified in the call flows in other solutions.

### 6.2.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# * * * * End of changes * * * *