3GPP TSG-SA2 Meeting #164	S2-2409308

# Maastricht, Netherlands, 19 - 23 August 2024			 (revision of S2-2409028)

Title:	LS on Subscription of UPF event via I-SMF

Response to:	S2-2407446

Release:	Rel-18

Work Item:	UPEAS

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	SA2 23.501 CR5473 and 23.502 CR4923

- Overall Description:

SA2 thanks CT4 for their LS. SA2 have discussed it and has the following answers to CT4 observations on CR 23.502 #4786 rev.3 :

- CT4 observation: “whether SA2 considered both approaches above, and if so, on which criteria should the anchor SMF invoke the Nsmf_PDUSession API vs. the Nsmf_EventExposure to subscribe to UPF events via the I-SMF?”

SA2 answer: SA2 has revised the 23.502 CR (see attachment) to use Nsmf_EventExposure for the anchor SMF to subscribe to UPF events via the I-SMF.

- CT4 observation: whether the anchor SMF should be able to determine if the requested UPF events/service are supported by the I-SMF/I-UPF, before triggering a subscription towards the I-SMF, and if so, how.

SA2 answer: SMF needs to be able to determine whether relaying of subscription to UPF events is supported by the I-SMF. About what UPF events/service are supported by local UPF(s) controlled by I-SMF, the determination is up to I-SMF and not to SMF. Which means I-SMF may reject a Nsmf_EventExposure SUBSCRIBE from the anchor SMF if one of the target local UPF(s) doesn’t support the event.

- CT4 observation: whether the anchor SMF may need to subscribe to UPF events at the ULCL/BP and/or at the local PSA UPF;

SA2 answer: the anchor SMF may need to subscribe to UPF events at the local ULCL/BP and/or at a local PSA UPF.

- CT4 observation: how to support subscriptions at the anchor SMF targeting ANY UE, e.g. ANY UE in a specific area, e.g. whether the anchor SMF should determine by itself the UEs present in the specific area and trigger then subscriptions, per PDU session, for the corresponding UEs?

SA2 answer: .SA2 needs more time to analyze this case and will contact CT4 when this analysis has been carried out.

- Actions:

To CT4:

ACTION: 	SA2 kindly asks CT4 to take the above information into account.

- Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#165 | October 14th – 18th, 2024 | Hyderabad | India |
| SA2#166 | November 18th –22nd, 2024 | Orlando | USA |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/