3GPP TSG WG2 Meeting #164	S2-2409317

19 - 23 August 2024, Maastricht, Netherlands	(revision of S2-2409041)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5454 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Handling of Headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Vodafone, ETRI, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification of KI#3 outcome Enhancement of UPF handling of headers according to approved WID: SP-240995 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to incorporate the agreed enhancements for KI#3 as in WID, as follows: - Enhance Nnef_TrafficInfluence service to accommodate requests from AFs for Handling of payload headers according to Table 6.1.2.1-1 from TR 23.700-63. - Enhance the procedure in clause 4.3.6 of TS 23.502 to provision the handling information to the SMF/UPF - A new data subset in the Application data set is provisioned in the UDR. - NEF, PCF, SMF and UPF are enhanced to support the Handling of payload headers - UPF profile in NRF is enhanced with the new UPF capability to support the corresponding UPF selection and discovery - Enhancing Nupf_EventExposure for Header/Tag reporting. - Enhancing N4 PFCP rules to instruct reporting of headers/tags handling upon AF request according to option 2 in TR 23.700-63. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of Headers Enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.x (new), 5.6.x.1 (new), 5.6.x.2 (new), 5.8.2.8.x, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 4877, TS 23.502 CR 1329 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all new) * * * *

### 5.6.x	Handling of Payload Headers

#### 5.6.x.1	General

Handling of Payload Headers is an optional feature that allows exchange of information in-band in the user plane packets that are exchanged between the application (in the UE or in the application server) and UPF in the 5GS. Handling of Payload Headers can be requested by the AF.

NOTE 1: 	The Handling of Payload Headers is not limited to the protocol header but can be any field or tag supported by a protocol.

NOTE 2:	Handling of Payload Headers enhances Header Enrichment (see clause 5.8.5.6) by allowing more actions than just insertion, that can be applied in uplink and in downlink directions.

The content of this clause applies to non-roaming and to Home Routed roaming scenario, i.e. to cases where the involved entities (AF, PCF, SMF, UPF) belong to the Home PLMN and the AF has an agreement with the Home PLMN.

The PCF controls Handling of Payload Headers in UPF by provisioning Header Handling Control Information in the PCC Rules as described in clause 6.3.1 of TS 23.503 [45]. The PCF derives the Header Handling Control Information in the PCC rule as described in 6.1.3.x of TS 23.503 [45] based on the AF request. SMF instructs UPF via N4 interactions.

Editor’s note:	Which rule to use for the N4 interaction is FFS.

The UPF performs Handling of Payload Headers (i.e. detection, reporting, insertion, replacement, or removal of headers) and may notify events related to the Handling of Payload Headers to SMF, to NEF/AF or to both as described in clause 5.6.x.2.

#### 5.6.x.2	Application Function influence on Handling of payload headers

For the Handling of Payload Headers, it is assumed that a service level agreement (SLA) exists between the operator and a third party. The mechanism(s) required for detection and manipulation of payload headers (e.g. protocol layer, type of encryption, etc.) to enable exchange of information in-band in user plane packets using header Handling information is agreed as part of the SLA. The reference to a system pre-configuration(s) corresponding to the header handling detection to be applied by the UPF is also part of the SLA. The AF is required to provide such reference  in the AF requests.

In the non-roaming scenario, Handling of Payload Headers, Application Function influence on Service Function Chaining (as defined in clause 5.6.16) and Application Function influence on traffic routing (as defined in clause 5.6.7) can be applied simultaneously.

In the Home Routed roaming scenario, Handling of Payload Headers and Application Function influence on Service Function Chaining (as defined in clause 5.6.16) can be applied simultaneously.

The AF requests are sent to the PCF via N5 (in the case of requests targeting specific on-going PDU Sessions of individual UE(s), for an AF allowed to interact directly with the 5GC NFs) or via the NEF. The AF requests that target existing or future PDU Sessions of multiple UE(s) or of any UE are sent via the NEF and may target multiple PCF(s), as described in clause 6.3.7.2. The PCF(s) transform(s) the AF requests into policies that apply to PDU Sessions. When the AF has subscribed to events related to Handling of Payload Headers, such notifications are sent by UPF either directly to the AF or via an NEF.

When the request is sent via NEF, Nnef_TrafficInfluence service is used. The procedures for AF to request Handling of Payload Headers are described in clause 4.3.6 in TS 23.502 [3]. For AF interacting with PCF directly or via NEF, the AF requests may contain the information as described in the Table 5.6.x.2-1:

Table 5.6.x.2-1: Information elements contained in AF request

| Information Name | Applicable for PCF or NEF (NOTE 1) | Applicable for NEF only | Category |
| --- | --- | --- | --- |
| Traffic Description | Defines the target traffic on which to apply handling of headers, represented by the combination of DNN and optionally S-NSSAI and application identifier or traffic filtering information. | The target traffic can be represented by AF-Service-Identifier, | Mandatory |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. an individual UE, a group of UE represented by Internal Group Identifier(s) (NOTE 2), or any UE accessing the combination of DNN, S-NSSAI | GPSI can be applied to identify the individual UE, or External Group Identifier can be applied to identify a group of UEs. | Mandatory |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. | Optional |
| Temporal Validity Condition | Time interval(s) when the request applies or duration(s). | N/A | Optional |
| AF transaction identifier | The AF transaction identifier refers to the AF request. | N/A | Mandatory |
| Handling of Headers Control information (UL/DL) |  |  |  |
| Header Handling Detection Reference | A reference that points to predefined configuration which defines in UPF how to detect header for the handling of this request (e.g. the protocol, message in a protocol). | N/A | Mandatory |
| Header handling Information | AF input to identify the headers to perform the Header Handling manipulation(s) or reporting a detection for a Header handling detection | N/A | Optional |
| Header Handling Additional Data (NOTE 3) | Additional data to complement the Header Handling information. | N/A | Optional |
| Header Handling Direction | Indicates if the header handling applies to UL or DL direction. | N/A | Optional |
| Header Handling manipulation(s) | The header handling manipulations the AF wants on the target traffic. | N/A | Optional |
| Header Handling Condition | It may be included to indicate if a requested manipulation is to be performed always or once. | N/A | Optional |
| Subscription information for header handling events | Provides the needed input for reporting events. | N/A | Optional |
| Header handling pre-configuration reference | A reference to a pre-configured parameter set in UPF | N/A | Optional |
| NOTE 1: When the AF request targets existing or future PDU Sessions of multiple UE(s) or of any UE and is sent via the NEF, as described in clause 6.3.7.2, the information is stored in the UDR by the NEF and notified to the PCF by the UDR. NOTE 2: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. NOTE 3: The NEF, PCF and SMF do not need to understand the Header Handling Additional Data. |  |  |  |

For each information element mentioned above as part of Handling of Headers control information in the AF request, a detailed description follows:

1)	Header Handling Detection Reference:

Reference to a predefined configuration in UPF which defines the header handling detection information (e.g. protocol layer, type of encryption, message type etc). This is mandatory information that is complemented by the following information elements.

2)	Header Information:

The Header Information contains input information to identify the Header(s) for the Header Handling  Manipulations and/or reporting. AF may provide the Header Information as follows:

-	Header Name and Header Value: Strings used to build header name and header value according to the syntax of the protocol selected. For example, AF provides Header Name = path, and Header Value= GDomain.com. Header Names are not standardised and they are based on local configuration and SLA.

- 	AF may request a Header Name without requesting a Header Value. This is the case of value information only known by the 5GC functions on a per PDU basis. In such a case, the 5GC provides the actual Header Value. E.g. AF provides Header Name = RAT and SMF, based on session information and predefined configuration, sends to UPF header Name= RAT and header Value=NR.

The SMF may have a pre-configuration for the Header Handling Detection Reference received from PCF. SMF checks the Header information received and adds Header values, if needed, according to its local configuration and session information before sending the Header Information to UPF.

NOTE 1:	Header name is an agreed identifier between the operator and 3rd-party as part of the SLA. For example, it can take the actual name of an HTTP header if HTTP is used.

Editor’s Note: it is FFS whether some use case requires additional alternatives for Header information, for example a Header Reference.

3)	Header Handling Additional Data:

This additional data is not standardised, but it can be interpreted by UPF based on SLA. It is sent transparently by NEF, PCF and SMF to UPF. In UPF, it complements the UPF Header Handling Detection Reference and the Header Information.

4) 	Header Handling Direction:

Header Handling Direction indicates to which direction the Header handling Control information applies to. Either uplink (UL) or downlink (DL).

5)	Header Handling manipulation(s):

The manipulation(s) that AF may request on the target traffic. One or more of the following can be requested:

-	Remove. It can be used to request removal of certain header. Remove only considers the header name.

-	Replace. It can be used to request replacement of certain header value. The header value to be replaced is detected by the header name. The header name is not modified, and the header value is replaced.

-	Insert. It can be used to add certain header and when applicable, the associated value.

When multiple manipulations are requested on the target traffic, the enforcement of the Header Handling Manipulation is done in the following order:  Remove, Replace, and Insert.

NOTE 2:	The execution order is relevant to build a request that has the intended impact on the traffic. For example, a header that is removed cannot be replaced.

NOTE 3: Any request from the AF to manipulate certain traffic implies a detection as indicated by the Header Handling Detection Reference. When no manipulation request exists, detection and report are still assumed.

6)	Header Handling Condition:

This defines how to apply the Header Manipulation. It may have one of the following values: every match related to the Handling of Headers Control Information or only for the first match

NOTE 3:	In addition, Spatial Validity Condition and Temporary Validity Condition parameters in the request can be used to indicate that the request applies only in certain UE location (s) or during certain time interval respectively.

7)	Subscription information for header handling events

The AF and SMF may subscribe to receive reporting of header handling detection or of header handling manipulation from UPF. The AF may need to subscribe via NEF. The AF subscription information may include:

-	Notification endpoint (including correlation id) of the consumer of the subscription.

-	Reporting Frequency. The manipulations can be requested to be notified only once for the PDU session, periodically (in this case, a reporting period is also provided) or whenever the manipulation is executed. In the latter case concatenation applies to avoid reporting for every matched/detected PDU.

-	Reporting suggestion information as defined in clause 5.8.2.17 to reduce the UPF performance impacts.

The PCF sends the Information on AF subscription to notification of UP Handling of Header event to SMF if any has been received. The SMF sends the Notification target address for receiving event notifications to UPF. If SMF determines based on local configuration that it shall receive the notifications, it instructs UPF to duplicate the reporting and subscribes to the UPF event exposure service.

Editor’s note: how to describe that only one report may be is sent for a set of manipulations is FFS

8)	Header handling pre-configuration reference:

This refers to a pre-configuration in UPF for parameters 2-6.

NOTE 4: 	In a deployment, for simplicity, an operator can choose to set this reference to the same value as the Header Handling Detection Reference

# * * * * 2nd change (all new) * * * *

#### 5.8.2.8.x 	Handling of Payload Headers by UPF

Handling of Payload Headers is an optional feature of UPF.

When UPF receives a request for Handling of Payload Headers from SMF over N4, the request contains a Header Handling Detection Reference which points to a predefined configuration in UPF which includes the details of the protocol and possibly the message for which a header handling shall be performed (e.g. HTTP, HTTP message). The interpretation of the Header Handling Detection reference is part of the SLA and thus, implementation specific.

The request may contain explicit Header Handling Control information to be used for handling the traffic detected, see clause 5.6.x.2. In this case of explicit information requested by an AF, UPF receives the Header Handling Detection Reference from SMF and other parameters that the AF has provided as described in clause 5.6.x.2 .

# * * * * 3rd change * * * *

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE: Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14

-	Exposure of the UE information, e.g. UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Handling of Payload Headers as defined in clause 5.8.2.8.x.

# * * * * End of changes * * * *