3GPP TSG-WG SA2 Meeting #164	S2-2409408

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-2408073, 8852, 9113)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1427 | rev | 3 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture for UE-satellite-UE communicaitons |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Thales, CATT, CSCN, Honor, Tencent, China Telecom, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. - Understanding the architecture is crucial for identifying the affected interfaces and assessing the potential impacts or enhancements needed to support the introduced feature. In the context of UE-SAT-UE, several interfaces will involve satellite transport links (e.g feeder links, and/or inter-satellite links). Implementing this feature requires specific assumptions and requirements for these links. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an architecture to describe the interfaces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-satellite-UE communication will not be activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.2.1.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is supposed to be implemented together with CR 1428, after the clause of AX.2 Architecture and functional entities |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# AX.2	Architecture and functional entities

## AX.2.1	Architecture

### AX.2.1.1		Support of UE-satellite-UE communication with IMS-AGW deployed on satellite(s)

Editor’s NOTE:	the terminology of UE-satellite-UE communication is FFS.

To support UE-satellite-UE communications, as depicted in Figure AX.2.1.1-1, the IMS-AGW is deployed on the satellite(s) that host the gNB and UPF (UL CL/BP and L-PSA) of the 5GC.

NOTE:	It is assumed that the satellite(s) can always connect to the ground with IP transport networks.

Figure AX.2.1.1-1: UE-Satellite-UE communication architecture in Reference points representation

NOTE 1:		Iq interface is over satellite transport layer links (feeder link and optionally inter-satellite links), where the lower layer protocol is out of 3GPP scope;

NOTE 2:	For clarity, the connections within 5GC and IMS core are not fully depicted in the architecture diagrams. For more information on 5GC architectures refer to clause 4.2.3 in TS23.501 [93].

# * * * * End of changes * * * *