3GPP TSG-SA SA2 Meeting #164	S2-2409506

Maastricht, The Netherlands, 19 Aug – 23 Aug, 2024

Source:	Ericsson, Huawei, Nokia, Nokia Shanghai Bell

Title:	KI#2-NW-Assisted/Ground-based DAA: Update Conclusions

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes to update the conclusions for KI#2 NW-assisted/ground-based DAA in TR 23.700-59.

# 1.	Discussion

Solution#5 in TR 23.700-59 also proposes to use Ranging/Sidelink Positioning to address KI#2 on network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information. However, this positioning technique is not mentioned in the conclusion even though the Ranging/Sidelink Positioning provides the 3GPP way of serving relative location requests between UAVs and thus addressing the issue described by KI#2.

# 2.	Text proposal

In line with the proposals made in the previous clause, it is proposed to agree the following changes to TR 23.700-59 v1.0.0:

* * * First Change * * * *

## 8.2	Conclusion for Key Issue#2

Following information are agreed to be utilised to enable NWDAA:

-	Absolute location of a UAV provided by LoCation Service;

-	Relative Location of UAVs provided by Ranging Service;

-	Output of Relative Proximity Analytics provided by NWDAF.

Absolute location of a UAV is generated/derived in 5G network by using the existing LCS feature defined in TS 23.273 [7]. USS may request to GMLC to obtain one or multiple UAV location as defined in clause 6.2.1, 6.3.1 or 6.20.5 in TS 23.273 [7]. As an example, USS may use the absolute location of the UAV(s) provided by the network to e.g. calculate relative distance of the pair of UAVs, and based on that to deal with the UAV tactical deconfliction, collision avoidance etc. which is out of SA2 scope.

Relative location of UAVs is provided in 5G network by using the existing LCS Ranging feature defined in TS 23.273 [7]. USS may send a Ranging request to GMLC to obtain a relative location between one and multiple UAVs as defined in clause 6.20.3 or 6.20.4 of TS 23.273 [7]. USS may use the reported relative location of the UAV(s) to avoid collisions.

USS (as an AF) may request Relative Proximity Analytics provided by NWDAF for the purpose of DAA.

NOTE 1:	How USS uses the relative distance of UAVs or absolute location of a UAV is out of 3GPP scope.

NOTE 2:	Applicability of the above solutions to support DAA will be evaluated during normative phase.

# *** End of Changes ***