SA WG2 Meeting #164	S2-2409507

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-2408942 of 07839)

Source:	LG Electronics, Ericsson, Futurewei, Nokia, Nokia Shanghai Bell, Huawei, CATT, Interdigital, Samsung, Deutsche Telekom

Title:	KI#3: Conclusion update

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes to update Conclusion for KI#3.

Discussion

KI#3 conclusion needs to be finalized to complete FS_UAS_Ph3.

SA2 discussed open issues related to KI#3 via NWM before SA2#164 (please refer to S2-2407847 "[FS_UAS_Ph3] Report of Pre-SA2#164 NWM Discussion for KI#3") and during SA2#164, but could not reach consensus in additional solutions where CN and RAN are aware of NTZ. Please also refer to S2-2407430 "LS from TSG RAN: Reply LS RP-240891 on NTZ solution impacts to RAN" about RAN impact and awareness for NTZ.

As part of the drafting session at SA2#164, the proposed way forward that is described in clause 8.4 "Final Conclusion for Key Issue #3: Support of No Transmit Zones" was derived. No impact is expected in RAN and CN.

Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

8.3	Interim Conclusion for Key Issue #3: Support of No Transmit Zones

8.3.1	General

The following principles are agreed to support NTZ.

1)	NTZ enforcement applies to the UE that is a UAV UE (aka an aerial UE) with aerial subscription.

2)	NTZs are supported for both LTE and NR.

3)	An NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

4)	No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in three dimensions to consider height restrictions), regardless of the service type.

5)	No UAV UE transmits or attempts to transmit in the restricted frequencies, location and height defined by NTZ. But UAV UEs can transmit/receive normally outside of these restrictions.

6)	UAV UEs supporting NTZ regulations are configured with NTZ information, so that these UAV UEs will be aware of the presence of NTZs.

7)	Reception of downlink data from the network by the UAV UE located in the NTZ is allowed given the regulatory requirements are not violated.

8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information (or NTZ policy) is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

NOTE 2:	Whether all options are required will be determined in normative phase.

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP Release.

The UAV UE supporting NTZ (i.e. Rel-19 and onward UAV UEs) performs the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data.

c)	does not select/reselect any cells impacted entirely by an NTZ.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

NOTE 4:	For the network to know about UAV presence inside NTZ, whether registration updates by the UAV UE before entering the NTZ and when exiting the NTZ are needed or the existing mechanism (e.g. AMF subscribing with NG-RAN by providing NTZ as AoI) can be applied will be decided during normative phase. This work does not have RAN impacts.

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

* * * * Start of Next Change * * * *

8.4	Final Conclusion for Key Issue #3: Support of No Transmit Zones

The Interim conclusion described in clause 8.3 is superseded by Final conclusion described in this clause.

The following principles are agreed to support NTZ.

1)	NTZ enforcement applies to the UE that is a UAV UE (aka an aerial UE) with aerial subscription.

2)	NTZ enforcement by UAV UE applies for both LTE and NR.

3)	An NTZ can map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE 1:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

NOTE 2: NTZ mapping to cells will have no impacts on 3GPP specifications, hence it is UE implementation specific.

4)	No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in five dimensions to consider 3D (including height) restrictions, frequencies and time), regardless of the service type.

5)	UAV UE neither transmits nor attempts to transmit in the NTZ. But UAV UEs can transmit/receive normally outside of the NTZ.

6)	UAV UEs supporting NTZ regulations are configured with NTZ assistance information, so that these UAV UEs will be aware of the presence of NTZs.

The UAV UE(s) configured with the following NTZ assistance information is responsible to provide such assistance information to the modem and to perform NTZ enforcement in implementation dependent ways.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 3:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE in this release by using:

-	pre-configured;

-	configured by using mechanism that is out of 3GPP scope.

The UAV UE(s) configured with the NTZ assistance information is expected to perform the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data with restricted frequencies.

c)	does not select/reselect any cells impacted entirely by an NTZ.

* * * * End of Changes * * * *