3GPP TSG-WG2 Meeting #164	S2-2409513

# Maastricht, The Netherlands, August 19-23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0128 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1.2: Adding a new NEF service operations for USS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Procedure for USS changeover uses new NEF service operation for interaction between NEF/UAS NF and USS, but they are not specified. These service operatons are: - Nnef_UAVFlightAssistance_Get, - Nnef_UAVFlightAssistance_Create, - Nnef_UAVFlightAssistance_Update, and - Nnef_UAVFlightAssistance_Notify |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a description the aforementioned services operations to clause 4.4.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new service operations are not specified, and the corresponding EN remains in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1, 4.4.1.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.256 CR 0126, 0127 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | If this CR is agreed, the corresponding EN in Step 4 in CR 0127 needs to be removed. Clause 5.X referenced in clause 4.4.1.1.X.2 is being proposed by CR 0126. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

#### 4.4.1.1	NEF Services

##### 4.4.1.1.1	General

In addition to those defined in TS 23.501 [2] clause 7.2.8 and TS 23.502 [3] clause 5.2.6, the following table illustrates additional NEF services to support UAS.

Table 4.4.2.1.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_Authentication | AuthenticateAuthorize | Request/Response | AMF, SMF |
|  | Notification | Subscribe/Notify | AMF, SMF |
| Nnef_UAVFlightAssistance | Get | Request/Response | AF/USS |
|  | Create | Request/Response | AF/USS |
|  | Update | Request/Response | AF/USS |
|  | Notify | Request/Response | AF/USS |

>>>> Next Changes <<<<

##### 4.4.1.1.X	Nnef_UAVFlightAssistance service

4.4.1.1.X.1	General

Service Description: This service enables the consumer to request assistance for USS changeover if/when a UAV/UE moves from a geographical area served by one USS to an area served by another USS. For this purpose, a USS requests a core network to provide the assistance information for the serving USS, to inform the core network about the flight trajectory assigned and time schedule for the UAV/UE, and to receive notifications when the UAV/UE moves out/in the geographical area served by a USS triggering and completing the changeover.

Editor’s Note: Whether the GET operation is needed is FFS.

4.4.1.1.X.2	Nnef_UAVFlightAssistance_Get service operation

Service operation name: Nnef_UAVFlightAssistance_Get

Description: Requesting from a NEF/UAS NF assistance with USS changeover.

Input, Required: UAV's identifier (e.g. GPSI), indication about USS changeover.

Input, Optional: list of addresses of suitable target USS, a list of candidate border-crossing point(s), acceptable deviations for flight plan/route, information about candidate flight path(s), requirements on the flight path as described in clause 5.X.

Output, Required: None.

Output, Optional: pre-mission flight planning assistance information as described in clause 5.X.

4.4.1.1.X.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a destination point (even when the destination point is located in geographical area served by different USS).

Editor’s Note: Representation of the destination point and border crossing point format is FFS.

Input, Required: UAV's identifier (e.g. GPSI).

Input, Optional: flight path information, which may include e.g. a list of TA, UAV’s speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells).

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

4.4.1.1.X.4	Nnef_UAVFlightAssistance_Update service operation

Service operation name: Nnef_UAVFlightAssistance_Update

Description: Used by the service consumer to update the core network about changes in the previously provided UAV flight path(s) and/or time schedule.

Input, Required: Flight plan configuration ID.

Input, Optional: flight path information, which may include e.g. a list of TA, UAV’s speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells).

Output, Required: None.

Output, Optional: None.

4.4.1.1.X.5	Nnef_UAVFlightAssistance_Notify service operation

Service operation name: Nnef_UAVFlightAssistance_Notify

Description: Notifying a USS about UAV leaving its serviced area and which border crossing point will be used for this purpose, informing a target USS about the UAV entering its serviced area, and confirming the completion of the changeover.

Input, Required: Flight plan configuration ID, status of the UAV according to the assigned flight path (leaving, entering, deviating)

Input, Optional: information about border-crossing point that will be used for USS changeover;

Output, Required: Indication whether the USS changeover is completed successfully when used by the target USS when responding to the notification

Output, Optional: None.

>>>> End of Changes <<<<