3GPP TSG-WG SA2 Meeting #164	S2-2409528

# Aug 19 – 23, 2024, Maastricht, NL		(revision of S2-2409247)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0547 | rev | 3 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General Support for MWAB involved positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson, CATT, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding support for UE positioning when an MWAB is involved, based on the conclusions in TR 23.700-06. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes added: - Update the definitions - New Abbreviation - LMF support of MWAB - NRF support discovery of LMF supporting when MWAB is involved. - Update the NL10 reference point - LMF discovery and selection supporting MWAB - New feature discription for Location Service involving Mobile Wireless Access Backhaul |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Location Service involving Mobile Wireless Access Backhaul. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.3.8, 4.3.12, 4.4.15, 5.1, 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.501 CR 5596 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G enhanced positioning area: see TS 22.261 [3].

5G positioning service area: see TS 22.261 [3].

LCS Client: entity that interacts with GMLC for the purpose of obtaining location information for one or more UEs. The LCS Client may reside in the UE.

For the purposes of the present document, the following terms and definitions given in TS 23.271 [4] apply:

Call Related: see TS 23.271 [4].

Codeword: see TS 23.271 [4].

Current Location: see TS 23.271 [4].

Deferred location request: see TS 23.271 [4].

DL Positioning: positioning of a target UE in which the target UE obtains downlink measurements for a 3GPP RAT.

GNSS Assistance Data: see clause 6.5.2.1 of TS 37.355 [20].

Immediate location request: see TS 23.271 [4].

Last Known Location: see TS 23.271 [4].

LCS (LoCation Services): see TS 23.271 [4].

Local Co-ordinates: see TS 23.032 [8].

Local Location: location determined by Local Co-ordinate(s).

Located UE: see TS 23.586 [40].

Location Estimate: see TS 23.271 [4].

LOS/NLOS measurement indication: LOS-NLOS-Indicator as defined in TS 37.355 [20] or LOS/NLOS information as defined in TS 38.455 [15].

Positioning Reference Unit (PRU): see TS 38.305 [9].

Mobile Base Station Relay: see TS 23.501 [18].

Mobile gNB with Wireless Access Backhauling (MWAB): see TS 23.501 [18].

PRU association: association of a PRU with an LMF by providing PRU related information to an LMF.

PRU dis-association: remove the PRU related information to dis-associate a PRU with an LMF.

Pseudonym: see TS 23.271 [4].

Pseudonym mediation device: Functionality that verifies pseudonyms to verinyms.

RAT Independent Positioning: positioning of a target UE in which the target UE obtains measurements not related to a 3GPP RAT.

Requestor: see TS 23.271 [4].

Requestor Identity: see TS 23.271 [4].

Response Method: for LCS Client using the OMA MLP protocol. Detail see TS 23.271 [4].

Scheduled Location Time: a future global time (e.g. UTC) at which a UE is to be located.

Service Type: see TS 23.271 [4].

Sidelink Positioning: see TS 23.586 [40]

Target UE: see TS 23.271 [4].

UL Positioning: positioning of a target UE in which NG-RAN obtains uplink measurements of the target UE for a 3GPP RAT.

UL+DL Positioning: positioning of a target UE using both DL Positioning and UL Positioning.

Velocity: see TS 23.271 [4].

Verinym: see TS 23.271 [4].

# * * * * Next change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LOS	Line-Of-Sight

LPI	LCS Privacy Indicator

LPHAP	Low Power and High Accuracy Positioning

LRF	Location Retrieval Function

MBSR	Mobile Base Station Relay

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

MWAB	Mobile gNB with wireless access backhauling

NLOS	Non-Line-Of-Sight

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

PRU	Positioning Reference Unit

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

SL-MO-LR	Sidelink Mobile Originating Location Request

SL-MT-LR	Sidelink Mobile Terminating Location Request

SUPL	Secure User Plane Location

# * * * * Next change * * * *

### 4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Support determination of indoor or outdoor for a location estimate.

-	Determine whether to use user plane or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF over TLS.

NOTE 2:	How the LMF uses the received UE user plane positioning capability for SUPL [49] is left to the implementation.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message and removes the PRU information after PRU Disassociation.

-	LMF keeps PRU information for PRUs which are in OFF state.

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU indication via NF profile update if PRU is stationary PRU.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

PRU ON/OFF states indicate temporarily availability of the PRU functionality of a UE at the serving LMF (e.g. PRU OFF due to other high priority tasks/energy saving at the UE, or the UE temporarily loses network coverage).

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

NOTE 3:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR or MWAB based on location and velocity of the MBSR or MWAB and the timing of the location estimations for the target UE and MBSR or MWAB.

-	For a regulatory location service, support reporting of multiple INTERMEDIATE location estimates to GMLC.

# * * * * Next change * * * *

### 4.3.12	Network Repository Function, NRF

In addition to the functions defined in TS 23.501 [18],	the NRF may perform the following functions:

-	Support to store or update PRU existence indication in TAI level in LMF profile based on request from PRU serving LMF.

-	Support LMF(s) with PRU function discovery by AMF.

-	LMF discovery by considering user plane Positioning Capability.

-	Support LMF(s) associated with PRUs discovery by other LMF(s), according to target area in the discovery request and the LMF profiles with PRU existence indication in TAI level.

-	LMF discovery by considering the support of LCS when MWAB is involved.

# * * * * Next change * * * *

### 4.4.15	NL10 Reference Point

The NL10 reference point supports LMF to send location requests to GMLC in case of 5GC-MT-LR involving MBSR and MWAB. Additionally, this reference point is used by LMF to send location result directly to GMLC in case of Deferred Location or 5GC-MT-LR multiple location or in case of support for PNI-NPN architecture.

# * * * * Next change * * * *

## 5.1	LMF Discovery and Selection

LMF selection functionality is supported by the AMF to determine an LMF for location estimation of the target UE or Ranging/Sidelink Positioning between Target UE and SL Reference UE. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, triggered location events, or Modification of User Plane Connection. The LMF selection functionality may also be supported by the GMLC and the GMLC provides the selected LMF ID to AMF.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The LMF selection/reselection may be performed at the AMF or LMF or GMLC based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF or GMLC, or by querying NRF.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE 1:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in clause 4.7.8 of TS 23.316 [21] or a HFC Node ID.

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities, including:

-	the support of Uu based positioning as defined in clause 4.3.8;

-	Ranging/Sidelink positioning as defined in clause 4.3.8 of TS 23.586 [40];

-	LMF user plane positioning capabilities (the capability to support LCS-UPP).

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

-	Support LCS when MBSR is involved.

-	Support LCS when MWAB is involved.

-	Requested UE has maintained user plane connection with certain LMFs.

When receiving a NAS message from UE, including an LMF ID together with a LPP message (refer to step 25 in clause 6.3.1 for event reporting for a deferred 5GC-MT-LR), AMF sends the LPP message to the LMF, as indicated by the LMF ID.

NOTE 2:	Description on how UE encapsulates the LMF ID in the NAS message is documented in TS 24.571 [36].

UDM may store the LMF ID in UE subscription data. During the location procedure, GMLC received the LMF ID from the UDM and provides it to AMF.

GMLC may be configured with the following parameters:

-	LMF ID and/or

-	per group ID and its correlating LMF ID.

The AMF may use locally provisioned configuration to determine LMF based on UE identify or its group information.

NOTE 3:	It is AMF implementation specific for the priority of different selection criteria from GMLC, AMF and LMF.

When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID based on the configured parameters for an LCS Client/AF. In case a group ID is provided or derived from the location request, GMLC determines the correlating LMF ID based on the provisioned group ID.

GMLC may have configuration with one or several dedicated LMF ID(s), irrelevant to any LCS client/AF. When the GMLC receives a MT location request from LCS client/AF, GMLC only determines the LMF ID within the configuration for all LCS client/AF.

NOTE 4:	When AMF can't access the LMF instance of the LMF ID, by default, AMF replies to corresponding error to GMLC. GMLC could retry or fail the request accordingly; with explicit configuration to serve as backup selection in this, AMF can also select different configured LMF instance.

# * * * * Next change * * * *

## 5.x	Location Service involving Mobile Wireless Access Backhaul

### 5.x.1	General

A MWAB may have location service capability as specified in TS 38.305 [9] and participate in the location service of a UE. As the MWAB may be moving, the location service procedures need to be enhanced as following for an accurate estimation of the UE position:

-	The UE reports the cell IDs of all the cells the UE performed DL positioning measurements on.

-	The MWAB which performed the location service procedures for the UE includes its cell ID in the reported UL positioning measurement.

-	The AMF serving the UE provides the cell ID of serving cell of the UE and indicates, if possible, that the cell-ID belongs to a MWAB to the LMF in the location request. The AMF serving the UE may also provide LMF with the additional ULI Information received from MWAB.

-	Additionally, the LMF uses the reported cell IDs to derive whether the cell ID(s) corresponds to a MWAB. There can be multiple MWAB cells in the measurement report.

-	The LMF may also decide whether the cell ID(s) corresponds to MWAB(s) based on information received in a TRP information exchange i.e., that the cell-ID belongs to a MWAB and the UE-ID (GPSI) associated with MWAB.

-	To aid the LMF to improve the accuracy of the UE location estimation, the MWAB velocity information and time of obtaining its location measurement data should be obtained by the LMF when available. The LMF uses the received location and velocity of the MWAB(s) when estimating the location of the Target UE.

The reference architectures for MWAB involved location services in different scenarios are specified in clause Annex.X of TS 23.501[18].

### 5.x.2	Obtaining location information for the MWAB

There are multiple options for an LMF to obtain the location information and velocity of the MWAB(s):

-	The LMF can derive the location and velocity of the MWAB by using NRPPa or by requesting the (H)GMLC to derive the location of the MWAB-UE using the UE-ID of the MWAB-UE. The GMLC triggers MT-LR procedure as specified in clause 6.11.1 or 6.11.2.

Editor’s note:	It is FFS if the GMLC in the PLMN serving the Target UE can trigger the (H)GMLC of the MWAB-UE to perform the location estimation of the MWAB-UE. I.e., the case when MWAB-UE H(PLMN) and the PLMN serving a MWAB-UE are different PLMNs than the PLMN serving the Target UE.

-	As the timing of the location estimations for the Target UE and MWAB(s) is important for the quality of the location estimation of the Target UE, the LMF needs to reduce the timing offset of the positioning measurements, i.e., the positioning of the Target UE and MWAB can be scheduled with using the same scheduled location time and compensate for the potential time difference of the positioning measurements, e.g. taking velocity of MWAB into account.

### 5.x.3	Privacy check for MWAB

If the positioning of the MBSR is performed for the location estimation of a Target UE (UE different from the MWAB), the privacy check in clause 5.4 is skipped for the MWAB. When the LMF requests the GMLC to derive the location of the MWAB (UE), the LMF includes a MWAB indication indicating the location of MWAB is requested to determine the location of a Target UE in the location request. The GMLC obtains the subscription information of the MWAB (UE) from the UDM. Based on the indication, and/or MWAB's subscription information, the GMLC skips the privacy check.

If the positioning of the MWAB is performed for the location estimation of the MWAB itself when it acts as a normal UE which is not authorized to operate as MWAB based on the subscription information, the UE privacy check procedure in clause 5.4 is performed.

# * * * * End of changes * * * *