SA WG2 Meeting #165	S2-2409648

14 - 18 October, 2024, Hyderabad, India

Source:	China Mobile

Title:	Discussion on the support of UL, DL and RT N6 delay measurement

Document for:	Discussion

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This paper discusses the support of UL, DL and RT N6 delay measurement.

1.	Discussion

According to general agreement, AF may provide SMF with N6 delay measurement assistance information via EDI, including measurement protocol(s) that supported by the application side. However, there are three kind of N6 delay i.e. UL,DL, and RT.

## 1.1	The requirement of UL, DL and RT N6 delay measurement

5GC may perform L-PSA UPF/EAS selection based on N6 delay values, in order to improve the E2E user experience. However, different services have different requirement on N6 delay. For example, for UAV remote controlling case, UL N6 delay is crucial to guarantee the real-time reaction of UAV, while DL or RT N6 delay is not critical; for live streaming case, live video stream is downlink packet, where DL N6 delay has much impact on user experience, and UL or RT N6 delay are usually not required; for real-time gaming case, both player controlling (uplink) and graph (downlink) have extreme requirements on low-latency, therefore certain RT N6 delay is required.

Observation 1:	The requirement of UL, DL and RT N6 delay measurement is obvious and should be supported in 3GPP specification.

## 1.2	IETF protocols support UL, DL and RT N6 delay measurement

According to conclusion of TR 23.700-49, IETF protocols may be utilized for N6 delay measurement, for example, ICMP, OWAMP, TWAMP, etc. These protocols may support different types (i.e., UL, DL or RT) of N6 delay measurement.

According to IETF RFC 792, Timestamp or Timestamp Reply Message of ICMP contains Originate Timestamp, Receive Timestamp and Transmit Timestamp:

The Originate Timestamp is the time the sender last touched the message before sending it, the Receive Timestamp is the time the echoer first touched it on receipt, and the Transmit Timestamp is the time the echoer last touched the message on sending it.

If the time is not available in miliseconds or cannot be provided with respect to midnight UT then any time can be inserted in a timestamp provided the high order bit of the timestamp is also set to indicate this non-standard value.

If ICMP supported by the application side, the PSA UPF may send Timestamp Message to the measurement endpoint, and either: 1) calculate UL N6 delay based on Originate Timestamp and Receive Timestamp, 2) calculate DL N6 delay based on Transmit Timestamp and the time the PSA UPF first touch the Timestamp Reply Message, or 3) calculate RT N6 delay based on Originate Timestamp, Receive Timestamp, Transmit Timestamp and the time the PSA UPF first touch the Timestamp Reply Message. ICMP may be used for measurement of UL, DL or RT N6 delay.

According to IETF RFC 4656, for OWAMP-Test, the sender sends packets to the receiver, containing Timestamp:

Regardless of any scheduling delays, each packet that is actually sent MUST have the best possible approximation of its real time of departure as its timestamp (in the packet).

The OWAMP-Test packet layout is the same in authenticated and encrypted modes. The encryption and authentication operations are, however, different. The difference is that in encrypted mode both the sequence number and the timestamp are protected to provide maximum data confidentiality and integrity protection, whereas in authenticated mode the sequence number is protected while the timestamp is sent in clear text. Sending the timestamp in clear text in authenticated mode allows one to reduce the time between when a timestamp is obtained by a sender and when the packet is shipped out. In encrypted mode, the sender has to fetch the timestamp, encrypt it, and send it; in authenticated mode, the middle step is removed, potentially improving accuracy (the sequence number can be encrypted and authenticated before the timestamp is fetched).

If OWAMP supported by the application side, the PSA UPF may send packet of OWAMP-Test to the measurement endpoint. The measurement endpoint may calculate UL N6 delay based on Timestamp and the time of the first touch the packet. Similarly, the PSA UPF may calculate DL N6 delay based on Timestamp and the time it first touch the OWAMP-Test packet sent by the measurement endpoint. OWAMP may be used for measurement of UL or DL N6 delay.

According to IETF RFC 5357, for TWAMP-Test, the Session-Reflector sends message to the Session-Sender, containing Timestamp, Receive Timestamp and Sender Timestamp:

Timestamp and Error Estimate are the Session-Reflector’s transmit timestamp and error estimate for the reflected test packet, respectively. The format of all timestamp and error estimate fields follow the definition and formats defined by OWAMP, Section 4.1.2 in [RFC4656].

Sender Timestamp and Sender Error Estimate are exact copies of the timestamp and error estimate from the Session-Sender test packet that corresponds to this test packet.

Sender TTL is 255 when transmitted by the Session-Sender. Sender TTL is set to the Time To Live (or Hop Count) value of the received packet from the IP packet header when transmitted by the Session-Reflector.

Receive Timestamp is the time the test packet was received by the reflector. The difference between Timestamp and Receive Timestamp is the amount of time the packet was in transition in the Session-Reflector. The Error Estimate associated with the Timestamp field also applies to the Receive Timestamp.

If TWAMP supported by the application side, the PSA UPF may transmit test message to the measurement endpoint. When receive the response message form reflector (i.e. EAS), the PSA UPF may either: 1) calculate UL N6 delay based on Sender Timestamp and Receive Timestamp, 2) calculate DL N6 delay based on Timestamp and the time the PSA UPF first touch the response test message, or 3) calculate RT N6 delay based on Sender Timestamp, Receive Timestamp, Timestamp and the time the PSA UPF first touch the response test message. TWAMP may be used for measurement of UL, DL or RT N6 delay.

Based on discussion above, current IETF protocols (e.g. ICMP, TWAMP) may support UL, DL and RT N6 delay measurement. Therefore, when such these protocols is used, the AF needs to provide the 5GC with the type of N6 delay measurement (i.e. UL, DL, or RT), and the SMF needs to indicate the PSA UPF with the type of N6 delay measurement.

Observation 2: UL, DL and RT N6 delay measurement between PSA UPF and EAS can be supported by current IETF protocols.

2.	Proposal

Based on the discussion above, it is proposed to discuss and agree the following proposals:

Proposal 1:	The UL, DL and RT N6 delay measurement should be supported in 3GPP specification, with using current IETF protocols.