3GPP TSG-SA WG2 Meeting #165	S2-2409682

# Hyderabad, India, Oct 14 – Oct 18, 2024								            (revision of S2-24xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5637 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 23.501 – TR 23.700-66 KI#3 Extending the NF Profile for Energy Efficiency and Consumption feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rakuten Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the conclusions in the TR 23.700-66 v19.0.0, clause 8.3 for KI#3; and - approved WID (SP-241388) from SA#105 this contribution proposes the below new feature addition to TS 23.501 “NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | New EnergySys feature addition for KI#3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#3 conclusion is not implemented in TS 23.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (New Clause), 6.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

## 5.x	Enablers for Energy Efficiency and Consumption

### 5.x.0	General

Editor’s Note: This clause describes 5G System features for Energy Efficiency and Consumption.

#### 5.x.1	NF profile based on energy related information

The NF profile is extended for the scope of selection based on energy information with the information described in Table 5.x.6.2-1. The information within NF profile specified in Table 5.x.6.2-1 is expected not to change frequently, e.g. constantly reflecting changes in energy consumption.

Table 5.x.1-1: Energy Information related information included in the NF profiles

| Information element name | Description |
| --- | --- |
| Renewable Energy Ratio | Indicating percentage of Energy Consumption using renewable sources in the total Energy Consumption using all sources (NOTE 4). |
| Carbon emission factor | Indicating amount of carbon emissions relative to an amount of resource consumption (NOTE 1, NOTE 2). |
| Normalized Energy Consumption | Indicating amount of energy consumption relative to an amount of resource consumption (NOTE 1). |
| Energy States | Energy state as specified in TS 28.310 [7], e.g. notEnergySaving state, energySaving state. Or other states if defined. |
| -Schedule of energy states | The schedule of the energy states, e.g. the time of entering the states regarding energy saving. (NOTE 5) |
| Energy inefficiency | Indicate if the NF face problems of excessive energy consumption as characterized by MDA in clause 8.4.4 of TS 28.104 [16]. |
| NF profile attributes per energy states. | This enables NFs to register in advance different profile attributes in their NF profiles which apply with e.g. different values or apply or do not apply depending on the energy state. |
| Energy saving state transition execution indication | Indicating whether the NF is subject to energy related execution (NOTE 3). (e.g., a UPF that is scheduled to be taken out of service to save energy in energy saving state). |
| Energy consumption | Indicating amount of energy consumption by the NF over time window. |
| Energy Efficiency Level performance metric | The criteria that is used to calculate the Energy Efficiency Level of the NF. The criteria includes e.g. forwarded bitrate sum for UPF |
| Energy Efficiency Level | Indicating energy efficiency that is derived by the relative ratio of Energy Efficiency Level performance metric (e.g. forwarded bitrate sum for UPF, etc.) over Energy consumption for the NF in the execution environment. |
| NF Energy consumption restriction level | The level that restricts the energy consumption allowed for the NF instance uses over time window. The restrict level may be a certain amount of energy consumption, or a range of amount of energy consumption. |
| NOTE 1: Resource consumption is defined in TS 28.554 [6] for a VNF. The indication refers to a relative value. The granularity of such indication and how to calculate is to be defined by SA WG5, e.g. it could be of three levels: Low, Medium, High. NOTE 2: Carbon emission factor can also indicate relative carbon efficiency level of the NF, e.g. an indication with Low, Medium, High, or a level value with 1, 2, 3, 4, 5. KPI can be further adjusted/defined based on related study in SA WG5. NOTE 3: Energy related execution refers to switch the NF to energySaving state for energy saving. NOTE 4: The Renewable Energy Ratio can also denote the type of energy type of the NF, e.g. renewable energy, non-renewable energy. NOTE 5: The NF service consumer may select at NF based on the time-based received information from NRF. |  |

Editor’s Note:    Coordination with SA WG5 is needed for the parameter which are expected to be configured and/or provisioned by OAM.

NOTE: In the current release of the specification, only the energy-related information of user plane communication (not control plane signalling) is supported.

# * * * * Next change * * *

#### 6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in clause 5.2.7.2.2 of TS 23.502 [3]. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information or serving scope for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, Home Network Public Key identifier, for UDM and AUSF.

-	For UDM, AUSF and NSSAAF in the case of access to an SNPN using credentials owned by a Credentials Holder with AAA Server, identification of Credentials Holder (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used for access to an SNPN using credentials owned by a Credentials Holder, identification of Credentials Holder (i.e. the realm if Network Specific Identifier based SUPI is used or the MCC and MNC if IMSI based SUPI is used); see clause 5.30.2.1.

-	For AUSF and NSSAAF in the case of SNPN Onboarding using a DCS with AAA server, identification of DCS (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used as DCS in the case of SNPN Onboarding, identification of DCS (i.e. the realm if Network Specific Identifier based SUPI, or the MCC and MNC if IMSI based SUPI).

-	One or more GUAMI(s), in the case of AMF.

-	For the UPF, see clause 5.2.7.2.2 of TS 23.502 [3].

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	For NWDAF, the following information are supported:

-	Analytics ID(s) (possibly per service).

-	NWDAF Serving Area information (refer to clause 6.3.13).

-	Supported Analytics Delay per Analytics ID (if available).

-	NF types of the NF data sources, NF Set IDs of the NF data sources, if available.

-	Analytics aggregation capability (if available).

-	Analytics metadata provisioning capability (if available).

-	ML model Filter information parameters include S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s).

-	ML Model Interoperability indicator (if available) per Analytics ID(s).

-	FL capability information per analytics ID including FL capability type (i.e. FL server and/or FL client, if available).

-	Time interval supporting FL (if available).

-	Accuracy checking capability for ML model accuracy monitoring or Analytics Accuracy Monitoring (if available).

-	Roaming exchange capability (if available).

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay i.e. the Analytics report can be generated with a time (including data collection delay and inference delay) in less than or equal to the Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Event Exposure service supported event ID(s) by UPF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs or Range(s) of GPSIs or a BSF Group ID, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, if available, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

-	For SNPN, capability to support SNPN Onboarding in the case of AMF and capability to support User Plane Remote Provisioning in the case of SMF.

-	IP address range, DNAI for UPF.

-	Operator configurable UPF capability for UPF features(s) as described in clause 5.8.2.21.

-	Supported DNS security protocols, in the case of EASDF.

-	Additional V2X related NF profile parameters are defined in TS 23.287 [121].

-	Additional ProSe related NF profile parameters are defined in TS 23.304 [128].

-	Additional MBS related NF profile parameters are defined in TS 23.247 [129].

-	Additional UAS related NF profile parameters are defined in TS 23.256 [136].

-	Additional Ranging based services and Sidelink Positioning related NF profile parameters are defined in TS 23.586 [180].

-	For additional information in PCF profile, see clause 5.2.7.2.2 of TS 23.502 [3].

-	Additional Energy Efficiency and Consumption related NF profile parameters are defined in clause 5.x

# * * * * End of changes * * *