3GPP SA WG2 Meeting #165	S2-2409715

# Hyderabad, India, 14 – 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1204 | rev | x | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General VFL training procedure when NWDAF and AF as VFL Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the previous meeting, the CR for General VFL training procedure (S2-2409416) has been discussed but noted due to time limitation, however, there are many ENs should be solved, and the procedure of VFL training initialized by AF is missing. This new CR proposes an update to the VFL training procedure to address some of the ENs on top of last version of the previous CR, and it also includes a description of VFL training initiated by the AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - for the case that NWDAF as VFL server, proposes an VFL training procedure based on the discussion last meeting; - for the case that AF as VFL server, adds a new procedure for VFL tranining. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature of VFL training process is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (All New Text) * * *

### 6.2X.Y.Z General training procedure for Vertical Federated Learning

Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server.

Editor’s Note: The details on whether and how to support that VFL clients provide intermediate results to other VFL clients is FFS.

Editor’s Note: It is FFS whether model storage after each training round is required.

Editor’s Note: The services for VFL training is FFS.

The figure 6.2X.Y.Z-1 below shows general training procedure for Vertical Federated Learning.

Figure 6.2X.Y.Z-1: General training procedure for VFL

This below sub-procedure aims for Vertical Federated Learning when NWDAF as the server.

1.	The NWDAF acting as VFL server selects the VFL client(s) that participate in VFL as described in the clause X, the VFL client(s) could be NWDAF(s) and/or AF(s).

NOTE: NWDAF as VFL Server determines to perform VFL training based on the operator’s policy.

2.	To start VFL training, NWDAF as VFL server sends a VFL training request to the selected VFL client(s) with the analytics ID (i.e. VFL training of models for analytics ID), a VFL model identifier and optional the analytic filter information (e.g., AOI). The VFL model identifier is allocated by the VFL server corresponding to the Analytics ID.

If a VFL client is an untrusted AF, NWDAF as VFL server sends the VFL training request to the AF via NEF with the analytics ID (i.e., internal analytics ID), the allocated VFL model identifier, AF ID and optional the analytic filter information (e.g., AOI). The NEF maps the internal Analytics ID to external Analytics ID, and may translate the analytic filter information, e.g. TAIs into geographical area.

3.	[Optional] Each NWDAF or AF as VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each NWDAF or AF as VFL client further trains the local ML model based on its own data, generates intermediate training result based on local ML model, and then reports VFL model identifier, intermediate training result of the local ML model to the VFL server. The intermediate training result could be per sample granularity, so optional with sample ID.

If a VFL client is an untrusted AF, the AF as VFL client sends the VFL training response to the NWDAF as VFL server via NEF. After receiving VFL training response from the AF, the NEF sends the VFL training response to the NWDAF as VFL server to provide the VFL model identifier, the intermediate training result and optional sample ID. The NEF may map the external UE ID into internal UE ID.

5.	The NWDAF as VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the backward model training information (e.g. gradient information or loss information) based on the intermediate training results and label data. The backward model training information is used for updating the local ML model(s) of VFL client(s). Different backward model training information may be computed for different VFL clients, respectively.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required, the step 7 and 8 continues. If yes, it determines the VFL Training is completed, in this case, the VFL Server terminates the current VFL training process. The VFL server sends VFL training termination message to VFL Client(s) if it decides to terminate the VFL training process.

The VFL training termination decision may be also made for the consumer request,

i	f the VFL server received a request from consumer or decide to stop the VFL Training process,

then the VFL Server terminates the current VFL training process.

7.	The NWDAF as the VFL server sends VFL training request to the NWDAF(s) or AF(s) as VFL client(s) with the analytics ID, the allocated VFL model identifier, the backward model training information and optional sample ID.

If a VFL client is an untrusted AF, NWDAF as VFL server sends the analytics ID, the allocated VFL model identifier, the backward model training information and optional sample ID to the AF(s) via NEF.

8.	Each NWDAF or AF as VFL client computes gradient of its local ML model and updates its local ML model based on backward model training information distributed by the VFL server at step 7. The NWDAF acting as VFL Server and each NWDAF or AF acting as VFL Client, stores the latest information about their locally trained ML Models.

The steps 4-8 should be repeated until the training termination condition is reached.

This below sub-procedure aims for Vertical Federated Learning when AF as the VFL server.

1.	The AF acting as VFL server selects the VFL clients that participate in VFL as described in the clause X, the VFL client(s) could be NWDAF(s).

2.	To start VFL training, AF as VFL server sends a request to the selected VFL client(s) with the analytics ID (i.e. VFL training of models for analytics ID), a VFL model identifier and optional the analytic filter information (e.g., AOI). The VFL model identifier is allocated by the VFL server corresponding to the Analytics ID.

If the VFL server is an untrusted AF, the untrusted AF as VFL server sends the VFL training request to the NEF with the temporary NWDAF ID(s) obtained after VFL client discovery procedure, the analytics ID (i.e. external analytics ID), a VFL model identifier and optional the analytic filter information (e.g. geographical area). The NEF forwards the VFL training request to each NWDAF as VFL client based on the temporary NWDAF ID(s) including the analytics ID (i.e. internal analytics ID), the VFL model identifier and optional the analytic filter information (e.g. AOI).

3.	[Optional] Each NWDAF as VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each NWDAF as VFL client further trains the local ML model based on its own data and generates intermediate training result based on local ML model. Each VFL client reports VFL model identifier, intermediate training result of the local ML model to the VFL server. The intermediate training result could be per sample granularity, so optional with sample ID.

If the VFL Server is an untrusted AF, each NWDAF as VFL client sends the VFL training response to the AF as VFL server via NEF. The VFL training response from each VFL client includes VFL model identifier, intermediate training result of the local ML model and optional sample ID. After receiving the VFL training response from the NWDAF, the NEF sends the VFL training response to the AF. The NEF may map the internal UD ID into external UE ID.

5.	The AF as VFL server may collect the local data and generate its own local intermediate training result. The AF acting as VFL Server computes the backward model training information (e.g. gradient information or loss information) based on the intermediate training results and label data. The backward model training information is used for updating the local ML model(s) of VFL client(s). Different backward model training information may be computed for different VFL clients, respectively.

6.	[Optional] The AF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the AF acting as a VFL Server determines another round of VFL training is required, the step 7 and 8 continues. If yes, it determines the VFL Training is completed, in this case, the VFL Server terminates the current VFL training process. The VFL server sends VFL training termination message to VFL client(s) if it decides to terminate the VFL training process.

The VFL training termination decision may be also made for the consumer request, if the VFL server received a request from consumer to stop the VFL Training process, then the VFL Server terminates the current VFL training process.

7.	The AF as the VFL server sends VFL training request to the NWDAF(s) as VFL client(s) with the analytics ID, the allocated VFL model identifier, the backward model training information and optional sample ID.

If the VFL server is an untrusted AF, the untrusted AF as VFL server sends the analytics ID, the VFL model identifier, the backward local ML model training information to the NEF with the temporary NWDAF ID(s).

8.	Each NWDF as VFL client computes gradient of its local model and updates its local ML model based on backward model training information distributed by the VFL server at step 7. The AF acting as VFL Server and each NWDAF acting as VFL Client, stores the latest information about their locally trained ML Models.

The steps 4-8 should be repeated until the training termination condition is reached.

* * * End of Changes * * *