SA WG2 Meeting #165	S2-2409722

# Hyderabad, India, 14 – 18 October, 2024                        (revision of S2-240xxxx)

Source: 	CATT

Title: 	Conclusions for KI#3

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper made some conclusion proposals for KI#3.

1	Discussion

As documented in TR 23.700-13, the Key Issue #3 is to address support of AIoT Services as followings.

5.3	Key Issue #3: Support of Ambient IoT Services

5.3.1	Description

This Key Issue pertains to the AIoT services. Considering that AIoT Devices are a new type of devices with reduced capabilities, the following need to be supported:

-	Inventory.

-	Command.

NOTE 1:	Further detailing of the inventory and commands will be addressed by solutions.

The key issue will study the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE 2:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

Based on the solutions documented in TR 23.700-13 and RAN2 agreements in the incoming LS (R2-2406150), it is proposed to conclude to support the following AIoT Services procedures in Rel-19:

- Inventory

- Command

Based on the existing solutions of TR 23.700-13, the high level procedure for Inventory/Command can be summarized and illustrated in Figure 1.

Figure 1: High level procedure for Inventory/Command Service

1. A RAN Reader or a UE Reader needs to be authorized before it performs Inventory/Command Service. For a RAN Reader, RAN Reader reports AIoT support capability to AMF and is authorized by AMF during NG setup procedure. For a UE Reader, UE Readers reports AIoT support capability to AMF and is authorized by AMF during Registration procedure.

2. AF sends AIoT Service (Inventory or Command) Request message to NEF, and the Request message includes Device filter info and Area info, etc.

3. The NEF authorizes the AF request whether the AF is authorized to perform the Inventory/Command Service.

4. The NEF discovers and selects AIoTF based on the Area info received from the AF.

5. The NEF sends AIoT Service Request message to the AIoTF, and the Request message includes Device filter info and Area info, etc.

6. The AIoTF discovers and selects RAN Reader/UE Reader based on the Area info received from the NEF.

7. The AIoTF sends AIoT Service Request message to the RAN Reader/UE Reader, and the Request message includes Device filter info, etc.

8. The RAN Reader/UE Reader sends Initial Trigger Message (Device filter info) to AIoT Device.

NOTE 1:	The step 8 is to be defined by RAN2.

9. The AIoT Device compares the received Device filter info with its Device ID.

10. If the AIoT Device matches the Device filter info, the AIoT Device performs Random Access procedure.

NOTE 2:	The step 10 is to be defined by RAN2.

11. The AIoT Device sends AIoT Service Response NAS message to the AIoTF via the RAN Reader/UE Reader and AMF. For Inventory service operation, the Response message includes Device ID. For Command service operation, the Response message includes Device ID and Command Result info.

12. The authentication and authorization procedure is performed for AIoT Device.

NOTE 3:	The step 12 is to be defined by SA3.

13-14. The AIoTF sends AIoT Service Response to the AF via the NEF.

2	Proposal

It is proposed to update TR 23.700-13 as followings.

# * * * * First change * * * *

8.1	Conclusions for Key Issue #3: Support of Ambient IoT Services

For Key Issue #3, the following baseline AIoT Service procedures are concluded for normative work:

1. A RAN Reader or a UE Reader needs to be authorized before it performs Inventory/Command Service. For a RAN Reader, RAN Reader reports AIoT support capability to AMF and is authorized by AMF during NG setup procedure. For a UE Reader, UE Readers reports AIoT support capability to AMF and is authorized by AMF during Registration procedure.

2. AF sends AIoT Service (Inventory or Command) Request message to NEF, and the Request message includes Device filter info, Device operation info and Area info, etc.

3. The NEF authorizes the AF request whether the AF is authorized to perform the Inventory/Command Service.

4. The NEF discovers and selects AIoTF based on the Area info received from the AF.

5. The NEF sends AIoT Service Request message to the AIoTF, and the Request message includes Device filter info, Device operation info and Area info, etc.

6. The AIoTF discovers and selects RAN Reader/UE Reader based on the Area info received from the NEF.

7. The AIoTF sends AIoT Service Request message to the RAN Reader/UE Reader, and the Request message includes Device filter info and Device operation info, etc.

8. The RAN Reader/UE Reader sends Initial Trigger Message (Device filter info, Device operation info) to AIoT Device.

NOTE 1:	The step 8 is to be defined by RAN2.

9. The AIoT Device compares the received Device filter info with its Device ID and takes Device operation info into account.

10. If the AIoT Device matches the Device filter info, the AIoT Device may perform Random Access procedure.

NOTE 2:	The step 10 is to be defined by RAN2.

11. The AIoT Device sends AIoT Service Response NAS message to the AIoTF via the RAN Reader/UE Reader and AMF. For Inventory service operation, the Response message includes Device ID. For Command service operation, the Response message includes Device ID and Command Result info.

12. The authentication and authorization procedure is performed for AIoT Device.

NOTE 3:	The step 12 is to be defined by SA3.

13-14. The AIoTF sends AIoT Service Response to AF via NEF.

# * * * * End of Changes * * * *