3GPP TSG SA WG2 #165	            S2-2409748

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1464 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS-AGW relocation in UE-SAT-UE communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#3 conclusion in TR 23.700-29, IMS-AGW relocation procedure should be defined in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the P-CSCF is notified about the serving satellite changes and how the IMS-AGW relocation procedure can be proceeded are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-satellite-UE communication over satellite constellation with IMS-AGW onboard architecture cannot work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

Annex XY (normative):
Support of UE-satellite-UE communications

XY.1.1	IMS-AGW relocation

During UE-satellite-UE communications, the IMS-AGW relocation is performed by the P-CSCF when the gNB, UPF, and IMS-AGW is onboard, and the satellite which is serving one of the UE changes.

During IMS session setup procedure, the P-CSCF may have subscribed to PCF to be notified about the change of serving satellite for the originating UE. The P-CSCF, as an AF, subscribes the event with the Policy Control Trigger Request, when the IMS session is for UE-satellite-UE communication. Upon event notification from the PCF, the P-CSCF determines with the serving satellite ID received from the PCF whether the IMS-AGW is onboard in the satellite, and whether the IMS-AGW relocation is needed. If the P-CSCF decides to relocate the IMS-AGW in the new satellite corresponding to the serving satellite ID received from the PCF, or the IMS-AGW on the ground. The P-CSCF sends a SIP message to the originating UE with the re-INVITE request indication. The originating UE sends the re-INVITE to the P-CSCF for the on-going IMS sessions, then the P-CSCF initiates the IMS-AGW relocation procedure and transfers the re-INVITE to the termingating network and the terminating UE, and the rest of the procedures are completed.

Editor's note:	The Policy Control Trigger to be notified about the change of serving satellite for the originating UE will be defined in TS 23.503 [95].

If the UE detected the change of serving satellite for the IMS sessions, the UE may send re-INVITE requests to the IMS network. In this case, the P-CSCF requests to PCF and checks the serving satellite ID. The P-CSCF determines the IMS-AGW relocation and corresponding procedures are performed as described in the second paragraph of this clause.

# * * * * End of changes * * * *