3GPP TSG-WG SA2 Meeting #165	S2-2409789

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-2409416)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile], [CATT], [ZTE], [OPPO], [China Telecom], [ETRI], Huawei, [Nokia], [Futurewei], [Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| s |  |  |  |  |  |  |  |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to specify the general training procedure for vertical federated learning between AF(s) and NWDAF(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added to specify the general training procedure for vertical federated learning between AF and NWDAFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 4: Following ENs are solved: Editor’s Note: Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. Editor’s Note: It’s FFS to determine the VFL preparation procedure. Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server. - Solution: Define the two types of procedures: (a) VFL Training Preparation procedure; and (b) VFL Execution Procedure. Additionally we propose to split the VFL Training Procedure between AF acting as VFL server and NWDAF acting as VFL in order to make the clear differentiation on the role of the Consumer of a ML Model (i.e., addressing EN #3) in the procedure. The interactions among the VFL Server and VFL Client during the steps of calculating intermediary results, checking the VFL process convergence, distributing information for the new round, are the same no matter if the VFL Server is an NWDAF or an AF. However, this is not the case for the preparation phase, were the trigger for the VFL procedures are different if AF or NWDAF is acting as VFL Server. Therefore, different procedures are essential in the VFL training preparation phase, but the same procedure can be used in the actual VFL training execution, differentiating only the services being invoke, but not the flow itself. Editor’s Note: How the VFL server and client register to the network, how to discover VFL server or VFL client is FFS. - Solution: Remove ENs and include a note pointing to the Clause in Ts 23.288 related to the VFL discovery procedure agreed during SA2#164. This issue has been solved in the agreed paper S2-2409419 during SA2#164. Any changes related to the VFL discovery procedure should be updated in the respective agreed clause. Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS. - Solution: De-prioritize in R19 the support for dynamic reconfiguration of either VFL Clients or changes in the sample alignment. The straight forward solution is to consider that any change to VFL Clients or sample alignment information should be handled as a new VFL process, therefore a new VFL Correlation ID should be assigned. Editor’s Note: A possible need for a process of sample exchange between VFL server and VFL client is FFS. The process may be determined after agreement on sample and/or feature alignment. - Solved when including the VFL preparation procedure, because the VFL Server and VFL Clients need the know what is the sample information that is related to the VFL training to be started. Additionally, Feature is vendor specific and should not be transferred via signalling Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment. - Solved by removing EN, because Feature is vendor specific and should not be transferred via signalling Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training round or there is a session concept. - Solved by two principles: for an assigned VFL correlation ID there is a unique sample information, and any change in VFL Clients leads to a new VFL correlation ID. So, no need to have a concept of session, because if the NWDAF assisting the UE in the agreed sample information for a correlation ID changes… this means that a complete new training needs to be triggered. Editor’s Note: It is FFS whether and how to use ML Model ID or VFL Model Correlation ID or VFL correlation ID in the VFL training process. Solved by proposal to use both the ML Model ID to identify a ML model that is being trained by NWDAF or AF; and the VFL Correlation ID to signal that the ML Model ID is associated with a VFL training process (i.e., a specific set of VFL participants and specific set of sample alignment). The reason to keep both is not to lose compatibility on R19 on the procedures involving ML Models. Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS. - Solution: Include the new parameter defined as VFL Group ID that identifies the NWDAFs and /or AFs that can collaborate in the same VFL task. In this case the NWDAF(s) and/or AF(s) share the same VFL Group ID, which can be pre-configured by the OAM.the participants of a VFL process. Editor´s Note: Parameters to be provided in the request need to be added - Solved because the proposal details the service and parameters. Editor’s Note: The details on whether and how to support that VFL clients provide intermediate results to other VFL clients is FFS. - Solution: Do not allow the VFL Clients to send intermediary results for other VFL clients because there is no clear scenario where this is required in the conclusions of TR 23.700-84 Editor’s Note: Whether weight of feature is computed by VFL server is FFS. - The EN is reworded to change from Weight of feature to weight of VFL Client. Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS. Editor’s Note: The flow should be updated to show the message exchange for VFL training termination. - VFL training termination flag is necessary because this can clearly indicate to the VFL Clients that training is finished and they each VFL participant can to the proper storing of the resulting training models. |  |  |  |  |  |  |  |  |

# * * * * First change (ALL NEW TEXT)  * * * *

6.2X	Vertical Federated Learning at Training

### 6.2X.1	General

A VFL joint model training is applicable when the training entities are not able to share their ML models (e.g., due to privacy between AF and NWDAF, or due to multi-vendor NWDAFs) nor the input data for training the ML models can be shared (e.g., due to privacy).

The VFL Training process is divided in two procedures: VFL Training Preparation and VFL Training Execution. During VFL Training Preparation the VFL Server determine the VFL clients and perform the sample alignment for the execution of the VFL Training among the selected VFL clients all supporting the same sample (e.g., UE IDs). The VFL Training execution is performed to allow each training entities participating of the VFL Training process to train their local ML model associated with the same VFL Correlation ID.

### 6.2X.2	Procedure for VFL Training Preparation with NWDAF as VFL Server

The procedure in Figure 6.2X.2.1 considers the case of VFL Training Preparation for NWDAF acting as VFL Server and having either NWDAFs acting as VFL Clients or AFs acting as VFL Clients. The procedure also considers the case where AFs are trusted by the operator and when the AFs are 3rd Party and therefore interactions between NWDAF acting as VFL server and AF acting as VFL Clients are executed via NEF.

Figure 6.2X.2.1-1: VFL Model Training Preparation with NWDAF acting as VFL Server

0.	The NWDAF acting as VFL Server based on the Nnwdaf_MLModelProvisioning_Subscribe service request received from a Consumer and/or operator’s policies decides to start a VFL Training Process.

1.	The NWDAF acting as VFL Server discovers the NFs candidates to participate in the VFL Training according to the definitions in Clause 6.2H.2.1.

2.	The NWDAF acting as VFL server determines a VFL Correlation ID to be used for the VFL training process to be executed.

Based on the VFL related configurations available at NWDAF acting as VFL Server and the information about the discovered participants, the NWDAF acting as VFL Server determines the VFL Training Preparation Information required for the VFL joint model training to be executed. The VFL Training Preparation Information comprises:

-	Analytics ID;

-	VFL Correlation ID;

-	VFL Group ID;

-	VFL capability type (VFL Client) – indicating the VFL Capability is required to be performed

-	Alignment Information:

-	Sample ID types (e.g., SUPI, GPSI, AF specific ID);

-	Sample space, which includes the list of Sample IDs to be considered for VFL Training.

-	Sample provision indication: indicate the VFL Client to provide Sample IDs corresponding to the dataset used for VFL model training.

3.	The NWDAF acting as VFL Server provides the VFL Training Preparation request for the discovered NFs capable to act as VFL Clients.

When the discovered NF acting as VFL Client is an NWDAF, the NWDAF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service with the input parameters: ML training methodology set with “VFL”, used by the VFL Client to distinguish the type of collaborative training that is being requested (e.g., “HFL” as per Clause 62C or “VFL”); ML Preparation Flag (set to VFL preparation start), to check if the VFL Client NWDAF(s) can meet the VFL Training requirements (e.g., VFL Training Preparation Information, Data Availability requirement, FL Availability time requirement (time span needed for the FL process), etc.). Data Availability requirement includes the dataset statistical properties, the time window of the data samples and optionally the minimum number of data samples.

When the discovered NF acting as VFL Client is a trusted AF, the NWDAF acting as VFL Server invokes the Naf_VFLTraining_Subscribe request or Naf_VFLTraining_Request service with the input parameters: ML training methodology set with “VFL”; ML Preparation Flag (set to VFL preparation start), to check if the VFL Client AF(s) can meet the VFL Training requirements (e.g., VFL Training Preparation Information, Data Availability requirement, FL Availability time requirement (time span needed for the FL process), etc.). Data Availability requirement includes the dataset statistical properties, the time window of the data samples and optionally the minimum number of data samples.

4.	Each VFL Client, NWDAF and/or AF acting as VFL Client, verifies whether it can join the VFL Training Process based on the support for the requested parameters received VFL Training Preparation Information and/or based on local policies. The Alignment Information received in the VFL Training Preparation Information is used by each NWDAF or AF acting as VFL Client to determine the Sample IDs it can support from the received Sample Space.

5.	The NWDAF acting as VFL Server receives from the discovered VFL Clients the VFL Training Preparation response.

When the NF acting as VFL Client is an NWDAF, the NWDAF acting as VFL Server receives the Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response with a confirmation that the NWDAF acting as VFL Client accepts the participation in the VFL Training Process and the Supported Alignment Information which includes the supported Sample IDs. Alternatively, the response includes or a rejection and the reason if it cannot join the VFL Training (i.e., VFL request not supported). If the NWDAF acting as VFL Client does not have the initial VFL model, it includes in the response the initial model provision indication.

When the NF acting as VFL Client is a trusted AF, the NWDAF acting as VFL Server receives the Naf_VFLTraining_Subscribe response or Naf_VFLTraining_Request response with a confirmation that the AF acting as VFL Client accepts the participation in the VFL Training Process and the Supported Alignment Information which includes the supported Sample IDs. Alternatively, the response includes or a rejection and the reason if it cannot join the VFL Training (i.e., VFL request not supported). If the AF acting as VFL Client does not have the initial VFL model, it includes in the response the initial model provision indication.

6.	The NWDAF Acting as VFL Server determines the final list of VFL Clients for the VFL Training identified by the VFL correlation ID based on the information received in Step 5 from each of the VFL Clients (NWDAF or AF), and optionally based on information acquired during discovery and/or VFL related configuration (e.g., parametrizations for a VFL Group).  The NWDAF acting as VFL Server also determines the final sample alignment information, i.e., the intersection Sample IDs of the VFL Clients.

7. 	The NWDAF acting as VFL server provides to each VFL Client determined in Step 6 the VFL Training Preparation Conclusion information, comprising the VFL Preparation Conclusion Flag, VFL correlation ID, and final sample alignment information.

When the determined NF acting as VFL Client is an NWDAF, the NWDAF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service including the VFL Training Preparation Conclusion information.

When the determined NF acting as VFL Client is a trusted AF, the NWDAF acting as VFL Server invokes the Naf_VFLTraining_Subscribe request or Naf_VFLTraining_Request service including the VFL Training Preparation Conclusion information.

NOTE:	Each VFL Client (NWDAF or AF) receiving the VFL Training Preparation Conclusion information, stores the mapping of the VFL Correlation ID to the local ML Model to be trained and the final sample alignment information to be used for the VFL Training Execution.

8. When the NWDAF acting as VFL Server determine that the VFL Training Execution procedure may start, it executes the VFL Training as defined in Clause 6.2X.3.

9.  When VFL Training has been triggered the request in Step 1 by a request for model provisioning, the NWDAF acting as VFL server provides the Nnwdaf_ Nnwdaf_MLModelProvisioning_Notify message to the Consumer including the VFL Correlation ID in addition to the ML Model ID in order to identify that the trained ML Model is related to a VFL process.

### 6.2X.3	Procedure for VFL Training Preparation with AF as VFL Server

The procedure in Figure 6.2X.2.2.1 considers the case of VFL Training Preparation for AF acting as VFL Server. The procedure considers the case where the AF acting as VFL Server is trusted by the operator and when the AF acting as VFL Server is a 3rd Party Entity and therefore interactions between AF acting as VFL server and NWDAF acting as VFL Client are executed via NEF.

Figure 6.2X.2.2.1: VFL Model Training Preparation with AF acting as VFL Server

0.	The AF acting as VFL Server determines to perform a VFL Training Preparation based on its internal logic and policies.

1.	The AF acting as VFL Server discovers via NRF the candidate NWDAF acting as VFL Clients.

When AF acting as VFL Server is trusted by the operator, it discovers the candidate NWDAF(s) acting as VFL Client(s) according to the procedure defined in Clause 6.2H.2.1.

2.	The AF acting as VFL server determine a VFL Correlation ID to be used for the VFL training process to be executed.

Based on the VFL related configurations available at AF acting as VFL Server and the information about the discovered participants, the AF acting as VFL Server determines the VFL Training Preparation Information required for the VFL Training to be executed. The VFL Training Preparation Information comprises:

-	Analytics ID;

-	VFL Correlation ID;

-	VFL Group ID;

-	VFL capability type (VFL Client) – indicating the VFL Capability is required to be performed

-	Alignment Information:

- 	Sample ID types (e.g., SUPI, GPSI, AF specific ID);

-	Sample space, which includes the list of Sample IDs to be considered for VFL Training.

-	Sample provision indication: indicate the VFL Client to provide Sample IDs corresponding to the dataset used for VFL model training.

3.	The AF acting as VFL Server provides the VFL Training Preparation request for the discovered NWDAFs acting as VFL Clients.

When the AF acting as VFL is trusted by the operator (Step 3a), the AF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service with the input parameters: ML training methodology set with “VFL”, used by the VFL Client to distinguish the type of collaborative training that is being requested (e.g., “HFL” as per Clause 62C or “VFL”); ML Preparation Flag (set to VFL preparation start), to check if the VFL Client NWDAF(s) can meet the VFL Training requirements (e.g., VFL Training Preparation Information, Data Availability requirement, FL Availability time requirement (time span needed for the FL process), etc.). Data Availability requirement includes the dataset statistical properties, the time window of the data samples and optionally the minimum number of data samples.

4.	Each VFL Client, NWDAF acting as VFL Clients, verifies whether they can join the VFL Training Process based on the support for the requested parameters received VFL Training Preparation Information and/or based on local policies. The Alignment Information received in the VFL Training Preparation Information, is used by each NWDAF or AF acting as VFL Client, to determine its supported Sample IDs corresponding to the dataset used for the VFL Training.

5.	The AF acting as VFL Server receives from the candidate NWDAF(s) acting as VFL Client(s) the VFL Training Preparation response.

When the AF acting as VFL Server is trusted by the operator (Step 5a), the NWDAF acting as VFL Client sends to AF acting as VFL Server the Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response with a confirmation that the NWDAF acting as VFL Client accepts the participation in the VFL Training Process and the Supported Alignment Information which includes the supported Sample IDs. Alternatively, the response includes or a rejection and the reason if it cannot join the VFL Training (i.e., VFL request not supported). If the NWDAF acting as VFL Client does not have the initial VFL model, it includes in the response the initial model provision indication.

6.	Based on the information received in Step 5 from each of the VFL Clients (NWDAFs) and optionally based on information acquired during discovery and/or VFL related configuration (e.g., parametrizations for a VFL Group), the AF Acting as VFL Server determines the final list of NWDAF(s) acting as VFL Client(s) participating the same VFL Training identified by the VFL correlation ID.  The AF acting as VFL Server also determines the final sample alignment information, i.e., the intersection Sample IDs received in Step 5 from the VFL Clients that accepted to participate in the VFL Training process.

7. 	The AF acting as VFL server provides to each NWDAF acting as VFL Client determined in Step 6 the response indicating the conclusion of VFL preparation phase and the final sample alignment information.

When the AF acting as VFL is trusted by the operator (Step 7a), the AF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service with the input parameters: ML Preparation Flag (set to VFL preparation concluded), the VFL Correlation ID (which indicates the VFL Preparation phase that is concluded) and the final sample alignment information.

NOTE:	Each NWDAF acting as VFL Client receiving the indication of VFL Training Preparation conclusion, stores the mapping of the VFL Correlation ID to the local ML Model to be locally trained and the final sample alignment information to be used for the VFL Training Execution.

### 6.2X.4	General Procedure for VFL Training Execution

The figure 6.2X.Y.Z-1 below shows general training procedure for Vertical Federated Learning. Both scenarios of NWDAF acting as VFL Server and AF acting as VFL Server are considered and described.

Figure 6.2X.Y.Z-1: General training procedure for VFL

Editor's Note: Further extensions are needed to show when any of the VFL participants are untrusted AF(s). In this case the procedure below will contain a NEF, and how the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor´s Note: It is FFS whether and how VFL server collects trained ML model from VFL clients after the VFL training process.

0.  The NWDAF acting as VFL Server or AF acting as VFL server and the respective VFL Clients (NWDAFs and/or AFs) complete the VFL Training Preparation procedure defined respectively in 6.2X.2 and 6.2X.3.

1.	The VFL Server (NWDAF or AF) determines based on the operator’s policy and configurations to start the VFL Training procedure with the VFL Clients (NWDAF(s) and/or AF(s)) defined in the VFL Training Preparation Phase.

The VFL server sends a request to the selected VFL clients indicating the start the local training of the ML Model related to the triggered VFL Training execution.

When NWDAF is acting as VFL Server, the NWDAF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service in the case of NWDAF(s) acting as VFL Client(s) and/or the Naf_VFLTraining_Subscribe request or Naf_VFLTraining_Request service in case of trusted AF(s) is acting as VFL Client(s) the with the input parameters: ML Preparation Flag (set to VFL execution start), VFL correlation ID, VFL capability type (VFL Client).

When a trusted AF is acting as VFL Server, the AF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service for selected the NWDAF(s) acting as VFL Client(s) with the input parameters: ML Preparation Flag (set to VFL execution start), VFL correlation ID, VFL capability type (VFL Client).

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

2.	[Optional] Each VFL client and the VFL Server collect their own local data by using the current mechanism for data collection if the VFL Client(s) and/or VFL Server do not have the local data already available. The data used by the VFL Server and VFL Client(s) is collected as per alignment information defined in the VFL Training Preparation procedure

3.	During VFL training procedure, each VFL participant, i.e., VFL Server and VFL Client(s) train their local ML model(s) associated with the same VFL Correlation ID based on their own collected or available data, generate each their local intermediary results.

4.	Each VFL Client reports its local intermediate training result of its local ML model to the VFL server.

When NWDAF is acting as VFL Server, the NWDAF acting as VFL Server receives the Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response in the case of NWDAF(s) acting as VFL Client(s) and/or the Naf_VFLTraining_Notify or Naf_VFLTraining_Request response service in case of trusted AF(s) is acting as VFL Client(s) the with the input parameters: VFL correlation ID, VFL Intermediary Result.

When a trusted AF is acting as VFL Server, the AF acting as VFL Server receives from the NWDAF(s) acting as VFl Client(s) the Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response including the parameters: VFL correlation ID, VFL Intermediary Result.

Editor’s Note: Whether and how to involve local ML Model training information in the VFL training process is FFS; What parameter contains in local ML model training information is FFS.

Editor’s Note: The details on the definition of the VFL intermediate result parameters is FFS.

5.	The VFL server computes the backward local ML model training information (e.g. gradient information or loss information) based on the intermediate training result received in step 4 from the VFL Client(s), and its own generated intermediary result and label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

The VFL server may also compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result and the label.

Editor’s Note: Whether weight of VFL Client is computed by VFL server is FFS.

6.	The VFL server evaluates (e.g., based on the convergence of a loss function or loss value, and/or the pre-set iteration number is reached) whether VFL Training process converged. If not, the VFL Server determines another round of VFL training is required. If yes, it determines the VFL Training is completed.

7.	In case the VFL Server determines another round of VFL training is required, the VFL server provides to each of the VFL Clients the backward local ML model training information determined in Step 5.

When NWDAF is acting as VFL Server, the NWDAF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service in the case of NWDAF(s) acting as VFL Client(s) and/or the Naf_VFLTraining_Subscribe request or Naf_VFLTraining_Request service in case of trusted AF(s) is acting as VFL Client(s) with the input parameters: VFL correlation ID, and backward local ML model training information associated with the VFL Client .

When a trusted AF is acting as VFL Server, the AF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service from the participating NWDAF(s) acting as VFL Client(s) with the input parameters:, VFL correlation ID, and backward local ML model training information associated with the NWDAF acting as VFL Client.

Each VFL Client computes the gradient of its local model and update its local ML model based on backward local ML model training information distributed by the VFL server, and the VFL Server executes the same actions (i.e., computes gradients and update its own local model) based on the backward local ML model training information it determined for itself.

The VFL Server and VFL Clients store the information related to the updated local ML model to use in the next round of VFL training.

NOTE:	Steps 2- 7 are executed until the VFL training termination condition is reached.

8.	In case the VFL Server determines the VFL training is completed, the VFL server sends an indication of VFL training termination  to each VFL Client.

When NWDAF is acting as VFL Server, the NWDAF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service in the case of NWDAF(s) acting as VFL Client(s) and/or the Naf_VFLTraining_Subscribe request or Naf_VFLTraining_Request service in case of trusted AF(s) is acting as VFL Client(s) with the input parameters: VFL correlation ID, and ML Preparation Flag set to VFL execution concluded.

When a trusted AF is acting as VFL Server, the AF acting as VFL Server invokes the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service from the participating NWDAF(s) acting as VFL Client(s) with the input parameters:, VFL correlation ID, and ML Preparation Flag set to VFL execution concluded.

9.	 Based on the obtained indication of VFL training conclusion, the VFL Server and VFL Clients store the latest information about their locally trained Models.

Editor’s Note: It is FFS whether model storage after each training round is required.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

# * * * * End of changes * * * *