3GPP TSG-WG SA2 Meeting #165	S2-2409790

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on VFL Training Open Issues

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: It is proposed to discuss the possible way forward to solve the editor's notes related to the procedures for VFL Training.

1. Introduction

During SA2#164 the paper S2-2409416, with the baseline procedure for the VFL Training has been noted. Different companies worked together to try to converge in a consistent and stable version of the VFL Training.

The table below shows the list of ENs resulting from the discussion in SA2#164. We clustered the ENs in different areas of topics:

- procedure details;

- service and parameters definition;

- NEF role in procedures;

- storage issues;

- model exchange.

| Area of Topic | EN# | Editor’s Note |
| --- | --- | --- |
| A | 1 | Editor’s Note: Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. |
| A | 2 | Editor’s Note: It’s FFS to determine the VFL preparation procedure. |
| A | 3 | Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server. |
| A | 4 | Editor’s Note: How the VFL server and client register to the network, how to discover VFL server or VFL client is FFS. |
| A | 5 | Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS. |
| C | 6 | Editor's Note: Further extensions are needed to show when any of the VFL participants are untrusted AF(s). In this case the procedure below will contain a NEF, and how the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS. |
| B | 7 | Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS. |
| B | 8 | Editor’s Note: A possible need for a process of sample exchange between VFL server and VFL client is FFS. The process may be determined after agreement on sample and/or feature alignment. |
| B | 9 | Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment. |
| B | 10 | Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training round or there is a session concept. |
| B | 11 | Editor’s Note: It is FFS whether and how to use ML Model ID or VFL Model Correlation ID or VFL correlation ID in the VFL training process. |
| B | 12 | Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS. |
| E | 13 | Editor´s Note: It is FFS whether and how VFL server collects trained ML model from VFL clients after the VFL training process. |
| B | 14 | Editor´s Note: Parameters to be provided in the request need to be added |
| E | 15 | Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process. |
| B | 16 | Editor’s Note: Whether and how to involve local ML Model training information in the VFL training process is FFS; What parameter contains in local ML model training information is FFS. |
| A | 17 | Editor’s Note: The details on whether and how to support that VFL clients provide intermediate results to other VFL clients is FFS. |
| B | 18 | Editor’s Note: Whether weight of feature is computed by VFL server is FFS. |
| B | 19 | Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS. |
| A | 20 | Editor’s Note: The flow should be updated to show the message exchange for VFL training termination. |
| D | 21 | Editor’s Note: It is FFS whether model storage after each training round is required. |
| A | 22 | Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS. |

2. Discussion

This paper focus on the discussion of solutions for the EN in the area of topic: A – D; i.e., procedure details (A), and service and parameters definition (B)

We cluster the ENs according with their area of topic and propose a way to solve the EN and the reason behind the proposal.

(A) Procedure details

| Area of Topic | EN# | Editor’s Note | Proposal of Solution | Rationale |
| --- | --- | --- | --- | --- |
| A | 1 | Editor’s Note: Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. | Solution Covers ENs 1 – 3. 1) Define the two types of procedures: - VFL Training Preparation procedure - VFL Execution Procedure 2) Split the VFL Training Procedure between AF acting as VFL server and NWDAF acting as VFL in order to make the clear differentiation on the role of the Consumer of a ML Model (i.e., addressing EN #3) in the procedure | TS 23.288 defines in Clause 6.2C the procedures for FL, which in fact is the Horizontal Federated Learning as defined in Clause 5.3 and where only NWDAFs are involved. Different than HFL, VFL is a technique that will involve NWDAFs, AF, and NEFs. Basically all interactions among the VFL Server and VFL Client during the steps of calculating intermediary results, checking the VFL process convergence, distributing information for the new round, are the same no matter if the VFL Server is an NWDAF or an AF. However, this is not the case for the preparation phase, were the trigger for the VFL procedures are different if AF or NWDAF is acting as VFL Server. Therefore, different procedures are essential in the VFL training preparation phase, but the same procedure can be used in the actual VFL training execution, differentiating only the services being invoke, but not the flow itself. |
| A | 2 | Editor’s Note: It’s FFS to determine the VFL preparation procedure. |  |  |
| A | 3 | Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server. |  |  |
| A | 4 | Editor’s Note: How the VFL server and client register to the network, how to discover VFL server or VFL client is FFS. | Remove ENs and include a note pointing to the Clause in Ts 23.288 related to the VFL discovery procedure agreed during SA2#164. | This issue has been solved in the agreed paper S2-2409419 during SA2#164. Any changes related to the VFL discovery procedure should be updated in the respective agreed clause. |
| A | 5 | Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS. | De-prioritize in R19 the support for dynamic reconfiguration of either VFL Clients or changes in the sample alignment. The straight forward solution is to consider that any change to VFL Clients or sample alignment information should be handled as a new VFL process, therefore a new VFL Correlation ID should be assigned. A note can be include to reflect this straight forward solution. | Given the short amount of TUs for AIML, the standardization effort should focus on the basic mechanism to make VFL Training procedure stable. The dynamicity in changing VFL clients and sample IDs in the sample alignment information should be simplified and treated as a new VFL process in R19. |
| A | 17 | Editor’s Note: The details on whether and how to support that VFL clients provide intermediate results to other VFL clients is FFS. | Removed EN. De-prioritize in R19 the support for exchange of intermediary results among VFL Clients | Reasons to de-prioritize such type of interactions are based on: - Exchange of intermediary results is a corner case of VFL execution, and the standardization process should focus the effort in standard practices and leave the variations up to implementation. |
| A | 22 | Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS. | Remove EN. De-prioritize in R19 the concept of training session, because the concept of VFL Correlation ID (agreed in the conclusion of TR 23.700-84)and the use of the VFL Training Conclusion Flag combined give in a simpler way the same meaning. | The concept of a training session could be required if dynamicity on changing VFL Clients and sample IDs in the alignment information would be supported. As per above, in R19 the standardization effort should focus on simple and direct alternatives to consolidate and define a stable procedure for VFL training. |

B. Service and parameters definition

| Area of Topic | EN# | Editor’s Note | Proposal of Solution | Rationale |
| --- | --- | --- | --- | --- |
| B | 8 | Editor’s Note: A possible need for a process of sample exchange between VFL server and VFL client is FFS. The process may be determined after agreement on sample and/or feature alignment. | The VFL Training Preparation phase is mainly the procedure for the sample alignment, where the VFL server includes in a request to the VFL clients the proposal of sample space (including the sample IDs) and the VFL Clients can agree with the proposed sample space, or indicate to the VFL Server a sub-set of the sample scape that they can support for the request ed VFL training. | Feature is vendor specific and should not be transferred via signalling |
| B | 9 | Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment. | Remove EN. | Feature is vendor specific and should not be transferred via signalling. |
| B | 10 | Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training round or there is a session concept. | Only sample alignment needs to be exchanged in the VFL Training preparation procedure to allow all VFL participants to determine the sample IDs to be used for the VFL training procedure. There is no support for dynamic changes in the sample space for a given VFL training process (i.e., VFL correlation ID). Any change in the sample IDs of a sample space should be treated as VFL training process and therefore require a new VFL Training Preparation procedure. | In order to simplify the standardization process and due to the time constraints in this process, it is straight forward to consider that once the sample space is agreed in the VFL preparation procedure, there is no change in the sample space during the VFL Training execution for a given VFL correlation ID. Therefore, there is no need to exchange the sample alignment at each round of training process. |
| B | 11 | Editor’s Note: It is FFS whether and how to use ML Model ID or VFL Model Correlation ID or VFL correlation ID in the VFL training process. | Proposal to use both the ML Model ID to identify a ML model that is being trained by NWDAF or AF; and the VFL Correlation ID to signal that the ML Model ID is associated with a VFL training process (i.e., a specific set of VFL participants and specific set of sample alignment). | Previous releases of NWDAFs base all the operations related to ML Models (e.g., model provisioning, model retrieval, model accuracy) on the unique ML model identifier. Substitute the ML model ID in the VFL training procedure by the VFL Correlation ID would create incompatibilities with many of the existing mechanisms in NWDAF architecture of previous releases. |
| B | 12 | Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS. | Include the new parameter defined as VFL interoperability information and it associated VFL interoperability indicator The VFL interoperability information defines all the parameters that can be applicable to the VFL process (e.g., possible set of VFL participants – e.g., Operator A, AF B, AF, C, etc – possible sample IDs that can be part of a sample alignment, ML model applicable requirements, etc. ) The VFL interoperability indication is the unique identifier of the parametrization defined in a VFL interoperability information. The VFL interoperability information is configured at each VFL participant and supports the VFL Server to determine the VFL Training Preparation request, and the VFL Clients to determine if they support the received VFL training Preparation request. The VFL interoperability indicator is included in the message exchange from VFL server to VFL Clients to indicate which of the VFL interoperability information are going to be used for the VFL Training Preparation request. | The simplest way to achieve a stable standardized support for VFL operation is to rely on information that has been previously agreed among the possible VFL Participants. By considering that configurations are available in each VFL participant, the standard work can focus on the essential interactions and leave to later stages the changes to the stable procedure to handle the introduction (if really required) of dynamicity’s in the system. |
| B | 14 | Editor´s Note: Parameters to be provided in the request need to be added | Remove EN | All parameters to all services request and response in the VFL Training Preparation and Execution procedures have been defined. |
| B | 18 | Editor’s Note: Whether weight of feature is computed by VFL server is FFS. | EN reworded: Editor’s Note: Whether weight of VFL Client is computed by VFL server is FFS. | As per previous comments, the feature space should be considered configured and known to each VFL participant. Therefore, the weight is related to the VFL participant not to a dynamically changed feature space. |
| B | 19 | Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS. | EN removed and inclusion of VFL Training termination flag | Due to the proposal to reuse the same server for the VFL Training Preparation and VFL Training Execution, it is necessary to also indicate to the VFL Clients when the training is complete. This will also allow the VFL Client to prepare and store properly the jointly trained model that later can be used for VFL inference. |

3. Conclusion and proposal(s)

The proposal is to enforce the afore mentioned changes in a revision of the baseline paper S2-2409416.