3GPP TSG-WG SA2 Meeting #165	S2-2409803

# 14 - 18 October, 2024, Hyderabad, India		(revision of S2-2408877)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4898 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 Support of N6 delay measurement and UPF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon], China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in KI#2 of FS_ eEDGE_5GC_Ph3, based on the AF input, the SMF may indicate UPF(s) to measure N6 delay. Since the N6 delay is per node level (i.e. between UPF and endpoint at application side), this contribution proposes to use N4 node level procdure for measurement and reporting. Discussion about the IEs provided from AF to SMF are proposed in our CR 5485. As discussed in S2-2405648, it is proposed that SMF indicates L-PSA UPF to measure UL/DL/RT N6 delay. Besides, protocol specific information is unclear, and it is suggested to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add a new clause for the procedure of N6 delay measurement by UPF. 2. Add clarification in the N4 node level procedure to support N6 delay measurement. Add parameter in step 4: Indication for UL N6 delay measurement/Indication for DL N6 delay measurement/Indication for RT N6 delay measurement. Protocol specific information is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N6 delay measurement by UPF as instructed by SMF cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X(new), 4.4.3.1, 4.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.X	Support of N6 delay measurement

This clause describes how the N6 delay measurement for the connection between the UPF(s) and the endpoint at the application side (e.g. EAS) is performed.

SMF

PCF

UPF

EAS

2. trigger for N6 delay measurement

6. N6 delay measurement using protocols required by AF

7. N6 delay report

UE

0. PDU session establishment

1. receive measurement assistance information via PCC rule

3. UPF selection

4. N4 Association Setup/Update Request

5. N4 Association Setup/Update Response

Figure 4.3.X-1: N6 delay measurement triggered by SMF

0.	UE establishes PDU session as described in clause 4.3.2.2.

1.	During PDU session establishment or modification procedure, SMF receives PCC rule including N6 delay measurement assistance information as defined in clause 5.6.7.1 of TS23.501[2] from PCF.

2.	The SMF determines to trigger N6 delay measurement if the N6 delay of the Measurement endpoint(s) is not availible yet. For example, as described in step 14 of clause of TS 23.548[74], the SMF receives EAS information notification from EASDF and the notified EAS IP address is included in the Measurement endpoint(s) for N6 delay indicated by AF.

3.	Per the received N6 delay measurement assistance information, the SMF selects and determines the candidate UPF(s) for N6 delay measurement (e.g. UPF(s) in the DNAI(s) indicated by AF in the Potential Locations of Applications).

4.	The SMF sends following information to candidate UPF(s) via N4 association setup/update request as described in clause 4.4.3:

-	Indication for considering N6 delay,

-	 Measurement endpoint(s) for N6 delay,

-	Measurement protocol,

-	Indication for UL N6 delay measurement/Indication for DL N6 delay measurement/Indication for RT N6 delay measurement,

-	Optionally, conditions for the UPF to report (e.g. immediate report, periodic report with the reporting timeperiod, event triggered report with delay threshold). For periodic report, the SMF generates the timeperiod based on the Measurement frequency for N6 delay received in step 1. For event triggered report, the SMF generates delay threshold based on end-to-end delay requirement received in step 1.

5.	The UPF response to SMF with an N4 association setup/update response message.

6.	Based on the information received from SMF, UPF(s) performs N6 delay measurement leveraging the measurement protocols (e.g., ICMP, TWAMP, OWAMP, STAMP, or other protocols) for the connection between the endpoint at the application side (e.g. EAS) and itself using measurement protocols indicated by the SMF.

7.	UPF(s) sends the N6 delay measurement result to the SMF.

Editor’s note: Whether the report is via PFCP or SBI is FFS.

# * * * * Second change * * * *

4.4.3	N4 Node Level Procedures

4.4.3.1	N4 Association Setup Procedure

The N4 Association Setup procedure is used to setup an N4 association between the SMF and the UPF, to enable the SMF to use the resources of the UPF subsequently to establish N4 Sessions. The SMF and UPF may exchange the supported functionalities on each side during these procedures.

The setup of an N4 association is initiated by the SMF. SMF and UPF may additionally support an N4 association initiated by UPF.

The SMF should only establish an N4 association with a UPF that supports F-TEID allocation at the UPF.

The SMF initiates the N4 Association Setup procedure to request to setup an N4 association towards a UPF prior to establishing a first N4 session on this UPF.

When receiving an N4 Association Setup Request, the UPF shall send an N4 Association Setup Response.

N4 Association Setup procedure can be used to request the UPF to measure and report the clock drift between the external time and 5GS time for one or more external time domains by provisioning External Clock Drift Report and providing the corresponding Time Domain number(s) as specified in TS 29.244 [69]. The SMF may omit the Time domain number in the request; in this case the UPF shall report the clock drift for all Time domains the UPF is connected to.

N4 Association Setup procedure can be used to request the UPF to measure and report the N6 delay for the connection between the UPF and the endpoint at the application side (e.g. EAS) as described in clause 4.3.X.

Figure 4.4.3.1-1: N4 association setup procedure initiated by SMF

The UPF may initiate the N4 Association Setup procedure to request to setup an N4 association towards a SMF prior to establishing a first N4 session on this UPF.

When receiving an N4 Association Setup Request, the SMF shall send an N4 Association Setup Response.

Figure 4.4.3.1-2: N4 association setup procedure initiated by UPF

4.4.3.2	N4 Association Update Procedure

The N4 Association Update procedure shall be used to modify an existing N4 association between the SMF and the UPF. It may be initiated by the UPF or by the SMF to update the supported features or available resources of the UP function.

N4 Association Update procedure can be used by the SMF to update the provisioning of External Clock Drift Report as specified in clause 4.4.3.1.

N4 Association Update procedure can be used by the SMF to request or update the provisioning measurement assistance information of N6 delay for the connection between the UPF and the endpoint at the application side (e.g. EAS) as specified in clause 4.3.X.

Figure 4.4.3.2-1: SMF initiated N4 association update procedure

Figure 4.4.3.2-2: UPF initiated N4 association update procedure

# * * * * End of changes * * * *