SA WG2 Meeting SA2#165	S2-2409819

Hyderabad, India, October 14 – 18, 2024	(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5660 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NF selection considering energy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EnergySys |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the KI#3 conclusion, there is following conclusion for NF discovery and (re-)selection: - NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy. NOTE 1: Which energy related information is included in the NF profile is to be further discussed in the normative phase. Only parameters captured in existing solutions will be considered. Coordinated with SA WG5 may be needed. NOTE 2: The information within NF profile is expected not to change frequently, e.g. constantly reflecting changes in energy consumption. The procedure for updating the information will be addressed in normative phase. - NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer. Above conclusion needs to be capatured in normative work. AMF needs to consider its own energy satus for AMF re-slection. During the traffic influence procedure, AF may provide maximum allowed energy consumption for a DNAI or energy saving indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NF profile is extended to include the new energy related information - Energy state as specified in TS 28.310 [7]. e.g. notEnergySaving state, energySaving state. - Energy consumption level, e.g., low, middle and high. - Energy Efficiency Level, e.g., low, middle and high. - Energy resource information, e.g., renewable sources. If an AMF instance needs to enter an energySaving state in which it stops serving UEs, it may: - deregister UE with congestion control; - invoke the AMF planned removal procedure in clause 5.21.2.2 to cause the NG-RAN to select other AMF instances to serve UEs. - deregister UE with energySaving state cause value and request UE re-registration for AMF reallocation. During registration, if the selected AMF instance is in energySaving state which can not serve UEs, it may initiate the Registration with AMF re-allocation procedure to instruct the NG-RAN to select other AMF instances to serve UEs. During the traffic influence procedure, AF may provide maximum allowed energy consumption for a DNAI or energy saving indication. SMF may use this information to decide UP path establishment and adjustment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NF discovery and (re-)selection will not consider the energy related info which is not aligned with the conclusion in TR23.700-66. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2, 5.X (new), 5.X.X(new), 5.X.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Changes

## 5.x 	Support for Energy Efficiency and Energy Saving

### 5.x.x	NF discovery and (re-)selection based on energy related information

NF profile as described in clause 6.2.6.2 further includes energy related information for NF discovery and (re-)selection which is to improve the system energy efficiency and energy saving. The energy related information may include the following parameters:

-	Energy state as specified in TS 28.310 [7]. e.g. notEnergySaving state, energySaving state.

-	Energy consumption level, e.g., low, middle or high.

-	Energy Efficiency Level, e.g., low, middle or high.

-	Energy resource information or ratio, e.g., renewable sources or ratio.

-	Carbon emission factor

Editor's note: the Energy resource information or ratio will be further clarified in cooperation with SA5.

The energy related information listed above may be provided by the NF via NF service Registration as in clause 4.17.1 of TS23.502[] or be configured and updated in NRF by OAM.

If an AMF instance needs to enter an energySaving state in which it stops serving UEs, it may:

-	deregister UE with congestion control;

-	invoke the AMF planned removal procedure in clause 5.21.2.2 to cause the NG-RAN to select other AMF instances to serve UEs.

-	deregister UE with energySaving state cause value and request UE re-registration for AMF reallocation.

During registration, if the selected AMF instance is in energySaving state which can not serve UEs, it may initiate the Registration with AMF re-allocation procedure to instruct the NG-RAN to select other AMF instances to serve UEs.

### 5.X.Y		UP Path management for energy saving and energy efficiency.

During PDU session establishment, SMF performs UPF discovery and selection taking the energy related information in the NF profile as described in clause 5.x.x of UPFs into account for energy saving and energy efficiency.

During the traffic influence procedure, AF may provide maximum allowed energy consumption for a DNAI or energy saving indication. SMF may use this information to decide UP path establishment and adjustment.

Second Changes

#### 6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in clause 5.2.7.2.2 of TS 23.502 [3]. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information or serving scope for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, Home Network Public Key identifier, for UDM and AUSF.

-	For UDM, AUSF and NSSAAF in the case of access to an SNPN using credentials owned by a Credentials Holder with AAA Server, identification of Credentials Holder (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used for access to an SNPN using credentials owned by a Credentials Holder, identification of Credentials Holder (i.e. the realm if Network Specific Identifier based SUPI is used or the MCC and MNC if IMSI based SUPI is used); see clause 5.30.2.1.

-	For AUSF and NSSAAF in the case of SNPN Onboarding using a DCS with AAA server, identification of DCS (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used as DCS in the case of SNPN Onboarding, identification of DCS (i.e. the realm if Network Specific Identifier based SUPI, or the MCC and MNC if IMSI based SUPI).

-	One or more GUAMI(s), in the case of AMF.

-	For the UPF, see clause 5.2.7.2.2 of TS 23.502 [3].

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	For NWDAF, the following information are supported:

-	Analytics ID(s) (possibly per service).

-	NWDAF Serving Area information (refer to clause 6.3.13).

-	Supported Analytics Delay per Analytics ID (if available).

-	NF types of the NF data sources, NF Set IDs of the NF data sources, if available.

-	Analytics aggregation capability (if available).

-	Analytics metadata provisioning capability (if available).

-	ML model Filter information parameters include S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s).

-	ML Model Interoperability indicator (if available) per Analytics ID(s).

-	FL capability information per analytics ID including FL capability type (i.e. FL server and/or FL client, if available).

-	Time interval supporting FL (if available).

-	Accuracy checking capability for ML model accuracy monitoring or Analytics Accuracy Monitoring (if available).

-	Roaming exchange capability (if available).

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay i.e. the Analytics report can be generated with a time (including data collection delay and inference delay) in less than or equal to the Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Event Exposure service supported event ID(s) by UPF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs or Range(s) of GPSIs or a BSF Group ID, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, if available, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

-	For SNPN, capability to support SNPN Onboarding in the case of AMF and capability to support User Plane Remote Provisioning in the case of SMF.

-	IP address range, DNAI for UPF.

-	Operator configurable UPF capability for UPF features(s) as described in clause 5.8.2.21.

-	Supported DNS security protocols, in the case of EASDF.

-	Additional V2X related NF profile parameters are defined in TS 23.287 [121].

-	Additional ProSe related NF profile parameters are defined in TS 23.304 [128].

-	Additional MBS related NF profile parameters are defined in TS 23.247 [129].

-	Additional UAS related NF profile parameters are defined in TS 23.256 [136].

-	Additional Ranging based services and Sidelink Positioning related NF profile parameters are defined in TS 23.586 [180].

-	For additional information in PCF profile, see clause 5.2.7.2.2 of TS 23.502 [3].

-	Energy related information is defined in clause 5.x.x.

End of Changes