SA WG2 Meeting #165	S2-2409833

# Hyderabad, India, Oct 14th–Aug 18th, 2024        										(was S2-2407584)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5424 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining Device Identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New WID on User Identities and Authentication Architecture is agreed to define normative specifications for identifying non-3GPP devices connecting behind a UE or 5G-RG. Hence it is required to define the same. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes as per the reason for change documented above. It is proposed to update the abbreviations, add additional section under 5.9 describing about the Device Identifier; explain the functionality of non-3GPP devices; corresponding updates to NEF and UDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature would be missed in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.X, 5.9.X.1, 5.X, 5.X.1, 5.X.2, 6.2.5, 6.2.5.0, 6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.9	Identifiers

### 5.9.X	Additional Identifiers

#### 5.9.X.1	Non-3GPP Device Identifier

A Non-3GPP Device Identifier is a permanent identifier whose format is a general string and is used by 5GS to identify non-3GPP devices behind a gateway UE or a 5G-RG.

NOTE X1:	How UE/5G-RG binds Non-3GPP Device Identifier to non-3GPP device is out of scope of 3GPP.

The 3GPP system stores Non-3GPP Device Identifier(s), their current status and corresponding QoS rules for a given UE subscription.

# * * * * Next change * * * *

## 5.X	Support of additional identifiers

### 5.X.1	General

### 5GS will identify the traffic from each individual non-3GPP device with a Non-3GPP Device Identifier for a specific UE subscription and use this Non-3GPP Device Identifier to obtain QoS Settings for the corresponding traffic. 5GS shall support identifying non-3GPP devices based on Non-3GPP Device Identifier described in clause 5.9.X.1.

NOTE X1:	The connection between non-3GPP devices and UE/5G-RG is not impacted.

Non-3GPP Device Identifier and their corresponding QoS rules associated with a UE/5G-RG subscription are provisioned into the UDR. This provisioning may be carried out through AF/NEF as described in clause 4.15.6.7 of TS 23.502 [3], which would enable the AF to manage Non-3GPP Device Identifier(s) in the UDR.

NOTE X2:	The UE/5G-RG supports the traffic from non-3GPP devices that are not linked with a Non-3GPP Device Identifier. Non-3GPP devices that do not need to be identified by 5GS can use the QoS Flow(s) established using Rel-18 Connectivity Group ID mechanisms.

UE/5G-RG may send the Non-3GPP Device Identifier and user plane address of the non-3GPP device to the SMF in a NAS-SM message as described in clause 4.3.2.2 and 4.3.3.2 of TS 23.502 [3] and the SMF provides Non-3GPP Device Identifier and user plane address to the PCF as described in clause 5.2.5.4.2 of TS 23.502 [3]. The PCF will consider, the Non-3GPP Device Identifier information retrieved from the UDR for decisions as described in clause 4.15.6.X of TS 23.502 [3] and as described in clause 6.2.1.2 of TS 23.503 [45]. Non-3GPP Device Identifier and the SUPI (of the corresponding UE subscription) is used to retrieve the information from the UDR. The user plane address for a non-3GPP device can be the UE IP Address and/or Port ranges in case of IPv4 or an IPv6 address for IP PDU session type and the MAC address and/or the VLAN tag ID that is associated with the non-3GPP device’s traffic for Ethernet PDU Session type.

NOTE X3:	For non-3GPP devices with Non-3GPP Device Identifier provisioned whose traffic need to be differentiated, in case of NAT, UE/5G-RG will ensure that it does not NAT the traffic from different non-3GPP devices to the same IP address and port number range combination.

### 5.X.2	QoS differentiation

QoS differentiation may be performed within a PDU Session (using different QoS Flows) or between PDU Sessions which shall be determined based on the URSP rules as described in clause 4.15.6.10 of TS 23.502 [3].

In the case of non-3GPP devices sharing a PDU Session, QoS Flows are implemented using PDU Session Modification procedures. AF session with required QoS may be implemented as per clause 4.15.6.6 of TS 23.502 [3] for the corresponding non-3GPP device.

PCF for the PDU session may query the policy parameters for the key identified by the SUPI and sub keys DNN/S-NSSAI or Non-3GPP Device Identifier to implement corresponding QoS rules.

NOTE X1:	Based on the operator policies, PCF may implement default QoS rules for the Non-3GPP Device Identifier for a UE/5G-RG received as part of the PDU Session Modification procedures, overriding the QoS rules defined in the UDR for the Non-3GPP Device Identifier.

# * * * * Next change * * * *

### 6.2.5	NEF

#### 6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, time synchronization service information and PDU Set handling service specific information. In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	Redirecting the AF to a more suitable NEF/L-NEF e.g. when serving an AF request for local information exposure and detecting there is a more appropriate NEF instance to serve the AF's request.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Support management of ECS Address Information.

-	Support management of relationship between DNAI and EAS Address Information.

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

-	Charging data collection and support of charging interfaces.

-	Support of Member UE selection assistance functionality:

-	NEF may provide one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information to the AF based on the parameters contained in the request from the AF as described in clause 5.46.2. NEF supports the translation of the member UE selection filtering criteria parameters received from the AF to the corresponding event or analytics filters that can be understood by the 5GC NFs for events or analytics related data collection. NEF interacts with 5GC NFs using existing services in order to collect the corresponding data and then derive the list(s) of candidate UE(s) and other assistance information as described in clause 4.15.13 of TS 23.502 [3].

-	Support of Multi-member AF session with required QoS for a set of UEs identified by a list of UE addresses:

-	Details are specified in clause 4.15.6.13 of TS 23.502 [3].

-	Support of UAS NF functionality:

Details are defined in TS 23.256 [136].

-	Support of EAS deployment functionality:

Details are defined in TS 23.548 [130].

-	Support of SBI-based MO SM transmit for MSISDN-less MO SMS:

Details are defined in TS 23.540 [142].

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Support management of common EAS and common DNAI:

Details are defined in TS 23.548 [130].

-	Support of non-3GPP devices behind UE or 5G-RG functionality as defined in clause 5.X.1.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

# * * * * Next change * * * *

### 6.2.11	UDR

The Unified Data Repository (UDR) supports the following functionality:

-	Storage and retrieval of subscription data by the UDM.

-	Storage and retrieval of policy data by the PCF.

-	Storage and retrieval of structured data for exposure.

-	Application data (including Packet Flow Descriptions (PFDs) for application detection, AF request information for multiple UEs, 5G-VN group information for 5G-VN management).

-	Storage and retrieval of NF Group ID corresponding to subscriber identifier (e.g. IMPI, IMPU, SUPI).

-	Storage and retrieval of Non-3GPP Device Identifier(s) and their corresponding QoS rules per UE subscription.

The Unified Data Repository is located in the same PLMN as the NF service consumers storing in and retrieving data from it using Nudr. Nudr is an intra-PLMN interface.

NOTE 1:	Deployments can choose to collocate UDR with UDSF.

# * * * * End of changes * * * *