3GPP TSG-WG SA2 Meeting #165	S2-2409839

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0580 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of event filter for GNSS assistance data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on LS C3-244452 from CT3, it is not clear whether LMF/NEF needs to indiciate an event filter (e.g., location area, target UE(s)) while subscribing for the event "GNSS Assistance Data" with the AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that a location area event filter may be optionally provided to AF, as LMF may only serve a specific area and only need the “GNSS Assistance Data” applicable to its serving area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear whether LMF/NEF needs to indiciate an event filter while subscribing for the event "GNSS Assistance Data" with the AF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15.1, 6.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.15.1	GNSS assistance data collection from untrusted AF via NEF

The following procedure is used by LMF to collect GNSS assistance information from AFs via the NEF.

Figure 6.15.1-1: GNSS assistance data collection from untrusted AF

1a-1c.	Reuse the steps 1a-1c in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	The AF should also provide its serving area to NEF. NEF should map the serving area into TA(s) if it is presented as geographical location and update the NEF profile in NRF including the serving area information.

1d-1e.	Reuse the steps 1d-1e in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	When LMF needs to discovery the available data from AFs and the appropriated NEF to collect GNSS assistance data, LMF additionally provides TAs in the Nnrf_NFDiscovery_Request_request service operation.

2-5.	Reuse the steps 2-5 in figure 6.2.2.3-1 of TS 23.288 [37] with following difference:

-	Besides available GNSS assistance data, AF also provides associated serving area of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data. NEF may map the serving area into a TA or TA list.

-    When subscribing for the event GNSS Assistance Data with the AF, the LMF may also indiacte an event filter which contains a location area (e.g., the LMF serving area) to AF via NEF. The NEF may map the location area into a geographical location.

6.	After LMF gets the GNSS assistance data, it can send the assistance data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

# * * * * Second change * * * *

6.15.2	GNSS assistance data collection from trusted AF

For trusted AFs, LMF collects GNSS assistance data through AFs directly. LMF discovers trusted AFs covering LMF serving area through NRF and subscribes to GNSS assistance data exposure from discovered trusted AFs.

Figure 6.15.2-1: GNSS assistance data collection from trusted AF

1a-1c.	AF invokes Nnrf_NFManagement_NFUpdate_request service operation to register its GNSS related information including Event IDs, and associated AF identification, Application ID(s) and its serving area (TAs). NRF stores the received AF update information and sends Nnrf_NFManagement_NFUpdate_response message to AF.

1d-1e.	When LMF needs to discovery the available GNSS assistance data from AFs, LMF provide TAs to NRF through Nnrf_NFDiscovery service to discover related AFs.

2.	LMF subscribes to or cancels subscription to GNSS assistance data in AF by invoking the Naf_EventExposure_Subscribe/Naf_EventExposure_Unsubscribe service operation. The LMF may also indiacte an event filter which contains a location area (e.g., the LMF serving area) to AF.

3.	AF notifies the LMF with the available GNSS assistance data by invoking Naf_EventExposure_Notify service operation. AF also provides associated serving area (TAs) of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data. The AF notifies the LMF with the data by invoking Nnef_EventExposure_Notify service operation.

4.	After LMF gets the GNSS assistance data, it can send the data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

# * * * * End of changes * * * *