3GPP TSG-SA2 Meeting #165	S2-2409861

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0582 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Bulk data collecion by MTLF from LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to conclusions for key issue 1 in TR 23.700-84, the MTLF may train models for location inference or train. Procedures to enable the MTLF to retrive related input data are required.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An LMF service that allows the MTLF to request input data to train models for LMF based positioning is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MTLF is not able to retrieve input data to train models for LMF based positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, new 5.18.X, 8.3.1, new 8.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 5.18	Support for UE positioning based on a ML Model at the LMF

### 5.18.0	General

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 5.18.1.

Editor's note:	The procedure of LMF collects input data from NG-RAN for ML model training is FFS.

LMF may also request a ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF. LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37]. LMF requests ML model training for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest. NWDAF containing MTLF provision the model to LMF as described in clause 6.2A of TS 23.288 [37]. The NWDAF containing MTLF collects input data from the LMF as described in Clause  5.18.x and trains the ML model for LMF-based AI/ML Positioning.

Editor's note:	The procedure of NWDAF containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

Editor's note:	The Performance monitoring procedures are FFS.

### 5.18.1	Data collection at LMF to train the AI/ML based positioning model to perform positioning based on UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Editor's note:	The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

Editor's note:	Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

3.	The LMF checks the user consent for data collection for model training.

Editor's note:	Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS.

Editor's note:	Further details on user consent for data collection for model training will be aligned with SA WG3.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the AI/ML based positioning model.

### 5.18.X	Data collection at MTLF to train the AI/ML based positioning model

The MTLF may request the LMF to collect and provide input data to train models AI/ML based positioning.

Figure 5.18.X-1: Data collection by MTLF to train the AI/ML based positioning model

1.	The MTLF decides to train a model for location deremination by an LMF, selects an LMF based on its capability to support the NlmfTrainingData service and the are of interest, and sends a Nlmf_TrainingData_Subscribe request to the LMF(Training Data type(UE or RAN measurements), area of interest, [number of training data samples], [time window for training data collection], notification target address)

2. The LMF selects UEs in the area of interest and collects training data samples as described in Figure 5.18.1-1

3.	The LMF notifies the LMF about collected data samples containing both maessurements and related ground truth labels. This step may be repeated to provide additional collected data samples.

Editor's note:	The content of the data samples needs to be defined based on related decisions of RAN WGs.

2nd change

### 8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify |  | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |
| Nlmf_TrainingData | Subscribe | Subscribe/Notify | MTLF |
|  | Unscubscribe |  | MTLF |
|  | Notify |  | MTLF |

3rd change

### 8.3.X	Nlmf_TrainingData service

#### 8.3.X.1	General

Service description: This service enables an NF service consumer to request training data to trainlocation related models. See Clause 5.18.X for examples pf usage of this service.

#### 8.3.X.2	Nlmf_TrainingData_Subscribe service operation

Service operation name: Nlmf_ TrainingData_Subscribe

Description: This service enables an NF consumer to subscribe to receive location related training data to train models.

Input, Required:. Training Data Type  (UE location maessurements, NG RAN location maessurements), area of interest, Notification Target Address (+ Notification Correlation ID)

Input, Optional: time window when to collect training data, number of samples

Output, Required: Success/Failure indication. When the subscription is accepted: Subscription Correlation ID (required for management of this subscription)

Output, Optional: none

When the requested number of samples has been provided or the time window for data collection has elapsed the subscription is automatically terminated.

#### 8.3.X.3	Nlmf_TrainingData_Unsubscribe service operation

Service operation name: Nlmf_ TrainingData_Subscribe

Description: This service enables an NFservice consumer to cancel a subscribtion to receive location related training data to train models.

Input, Required: Subscription Correlation ID

Input, Optional: none

Output, Required: Success/Failure indication

Output, Optional: none

#### 8.3.X.4	Nlmf_ TrainingData_Notify service operation

Service operation name: Nlmf_TrainingData_Notify.

Service operation description: This service enables LMF to send data samples as requested by the subscribtion to the NF service consumer. The service operation may be executed several times, until the requested number of data samples has been provided.

Input, Required: Notification Correlation ID, Training Data Sample(s).

Input, Optional:.

Output, Required: None.

Output, Optional: None.

End of changes