3GPP TSG-SA2 Meeting #165	S2-2409867

# Hyderabad, IN  October 14-18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1220 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: QoE Assistance information analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR updates TS 23.288 to include analytics for QoE assistance information analytics as described in the S2-240xxx discussion paper. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new analytics in TS23.288 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#3 – conclusion in TR23.700-81 to support QoE assistance information analytics is not implemented for Rel .19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.x	QoE Assistance Information Analytics

#### 6.x.1	General

The consumer (e.g. PCF) of QoS Parameters Assistance analytics may obtain analytics from the NWDAF to assess the QoE that may be obtained using Candidate QoS parameter sets proposed by the consumer. For each candidate set of QoS parameters provided by the PCF, the NWDAF returns QoE analytics (statistics and/or predictions). The QoE provided to the consumer is similar to the QoE obtained with Observed Service Experience analytics - i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming, V2X services and Web Browsing services as indicated by the consumer.

The PCF may request the service experience for the candidate QoS parameter set(s) for the following scenarios:

- Service Experience for the Candidate QoS parameter set(s) for a Network Slice: Service Experience for a UE or group of UEs or any UE in a Network Slice;

- Service Experience for the Candidate QoS parameter set(s) for an Application: Service Experience for a UE or group of UEs or any UE using an Application or a set of Applications;

- Service Experience for the Candidate QoS parameter set(s) for an Edge Application over a UP path: Service experience for a UE or a group UEs or any UE using an Application or a set of Applications over a specific UP path (UPF, DNAI and EC server);

- Service Experience for the Candidate QoS parameter set(s) for an Application over a RAT Type or Frequency or both: Service experience for a UE or group of UEs using an Application or a set of Applications over a RAT Type or over a Frequency or both as defined in Table 6.4.1-1.

- Service Experience for the Candidate QoS Parameter set(s) for an Application transferring data over a PDU Session: Service experience for a UE or group of UEs or any UE using an Application or a set of Applications transferring data over a PDU Session with PDU Session parameters i.e. S-NSSAI, DNN, PDU Session Type , SSC mode and optionally an Access Type or with combination of PDU Session parameters such as a list of the tuple (PDU Session Type, SSC mode) optionally per Access Type.

The analytics may be used to assist the consumer (i.e. PCF) in determining QoS parameters.

A consumer may decide according to policy/implementation: the one or multiple candidate QoS Parameter sets, comprising a 5QI and/or individual QoS parameters (including candidate values for each parameter) for which it wants to obtain analytics, and optionally Application IDs for which the analytics are to be determined.

The service consumer may be a PCF

The consumer of these analytics shall indicate in the request or subscription the same information defined for Observed Service experience analytics in clause 6.4, with the following changes:

-	Analytics ID =  “QoE Assistance Information".

-	Analytics Filter Information shall in addition include:

- One or more Candidate QoS Parameter Sets (mandatory), each comprising:

-	A Candidate QoS Parameter Set Identifier (mandatory)

-	5QI (standardized or pre-configured) and applicable additional QoS parameters and the corresponding values (conditional, e.g if needed for GBR 5QIs to know the GFBR); and/or

-	the QoS Characteristics attributes including e.g.: Resource Type, PDB, PER and their values;

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.x.3.

#### 6.x.2	Input Data

The NWDAF collects data, as defined in clause 6.4.2, with the following changes:

- In table 6.4.2-2 – “QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service”, QoS Parameters and Characteristics Information (e.g. 5QI) may be obtained from the SMF.

#### 6.x.3	Output Data

The NWDAF provides the Service Experience Statistics, as defined in clause 6.4.3, with the following changes:

- In table 6.4.3-1 – “Service Experience statistics”, the NWDAF provides “Candidate QoS Parameter Set Identifier(s)” to indicate the candidate QoS parameter set(s) for which the statistics are applicable.

- In table 6.4.3-2 – “Service Experience Predictions”, the NWDAF provides “Candidate QoS Parameter Set Identifier(s)” to indicate the candidate QoS parameter set(s) for which the predictions are applicable.

#### 6.x.4	Procedures

##### 6.x.4.1	QoS Parameters Assistance Analytics

Figure 6.x.4.1-1: QoS Parameters Assistance Analytics procedure

1.	5GC NF consumer (e.g. PCF) sends a Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information). The consumer may specify an Analytics Filter containing  candidate QoS parameter set(s) for which analytics are requested.

The Analytics ID is set to "QoE Assistance Information”.

2.	The NWDAF collects data from OAM, NF(s) and/or AF as specified in clause 6.4.4, 6.4.5 and 6.4.6.

3.	The NWDAF derives requested analytics. Separate analytics are determined for each candidate QoS parameter set.

4.	The NWDAF sends a Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify to the consumer.

5-6.	If the consumer subscribed for analytics in step 1, when the NWDAF may collect additional data, generate new analytics and provides the new generated analytics to the NF.

End of changes