3GPP TSG-SA2 Meeting #165	 S2-2409889

# 14 – 18 October 2024, Hyderabad, India							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0584 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Collection by LMF procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR provides updates to procedure for Data Collection from the UE described in clause 5.18.1 and also addresses the following ENs. Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS. Editor's note: Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following clarifications are made: - LMF may be triggered by NWDAF to collect data for training an AIML model for Direct AIML positioning - LMF triggers UE to collect data by sending a supplamentatry service “LCS Periodic-Triggered Invoke Request” which includes configuration to collect data within an LPP message - UE collects data based on the LPP configuration - When the UE is ready to report data the UE sends an Event Report - LMF and UE establish an LPP session to retrieve data - LMF may store data in ADRF. The ENs for UE capabilities are removed as this is implicitly supported by the UE acknowledging the LCS Periodic-Triggered Invoke Request for Data collection. It is assumed that RAN will specify the data collection procedure using LPP messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for data collection from the UE is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

### 5.18.1	Data collection at LMF to train the AI/ML based positioning model to perform positioning based on UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Editor's note:	The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

0.	The LMF is triggered to collect data from the UE or the NG-RAN. This may be an internal trigger in the LMF or a request from the NWDAF.

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

3.	The LMF checks the user consent for data collection for model training.

Editor's note:	Further details on user consent for data collection for model training will be aligned with SA WG3.

For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

4.	The LMF sends the LCS Periodic-Triggered Invoke Request to request Data collection. The procedure maybe carried out via control plane is as described in step 15 of Figure 6.3.1-1 or the LMF may send the request via user plane. The request includes an LPP message that provides measurement configuration relevant to the data collection.

5.	The UE returns an acknowledgement via control plane or user plane based on the request in step 4. The acknowledgement is an indication that the UE supports providing the requested measurement information.

6.	The UE collects the measurement information as per LPP measurement configuration.

7.	The UE sends an Event Report indicating that the UE is ready to report data. This may be due to the UE buffer for collecting measurement information being full.

8.	LMF provides acknowledgement

9.	LMF and UE initiate an LPP protocol session in order for the UE to report the measurement data.

Editor’s Note:	The details for providing measurement information using the LPP protocol is to be described in RAN groups.

10.	LMF reports data to consumer

11.	LMF may store data to ADRF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the AI/ML based positioning model.

************************************ END OF CHANGE *****************************