3GPP TSG-SA2 Meeting #165	S2-2409897

Hyderabad, IN, 14th Oct -  18th Oct, 2024                                            (revised of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5672 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overload control considering signaling storm analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-84 v1.0.0 has documented the conclusion for key issue#4 “NWDAF enhancements to support network abnormal behaviours (i.e. signalling storm) mitigation and prevention”. - Examples of mitigation or prevention for NFs will be discussed and provided in the normative work, considering a similar format as in table 6.7.5.3-3 in TS 23.288 [5] as a basis, the final mitigation or prevention operations are based on operator's policy/configuration and NF implementation. The mitigation and prevention described in Solutions #35 and #39 will be consider for used as a basis for such examples: - UE-related configurations (back-off timers, thresholds, slice or priority based overload control, etc.); - NF-related configuration ((re)discovery/selection, prioritizations, threshold, etc.). NOTE 5: Examples of prevention/mitigation actions based on NWDAF analytics on signalling storm prediction and/or detection may be further refined during the course of the normative work. NOTE 6: Whether SCP entity itself can be a source of input data will be discussed and specified in normative work. This contribution provides that the 5G NF may take measures to prevent overload based on the analytics from NWDAF（e.g. signaling storm）. |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the general description that 5G NF may take measures to prevent overload based on the analytics from NWDAF（e.g. signaling storm） |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified the 5G NF may take measures to prevent overload based on the analytics from NWDAF（e.g. signaling storm） |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.19.1	General

In order to ensure that the network functions within 5G System are operating under nominal capacity for providing connectivity and necessary services to the UE. Thus, it supports various measures to guard itself under various operating conditions (e.g. peak operating hour, extreme situations). It includes support for load (re-)balancing, overload control and NAS level congestion control. A 5GC NF is considered to be in overload when it is operating over its nominal capacity resulting in diminished performance (including impacts to handling of incoming and outgoing traffic).

A 5G NF may take measures to prevent or mitigate the overload based on the analytics from NWDAF (e.g. signalling storm analytics, NF load analytics)

# * * * * End of changes * * * *