3GPP SA WG2 Meeting #165	 S2-2409909

# Hyderabad, India, 14-18 October 2024									                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1468 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Nimsas_ImsSessionManagement Service to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two Editor’s Notes were captured in SA2#164 on the specification of Nimsas_Session Management Service: Editor's note: How to support MDC1 and MDC2 media information is FFS. Editor's note: The parameter design is FFS. This CR updated the parameters and descriptions in the service specification to resolve the above ENs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MDC1 and MDC2 endpoint information. Replaced the parameters of “Serving Number”, “Destination Number” with “Calling ID” and “Called ID” to align with existing parameters specified in Nimsas_SessionEventControl service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Nimsas_Session Management Service is not completely specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.4.1, AA.2.4.4.2, AA.2.4.4.3, AA.2.4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### AA.2.4.4	Nimsas_ImsSessionManagement Service

#### AA.2.4.4.1	General

Service description: This service enables the consumer to request IMS AS to create and release an IMS session and modify the media in a specific IMS session.

NOTE:	In this release only session management of IMS data channel is specified.

#### AA.2.4.4.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with standalone data channel media.

Inputs, Required: Calling ID,  Called ID, Media Information set.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Media Information set includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Type: It indicates the media type used in the created session, e.g., Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC):

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

-	MDC1/MDC2 media endpoint address of the application layer.

Inputs, Optional: None.

Outputs, Required: Result indication and Session ID when the operation is successful.

Outputs, Optional: None.

#### AA.2.4.4.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as creating or releasing bootstrap data channel(s) or application data channel(s).

Inputs, Required:Calling ID, Called ID, Media Information set.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Media operation set includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the operated media is Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC):

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

-	MDC1/MDC2 media endpoint address of the application layer.

Inputs, Optional: Session ID.

Outputs, Required: Result indication and Session ID when the operation is successful.

Outputs, Optional: None.

#### AA.2.4.4.4	Nimsas_ImsSessionManagement_Delete service operation

Service operation name: Nimsas_ImsSessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required:Calling ID, Called ID.

Inputs, Optional: Session ID.

Outputs, Required: Result indication.

Outputs, Optional: None.

# >>>>END OF CHANGES<<<<