3GPP TSG-SA2 Meeting #165	S2-2409925

14 - 18 October, 2024, Hyderabad, India

Source:	Nokia

Title:	KI#2: VFL model correlation ID

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: whether a VFL model correlation IF is required. Related changes are proposed in S2-2409926.

# Discussion

The following EN should be resolved:

Editor's note:	Whether the VFL model correlation ID is needed or using the model ID is FFS

The conclusions for key issue 2 in the TR 23-700-84 contain the following

P#2.4:  For VFL training process:

P#2.4.3: An identifier is allocated by a VFL server, which is used to correlate the participants during the VFL training and subsequent VFL inference processes and it is associated with the distributed ML Models in the VFL joint model training process.

Also TS 23.288 contains

## 5.4	Vertical Federated Learning (VFL)

…

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

Editor's note:	Support for AF acting as VFL server selecting multiple NWDAFs is FFS.

-	requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL model correlation ID, and it requests to report intermediate results.

-	aggregates intermediate results from VFL client(s) and computes intermediate training results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training results towards VFL clients involved in the joint VFL training process.

-	It initiates the VFL inference process using VFL model correlation ID.

-	It aggregates local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

#### 6.2G.2.1	General inference procedure for vertical federated learning

If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the NWDAF and the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Whether the VFL model correlation ID is needed or using the model ID is FFS.

Observation 1: While some agreement relating to the VFL model correlation ID was already achieved during the study conclusions and also in the normative text, there was also a desire to compare it with a solution merely based on model IDs

The underlying issue appears to be that models that have been trained together in the VFL training process also need to be used together for the VFL inference process.

Option 1: Use VFL model correlation ID to indicate applicable models to clients. The client stores how the VFL model correlation ID relates to its local model ID.

Option 2: Each Client communicates the model ID it trained for a feature to the to the VFL server, and the VFL server in turn communicates the applicable model ID to the VFL clients when requesting them to perform inference.

Option 1 also allows that VFL model correlation ID is stored together with model IDs in the ADRF. This allows for a reconstruction of how inference results were achieved based on database information.

Observation 2: Option 1 (VFL model Correlation ID) offers advantages when trying to reconstruct how inference results were achieved, on particular when stored in the ADRF together with the model ID.

For an AF as VFL client, there are no model IDs and it would need to be added for option 2.

For an AF as VFL server, internal model IDs should not be displayed for privacy reasons.

Observation 3: Option 1 (VFL model Correlation ID) offers adavantages in particular when AFs are involved as VFL participants.

The VFL model correlation ID might also be used to discover clients for the VFL inference process, e.g. if stored in the NRF. (FFS)

Summary of Proposals

Proposal: Confirm agreement to use VFL model correlation ID (Option 1)