3GPP TSG-SA2 Meeting #165	S2-2409928

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1235 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL procedures with AF as server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are open issues and missing documentation for VFL procedures with AF as server. See considerations in S2-2409927. |  |  |  |  |  |  |  |  |
| Summary of change: |  | VFL procedures with AF as server are documented. The following related principles are applied: A trusted AF as VFL server directly accesses NRF, select NWDAFs as VFL clients, and communicates with them directly without an intermediate NEF NEF selects VFL clients based on NRF entries and requirements provided by AF as VFL server, provides temporary IDs for those VFL clients to the AF, and proxies subsequent communication based on those IDs Introduce an “AF VFL service ID”, store it in the NRF, and use it for the selection of NWDAFs as VFL clients and in training and inference request towards NWDAFs as VFL clients. An untrusted AF as VFL server may supply “Vendor ID” and “AF VFL service ID” as NWDAF selection criteria, but NEF needs to authorizes the use. The NEF supports sample alignment by proxying related messages and modifying the identifiers/description of samples between an external and internal format where required (e.g. Transforming GPSIs into SUPIs, and modifying description of an area. For UEs as samples, the AF can reuse the Nnef_MemberUESelectionAssistance service to check the availability of UEs. The AF as VFL server may be triggered by internal logic or the subscription to an event received via the Naf_EventExposure service from an NWDAF to perform a VFL inference procedure Document call flows when AF is VFL server separately from call flows when NWDAF is VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing documentation for VFL procedures with AF as server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, new 6.2H.2.1X, new 6.2H.2.Y, new 6.2H.2.Z, new 6.2H.2.A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning techinique without exchanging/sharing local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AF.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

-	An AF acting as VFL server selects VFL client(s) NWDAF(s) to participate in a VFL procedure. For an untrusted AF, the NEF discovers VFL client(s) NWDAF(s) on behalf of the AF and hides detailed information about the NWDAFs from the AF.

-	requests VFL clients to do local ML model training, it assigns VFL model correlation ID, and it requests to report intermediate results. For an NWDAF acting as VFL server, the VFL procedure relates to an Analytics ID. For an AF acting as VFL server, the VFL procedure relates to an VFL service ID.

-	aggregates intermediate results from VFL client(s) and computes intermediate training results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training results towards VFL clients involved in the joint VFL training process.

-	It initiates the VFL inference process using VFL model correlation ID and requests the VFL clients to determine local inference results.

-	It aggregates local inference result from VFL clients and generates the final VFL inference result.

-	An NWDAF acting as VFL server may send the final VFL inference result to the analytics consumer.

-	An untrusted AF acting as VFL server interacts with NWDAFs as VFL clients through an NEF.

-	An NWDAFs acting as VFL server interacts with untrusted AFs as VFL clients through an NEF.

VFL client:

-	locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients due to e.g. data privacy, data security, data access rights.

-	computes the intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	performs inference based on the local model and local data and provides inference results to VFL server.

Editor's note:	Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS.

Editor's note:	Accuracy monitoring in VFL is FFS.

Editor's note:	For an NWDAF impacts of the split into AnLF and MTLF are FFS.

2nd change

#### 6.2H.2.X	Registration and Discovery procedure for Vertical Federated Learning with untrusted AF as the VFL server

Figure 6.2H.2.X-1: Registration and Discovery procedure for Vertical Federated Learning when Untrusted AF is the VFL server and NWDAF(s) are the VFL clients

1 1.	An NWDAF as VFL client registers its profile at the NRF. The NWDAFshall include its VFL capability information including VFL capability type (i.e. VFL client). It shall in addition indicate VFL service ID(s) and for each VFL service ID Vendor ID(s) for which it supports VFL, and may in addition include for each VFL service ID and Vendor ID vendor specific-feature information. It may also include a Time interval for which it supports VFL

2.	The untrusted AF acting as the VFL server determines that VFL operations are required and the NWDAF(s) as VFL client(s) are required. For each VFL client, the AF sends a request to assign the VFL client to the NEF and provides selection criteria for the VFL client in the request (VFL service ID, Vendor ID, optional vendor specific-feature information, optional time interval, optional AnLF/MTLF indicator)

3.	The NEF checks based on configured policies whether the AF is entitled to request a VFL client assignment fot the VFL service ID and Vendor ID.

4.	The NEF discovers at the NRF NWDAFs capable as acting as VFL clients and matching the received selection criteria

5.	The NEF selects an NWDAFscapable as acting as VFL clients and matching the received selection criteria

6.	The NEF assigns a temporary VFL client ID to the selected NWDAF.

7.	The NEF provides the assigned temporary VFL client ID to the AF.

8.	The AF stores the received temporary VFL client ID and uses it subsequent interactions with the NEF to indicate the target VFL client.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

3rd change

#### 6.2H.2.Y	Vertical Federated Learning (VFL) sample allignment procedures with AF as VFL server

Figure 6.2H.2.Y-1: VFL sample alignment with untrusted AF as VFL server

- Before starting a vertical federated learning, the AF acting as VFL training server determines an initial list of samples, e.g. of target UEs to be used as sample for vertical federated learning.

2.-4 If samples are UEs, for each candidate UE, steps 2 to 4 may be performed. The AF uses the Nnef_MemberSelection service to ask the NEF to reduce the list of candidate UEs based on filter criteris iot provides.

For each NWDAF acting as VFL training client, steps 5 to 11 may be performed

5.	The AF sends a request to the NEF to check the availability of samples, e.g. target UEs and provides the list of target UEs. It identifies the VFL client usinfg the temporary VFL client ID

6.	If external identifiers are used for smples, the NEF converts them to internal identifiers. The NEF looks up stored information about VFL client and selection criteria.

7.	The NEF sends a request to the NWDAF to check the availability of samples, e.g. target UEs and provides the list of target UEs. It provides stored information about the VFLclient  selection ctiteria.

8.	The NWDAF checks for each sample, e.g.target UE, whether it can retrieve related input data and otherwise removes the sample from the list of target samples.

9.	The NWDAF replies with a possibly reduced list of target samples.

10.	If internal identifiers are used for smples, the NEF converts them to external identifiers

11.	The NEF replies with the possibly reduced list of target samples to the AF.

- The AF acting as VFL server computes the intersection of the possibly reduced list of target samples provided by the VFL clients and uses the resulting intersection list as as sample for the VFL model training.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

4th change

#### 6.2H.2.Z	Vertical Federated Learning (VFL) training procedures with untrusted AF as VFL server

Figure 6.2H.2.Y-1: VFL model training with Untrusted AF as VFL server

- The AF acting as VFL training server assigns a unique VFL Model Correlation ID.

Steps 2 to 5 are repeated until the MTLF acting as VFL training server determines that the training is complete.

2.	For each VFL client, the AF acting as VFL training server sends a request to start a VFL training iteration to the NEF. It identifies the VFL client usinfg the temporary VFL client ID

3.	The NEF looks up stored information about VFL client and selection criteria. It forwards the request to the VFL client and includes the stored selection criteria.

4.	Each NWDAF/MTLF acting as VFL client collect input data, updates the own model based on the input data, and calculates so-called “gradients” as intermediate results based on the updated model.

5.	Each NWDAF/MTLF as VFL training client sends the intermediate results to the NEF

6	For each NWDAF/MTLF as VFL training client, the NEF forwards the the intermediate results to the AF

7.	The AF acting as VFL server updates the own model based on the received intermediate results. The AF may in addition collect own input data and also use those data to update the model. For each VFL client, it determines intermediate results based on the updated model and provides those intermediate results to the VFL clients in the request to start the next training iteration (step 2).

8.	Once the AF acting as VFL server determines that the trained model is sufficiently stable to terminate the training, it informs each VFL clients that the VFL training is completed.

9.	The NEF looks up stored information about VFL client and selection criteria. It forwards the request to the VFL client and includes the stored selection criteria.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

5th change

#### 6.2H.2.A	Inference procedure for vertical federated learning with untrusted AF as VFL server

Figure 6.2H.2.A-1: inference procedure for vertical federated learning with untrusted AF as VFL server.

1.	The inference procedure may be triggered by internal service logic of the AFD acting as VFL server, or ba subscription for related events from a 5GC entity, for instance a subscription to service experience information.

2.	VFL server decides to iniate the VFL inference procedure with the VFL clients and for each client sends an interence request or subscription to the NEF including the temporary VFL client ID, Target of Analytics Reporting e.g. UE IDs, possible analytics filters, and VFL model correlation ID

3.	The NEF looks up stored information about VFL client and selection criteria. It forwards the request to the VFL client and includes the stored selection criteria.

4.	Each VFL Client collects its local data if the VFL Client does not have local data available already.

5.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

6.	Each VFL Client sends the intermediate local inference results to the NEF. The VFL Client may in addition include validity period, confidence, and Client Analytics Metadata Information

7	For each VFL client, the NEF forwards the inference result to the VFL server.

If the VFL server used an inference subscription in step 2, step 6 and 7 may be repeated.

8.	The AF acting as VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

9.	If the inference was triggered by an event subscription, the AF may report results derived by the inference process to the consumer.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

End of changes