3GPP TSG-SA2 Meeting #165	S2-2409930

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1236 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | See considerations in S2-2409929 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the following new services to support VFL: Nnwdaf_VflInference, Naf_VflTraining, Naf_VflInference, Nnef_VflTraining, Nnef_VflInference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL cannot be implemted asc services are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, new 7.X, new X, new Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

71st change

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_VflInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Editor´s note: It is FFS whether AF and NEF can also be consumers of the Nnwdaf_VflInference service.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |

2nd change

## 7.X	Nnwdaf_VflInference Service

### 7.X.1	General

Service Description: This service is provided by an NWDAF/AnLF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the NWDAF containing AnLF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

### 7.X.2	Nnwdaf_VflInference_Subscribe service operation

Service operation name: Nnwdaf_ VflInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### 7.X.3	Nnwdaf_ VflInference_Unsubscribe service operation

Service operation name: Nnwdaf_ VflInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.4	Nnwdaf_ VflInference_Notify service operation

Service operation name: Nnwdaf_VflInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the VFL inference subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. NWDAF overload, UE moved out of NWDAF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.5	Nnwdaf_VflInference_Request service operation

Service operation name: Nnwdaf_VflInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

3rd change

# X	AF Services to support to support network data analytics

## X.1	General

Table X.1-1 illustrates the AF Services to support to support network data analytics.

Table X.1-1: NF services provided by AF to support to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_VflTraining | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
|  | Request | Request / Response | NWDAF, NEF |
| Naf_VflInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |

Editor´s note: The service operations of the Naf_VflTraining service are FFS.

## X.2	Naf_VflTraining Service

### X.2.1	General

Service Description: This service is provided by an AF acting as VFL client and enables an VFL server as consumer to request the AF to participate in VFL model training as VFL client.

Editor´s note: Service Operations and Parameters of the service are FFS

## X.3	Naf_VflInference Service

### X.3.1	General

Service Description: This service is provided by AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

### X.3.2	Naf_VflInference_Subscribe service operation

Service operation name: Naf_ VflInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### X.3.3	Naf_VflInference_Unsubscribe service operation

Service operation name: Naf_ VflInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### X.3.4	Naf_VflInference_Notify service operation

Service operation name: Naf_VflInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the VFL inference subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. NWDAF overload, UE moved out of NWDAF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### X.3.5	Naf_VflInference_Request service operation

Service operation name: Naf_VflInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

4th change

# Y	NEF Services to support to support network data analytics

## Y.1	General

Table Y.1-1 illustrates the NEF Services to support to support network data analytics.

Table Y.1-1: NF services provided by NEF to support to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_VflTraining | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
|  | Request | Request / Response | NWDAF, NEF |
| Nnef_VflInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |

Editor´s note: The service operations of the Nnef_VflTraining service are FFS.

## Y.2	Nnef_VflTraining Service

### Y.2.1	General

Service Description: This service is provided by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request the AF to participate in VFL model training as VFL client.

Editor´s note: Service Operations and Parameters of the service are FFS

## Y.3	Nnef_VflInference Service

### Y.3.1	General

Service Description: This service is provided by by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

### Y.3.2	Nnef_VflInference_Subscribe service operation

Service operation name: Nnef_ VflInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### Y.3.3	Nnef_ VflInference_Unsubscribe service operation

Service operation name: Nnef_ VflInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.3.4	Nnef_ VflInference_Notify service operation

Service operation name: Nnef_VflInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the VFL inference subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. NWDAF overload, UE moved out of NWDAF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.3.5	Nnef_VflInference_Request service operation

Service operation name: Nnef_VflInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

End of changes